For Windows XP

- User's Manual

- Core Technologies for Laser Markers

- 1103 Bob White Trail, Chuluota, FL 32766

Phone: (407) 366-2237 - Fax: (407) 366-4691

www.laserware.com

- Copyright (c) 1992-2004 by American Laserware Inc. and Walter K. Cudebec.  All rights reserved.

All American Laserware products are trademarks or registered trademarks of American Laserware, Inc.  Other brand and product names are trademarks or registered trademarks of their respective holders.

This manual and software may not be copied (except for legitimate backup purposes), reproduced, transmitted by any means, modified, or edited, in whole or in part without the prior written consent and approval of American Laserware Inc.  Any modifications to this manual and/or software are considered derivative works and are the property of American Laserware, Inc. Use of the security locking device is a condition of this ProLase software license.  Any attempt to run ProLase without this device is a violation of the License and U.S. Copyright law.  Software theft is a crime, a $1000 USD reward is offered for any information leading to the arrest and conviction of anyone violating American Laserware copyrights.

- Limited Warranty:

American Laserware Inc. will repair or replace this product, for one (1) year from the date of original purchase in the event of a defect in materials or workmanship.

The vector graphic import capability of this product may be provided by the included ImageStream technology, copyright INSO Corp. for which American Laserware Inc. holds a valid license.  The license agreement is for object code only, American Laserware Inc. does not have any access to the source code.  American Laserware Inc. provides no warranty whatsoever related to the ability of ImageStream to import particular files or file formats.  This component is completely controlled by INSO Corp.  The specifications and any warranty are the responsibility of INSO Corp.

- Limits and Exclusions:

No warranties are expressed or implied except for the expressed warranty listed above.  There is no warranty of merchantability or fitness for a particular purpose.  American Laserware shall not be liable for incidental or consequential damages resulting from the use of this product, the information in this manual, or arising out of any breach of this warranty.

Some states do not allow the exclusion or limitation of incidental or consequential damages, or limitations on how long an implied warranty lasts, so the above exclusions or limitations may not apply to you.

- This warranty gives you specific legal rights and you may have other rights that vary from state to state.

- American Laserware, Inc.

- P.O. Box 845

- Goldenrod, FL 32733

- Phone support:

(407) 366-2237

- Table of Contents

Chapter 1 Introduction	1-6

Chapter 2 Hardware requirements	2-9

Security Key:	2-9

CPU:	2-9

Operating System:	2-9

Graphic Card:	2-9

Control I/O:	2-9

DADIO CARD:	2-11

DB25 Digital I/O	2-11

DB9 DAC Outputs	2-11

CIO-DDA06/12 or /16 or PCIM-DDA06/16:	2-11

Digital Inputs and Outputs:	2-12

Analog Outputs (DACs):	2-12

PCI-DDA02/12 or /16, PCI-DDA04/12 or /16, PCI-DDA08/12 or /16:	2-12

Digital Inputs and Outputs:	2-12

Analog Outputs (DACs):	2-13

PC1000:	2-13

Laser Control:	2-13

Optional Pulse Control:	2-15

SCANLAB RTC2 and RTC3:	2-18

RTC2:	2-20

RTC3:	2-21

Synrad FLCC:	2-22

Chapter 3 Basic software controls	3-23

Mouse Control:	3-23

Layer / Object Control:	3-23

Layer Creation:	3-25

Layer General Settings:	3-26

Layer I/O Settings:	3-26

Positioning Settings:	3-29

Object Creation:	3-30

Default Object Properties:	3-30

General Object Properties:	3-31

Image  Object Properties:	3-32

Orientation and Sizing:	3-43

Variable Text:	3-45

Laser Control:	3-49

Chapter 4 Touring the Menus	4-51

File Menu:	4-51

New:	4-51

Open:	4-51

Save:	4-52

Save As:	4-52

Import Graphic File:	4-52

Import ALI File:	4-52

Mark:	4-52

Edit Menu:	4-53

Cut:	4-53

Copy:	4-53

Replace:	4-54

Insert:	4-54

Delete:	4-54

Duplicate:	4-54

View Menu:	4-54

Render Fills:	4-54

Zoom:	4-55

Standard Toolbar:	4-55

Stock Objects Toolbar:	4-56

Object Control Toolbar:	4-57

Zoom Toolbar:	4-58

Insert Menu:	4-59

New Layer:	4-59

Delete Layer:	4-60

Layer Properties:	4-60

New Object:	4-60

Delete Object:	4-60

Object Properties:	4-60

Tools Menu:	4-61

Snap Settings:	4-61

Mirror:	4-62

Justification:	4-63

Aspect Mode:	4-63

Ring Mode:	4-64

Rotate:	4-65

Italicize:	4-65

Marking Menu:	4-65

Mark Control:	4-65

Fixture:	4-67

Material:	4-69

Config:	4-71

Window Menu:	4-83

QUE:	4-85

Add:	4-85

Insert:	4-85

Delete:	4-85

RUN:	4-85

Program:	4-86

Que:	4-86

Simulate and Simulate_Que:	4-87

Chapter 5 Supplemental User Instructions ProLase PLUS	5-88

Indexer LPT	5-88

‘Other’ Stepper Motor Controllers	5-89

Initialization Procedure	5-89

X/Y/Z/A Axis by Layer:	5-91

Cylindrical marking (A axis):	5-93

External Axis Step & Repeat:	5-93

Chapter 6 Supplemental Information ToolKit	6-95

Files and Directories:	6-95

Member variable naming convention:	6-95

Units:	6-96

Exported ProLase.dll Classes:	6-96

CWSData Class:	6-96

CLayer Class:	6-99

CItem Class:	6-104

CMark Class:	6-110

TestDLL Sample Application:	6-114

AXLase Active X Sample Application:	6-116

Chapter 7 Supplemental Information  ProLaseXP Server	7-117

Command Line interface:	7-117

Automation Server interface:	7-117

Document Functions	7-118

BOOL	 dLoadLazDocument(LPCTSTR lazfilename)	7-118

BSTR	 GetdMaterialFileName()	7-118

BOOL	 SetdMaterialFileName(LPCTSTR materialfilename)	7-118

BOOL	 dIsDocumentLoaded()	7-118

BOOL	 dIsValidLayer(short LayerIndex)	7-118

BOOL   dIsValidItem(short LayerIndex, short ItemIndex)	7-118

Layer Functions	7-118

Item Functions	7-119

void 	SetiFixedText(short layerindex, short itemindex, short textline, LPCTSTR text)	7-120

BSTR	GetiFixedText(short layerindex, short itemindex, short textline)	7-120

void 	SetiSerialNum(short layerindex, short itemindex, LPCTSTR text)	7-120

BSTR	GetiSerialNum(short layerindex, short itemindex)	7-120

BOOL	SetiGraphicName(short layerindex, short itemindex)	7-120

BSTR	GetiGraphicName(short layerindex, short itemindex)	7-120

Marking Functions	7-125

Appendix A	7-129

Appendix B	7-131

*.fnt	7-131

Appendix C	7-136

# Introduction

Welcome to ProLase for WindowsXP!  ProLase has been designed to meet the needs of all types of users of laser marking systems, at a reasonable price.  ProLase was developed to be a retrofit package for existing systems, or as original software on new systems.  The package provides significant advancements over previous laser marking control systems, while remaining extremely user-friendly.  It's an object oriented, graphically interactive, PC control system providing a user the ability define and execute laser marking jobs (LAZ files).  Multiple hardware interfaces are supported giving ProLase the ability to control most Nd:YAG and CO2 laser marking systems.

File links to several internal databases makes ProLase programs flexible and powerful.  These databases include a materials application system, and a fixture database.  The materials application system allows a user to define a laser process, give the process a unique name (usually a material) and subsequently link the process to graphic programs.  A process can include up to seven passes using different values for power, frequency, and speed on each pass.  The database can contain and manage many thousands of different process ‘recipes’.

The fixture database allows the user to control fixture offsets and define step & repeat processes.  Just like the material database, any LAZ job can use any fixture defined in the fixture database.  The links allow all appropriate graphic and process information to be automatically loaded when the operator selects the LAZ file.  At any time the operator can change the links, for example a LAZ job that is normally marked on stainless steel, can be marked on brass by selecting the brass process file prior to executing the job program file.

Unlike some marking software, the operator never has to remember what fonts and logo's need to be loaded for a particular job.  ProLase automatically performs all required graphic loading.  ProLase does not require users to learn any programming languages or special codes, and yet ProLase provides all of the flexible, graphic control users are accustomed to, including radial marking, aspect control, character spacing, angular rotations, and full justification.  Text to be marked can be fixed or variable.  Variable text can be retrieved at runtime from a variety of sources including, the keyboard, a bar code reader, and disk files.  Automatic date coding and alphanumeric serialization are included as variable text types.  Fonts include laser engraving fonts and True Type fonts.  True Type fonts can be vector filled using user specified density, angle and kerf values.  Graphics (sometimes called ‘logos’ on other systems) can be imported from a large variety of common vector formats.  All graphic features are either menu controlled or graphically controlled via the mouse and keyboard.

The best way to become familiar with ProLase is to read this manual while running a copy of the software.  Experiment with different combinations of graphic controls while reading about them.  The menu maps and the table of contents are useful for finding a particular function within the menu system.  If at any time a question comes up, and the answer cannot be found in the manual please call American Laserware (407-366-2237), we want to help you get the most out of ProLase.  Free software updates are available on our web site www.laserware.com.  It is recommended that the user periodically check the date on the support file named "UPDATE.ZIP".  The user should download "UPDATE.ZIP" and install it if it is newer.  This software is heavily supported and new versions are released on a regular basis.  The latest version may contain new features, bug fixes, and other improvements that could enhance the user's production.

If the new version does not work correctly, restore the old version, and notify American Laserware so the problem can be solved.

ProLase is quite flexible and meets many needs.  Sometimes, however the system must be customized to meet specific customer requirements.  Please contact us if you feel a modification is required.  Normally this can be done quickly at a very reasonable price.

# Hardware requirements

## Security Key:

ProLase requires a software security device sometimes known as a 'dongle'.  This device plugs into the USB port on the PC.  Any attempt to run the ProLase software without a properly attached dongle will generate an error.

WARNING:  The security device is a valuable piece of hardware (its value is the cost of the ProLase license fee).  Please treat it that way.  Damaged devices can be replaced for a small fee.  LOST DEVICES WILL NOT BE REPLACED!  Protect your software investment; keep track of your dongle.

## CPU:

The ProLase software was developed on an Intel Pentium running at 133 MHz with 32 MB RAM.  The performance was adequate although a 450 MHz Pentium works much quicker, and smoother.  In general, we recommend the fastest PC available.

## Operating System:

ProLase was developed for Microsoft Windows 2000/XP and will run in Windows 95, Windows 98, Window’s XP, Window’s 2000, or Windows Millennium Edition.  ProLase is an unusual program in that it must ‘disable’ the normal operation of the operating system while marking or waiting for a start signal.  Specifically ProLase will not process any Windows messages in these cases.  While the laser is on, during marking, all operating interrupts are disabled.  To avoid problems it is best to not use other user interface applications while ProLase is in any mark mode (manual, auto, etc.).  Applications and communications that happen in the background like network control do not have any adverse effects on ProLase, even while marking.  It is best if the screen saver is disabled on a PC using ProLase for marking, this is because the screen saver is basically a user interface and can interfere with the normal operation of ProLase.

## Graphic Card:

ProLase is a high resolution, 16 color, graphic program requiring the use of a VGA graphic card and monitor.  640x480, 16 colors is the MINIMUM resolution and color depth supported.  Like most graphic programs, ProLase is best when used with higher resolutions and larger monitors.

## Control I/O:

ProLase can be configured to drive several different types of laser engraving/marking equipment.

When configured for an HPGL device such as the Universal Laser Systems laser plotters, the PC Centronics port is all the I/O capability required for normal manual operation.  Other digital I/O can be optionally provided for use of parts handling equipment.

When configured for a galvanometric (see Fig. 1) device such as General Scanning or Cambridge equipment, a Techmar DADIO or Computer Boards, Inc. CIO-DDA06/12 card is required.  Examples of existing equipment that might use this I/O include Quantrad, Control Laser, General Laser, Lumonics, AB, JEC and General Scanning laser marking systems.

ProLase uses all I/O addresses between 300 hex and 34F hex.  Your system does not use all these addresses, but different customers use different interface hardware that utilize all of these spaces, many times we do multiple outputs for the same function, like outputting the Synrad power command and the PWM from the CIO-CTR card at the same time.  The proper thing for all ProLase customers to do is to make sure that there are no hardware devices (except ProLase devices) addressed between 300 and 34F hex.  You can do this by checking the RESOURCES tab for all devices listed in DEVICE MANAGER.  If you find any devices within the ProLase address space you should change those devices so that they use addresses outside of ProLase address space.  You can RESERVE ProLase address space in Windows so that no PLUG & PLAY devices are assigned those addresses.  To reserve ProLase I/O space:

Go to Control Panel and click on the SYSTEM icon.

Click on the DEVICE MANAGER tab.

Right Click on the COMPUTER icon.

Click on the PROPERTIES menu selection.

Click on the RESERVE RESOURCES tab.

Check Input/Output I/O.

Click on the ADD button.

Enter STARTING VALUE 300 Hex

Enter ENDING VALUE 34F Hex

Reboot System

ProLase address map (ISA cards).

240 Hex - 24F Hex                   SCANLAB RTC2 card

300 Hex - 30F Hex                   Cambridge Digital interface

310 Hex - 31F Hex                   CIO-CTR05 counter Timer card

320 Hex - 32F Hex                   RESERVED (custom interfaces)

330 Hex - 33F Hex                   DDA06, DADIO or Synrad FLCC card

340 Hex - 34F Hex                   SCANLAB PC1000 card

- DAC Cards:

ProLase supports several off the shelf DAC cards.  All ISA DAC cards should be addressed to 330 Hex.  The DADIO (Scientific Solutions or Techmar) card provides 4 12-bit DAC outputs, and 16 bits of digital I/O.  The CIO-DDA06/12 (Computer Boards Inc.) provides 6 12-bit DAC outputs and the same 16 bits of digital I/O.  On either card, DACs 2 and 3 are configured for +/-10 volt outputs (X and Y) for connection to standard analog servo electronics.  DACs 0 and 1 normally set for 0-5 volt outputs for analog control of power and Q-Switch frequency.  In the case of the Synrad lasers with a UC1000, DAC 0 should be set for 0-10 volt outputs.  A male DB9 connector provides the DAC outputs on the Techmar card.  A DB37 connector provides all signals on the CIO-DDA06/12 board.  These cards must be addressed by dipswitch to hex 330 (see card manuals for setting address and connector pin assignments).  Remember only one of these cards should be used.

On either card, the digital I/O is configured in two banks of eight bits each.  The first bank is inputs and the second outputs.  These I/O points are provided by a male DB25 connector on the DADIO card and by the DB37 on the CIO-DDA06/12 card.  It is recommended that the digital I/O is optically isolated by use of an OPTO 22 rack and modules (or equivalent).  Currently the following I/O definitions are implemented in the software (see tables below for connector pin assignments):

## DADIO CARD:

This section describes the pin outs for the Scientific Solutions Inc. DADIO DAC Card.  All digital signals are on the DB25 connector and all analog signals are on the DB9 connector.

### DB25 Digital I/O

INS:		PIN#:	DESCRIPTION:

#0		17	Start write signal.

#1		4

#2		16

#3		3

#4		15

#5		2

#6		14

#7		7	Optional Hardware Abort signal, can be programmed to abort run on high or low  input.

OUTS:		PIN#:	DESCRIPTION:

#0		25*	Laser On, connects to gate input on laser system.

#1		12

#2		24*	NOT Laser ON, reverse of #1, connects to gate input

#3		11

#4		23	System  ready for start signal - for auto control

#5		10

#6		22

#7		9

GROUND		13	Common signal ground for all ins and outs

* Use either OUT#0 or OUT#2 for gate control, not both.  NOTE: Synrad customers should use pin 24 for the UC1000 gate control.

### DB9 DAC Outputs

DAC		PIN#:	DESCRIPTION:

#0		6	Power Control DAC (0-5 or 0-10 VDC)

#1		7	Frequency Control DAC (0-5 or 0-10 VDC)

#2		8	X Galvo Control DAC (+/- 5 or +/- 10 VDC)

#3		9	Y Galvo Control DAC (+/- 5 or +/- 10 VDC)

GROUND		1-5	Signal grounds for DACs

## CIO-DDA06/12 or /16 or PCIM-DDA06/16:

This section describes the pin outs for the Computer Boards Inc. DDA06 DAC Card.  This card uses a single DB37 connector for both digital and analog signals.  The UPDATE/XFER jumper can be set either way for the ISA versions of the card, but the PCIM-DDA06/16 needs that jumper set to XFER mode.

### Digital Inputs and Outputs:

INS:		PIN#:	DESCRIPTION:

#0		37	Start write signal.

#1		36

#2		35

#3		34

#4		33

#5		32

#6		31

#7		30	Optional Hardware Abort signal, can be programmed to abort run on high or low input

OUTS:		PIN#:	DESCRIPTION:

#0		10*	Laser On, connects to gate input on laser system.

#1		9

#2		8*	NOT Laser ON, reverse of #1, connects to gate input

#3		7

#4		6	System ready for start signal - for auto control

#5		5

#6		4

#7		3	Last Part in series being marked

GROUND		11	Common signal ground for all digital ins and outs

CW		25	High when CW mode is active/ Low when not CW (pulsing)… PORT C, bit 4

* Use either OUT#0 or OUT#2 for gate control, not both.  NOTE: Synrad customers should use pin 8 for the UC1000 gate control.

### Analog Outputs (DACs):

- DB37 (Cont) DAC

DAC#		 PIN#:	DESCRIPTION:

#0		18	Power Control DAC (0-5 or 0-10 VDC)

#1		16	Frequency Control DAC (0-5 or 0-10 VDC)

#2		14	X Galvo Control DAC (+/- 5 or +/- 10 VDC)

#3		12	Y Galvo Control DAC (+/- 5 or +/- 10 VDC)

#4		2	Focus

#5		1	Spare

LLGND		13,15,17,19,20,21	Signal grounds for DACs

## PCI-DDA02/12 or /16, PCI-DDA04/12 or /16, PCI-DDA08/12 or /16:

This section describes the pin outs for the Computer Boards Inc. PCI-DDAxx DAC Cards.  These cards use a single 100-pin connector for both digital and analog signals.

### Digital Inputs and Outputs:

INS:		PIN#:	DESCRIPTION:

#0		58	Start write signal.

#1		57

#2		56

#3		55

#4		54

#5		53

#6		52

#7		51	Optional Hardware Abort signal, can be programmed to abort run on high or low input

OUTS:		PIN#:	DESCRIPTION:

#0		66	Laser On, connects to gate input on laser system.

#1		65

#2		64	NOT Laser ON, reverse of #1, connects to gate input

#3		63

#4		62	System ready for start signal - for auto control

#5		61

#6		60

#7		59	Last Part in series being marked

GROUND		100	 Common signal ground for all digital ins and outs

* Use either OUT#0 or OUT#2 for gate control, not both.  NOTE: Synrad customers should use pin 8 for the UC1000 gate control.

### Analog Outputs (DACs):

- 100-pin (Cont) DAC

DAC#		 PIN#:	DESCRIPTION:

#0		1	Power Control DAC (0-10 VDC)    	NOTE: X GALVO CONTROL DAC ON DDA02

#1		3	Frequency Control DAC (0-10 VDC)	NOTE:Y GALVO CONTROL DAC ON DDA02

#2		5	X Galvo Control DAC (+/- 5 or +/- 10 VDC)

#3		7	Y Galvo Control DAC (+/- 5 or +/- 10 VDC)

#4		9	Focus Control DAC (0-10 VDC)

#5		11	Spare

#6		13	Spare

#7		15	Spare

LLGND 		2,4,6,8,10,12,14,16	Signal grounds for DACs

## PC1000:

This section describes the use of the PC1000 XY-Head Interface board from Scanlab AG.  This card allows control of the SK1000 Scan Head family from Scanlab, the XY2/100 head from General Scanning Inc., and other hardware.  The user should consult with Scanlab about any compatibility issues concerning the PC1000 card.  ProLase requires the PC1000 card to be jumper addressed to hex 340.  All wiring should be done as specified by Scanlab.  The PC1000 supplies the X/Y galvo control, however a CIO-DDA06/12 (ISA) and CTR05 (either ISA or PCI) are used for all other control including digital I/O, laser power, focus, pulsing, and timing.

## Laser Control:

The LASER ON signal can be used to gate the internal Q-switching frequency generator provided by most Nd:YAG laser manufacturers (set frequency mode to Internal/Gated).  When this signal is gated ON the Q-switching pulse train is sent to the RF electronics, when gated OFF the pulse train will be prevented from reaching the RF driver (Q-switch hold-off).  It can also be used to gate pulse width modulation electronic equipment supplied by RF excited CO2 laser manufacturers.  In either case, the laser input for this signal is normally labeled as GATE IN.  The signal is used by the computer to control laser firing.  It is set ON while a vector is being drawn and OFF during non-marked movements (between letters for example).  The GATE signal can also be used to initiate the First Pulse Quencher circuit on many older Nd:YAG lasers. Newer lasers have a more sophisticated First Pulse Suppression logic which is supported using a counter/timer board as explained later in this manual.

The START signal is used for parts handler interfaces.  It initiates a marking sequence just as the keyboard <ENTER> does.  The START signals are active only when ProLase is in one of it's marking modes.  The READY signal is used to tell a parts handler when the computer is ready to accept a START command.  These two signals are all that are required for most part handler system handshaking.  The LAST PART signal is used for queuing handler systems.  It informs the handler that the last part in a sequence is being marked.  ProLase adds user programmability to this hardware for customer specific control requirements.

- Figure 1: Connections for a typical Nd:YAG galvo system.

Figure 2: Connections for a typical CO2 galvo system.

### Optional Pulse Control:

Actual pulsing signals can be optionally provided.  This TTL signal connects directly to the RF driver MOD IN, providing Q-switch control (set frequency mode to External).  When using this option, the FPS (pin 32) signal is used to provide first pulse suppression logic to Nd:YAG lasers with this feature.  The interface is accomplished using a counter/timer card.  ProLase supports 3 models: CTM-05, CIO-CTR05, or PCI-CTR05.  This option is also required for the use of the ProLase raster (bitmap marking) capability. The card is used to precisely coordinate the galvo positioning with the laser pulsing.  The DAC frequency control output is not used with this option.  To use this option:

- Address the CTM-05 board to hex 310.

- Use Pin 33 for Nd:YAG MOD OUT.

- Use Pin 11 as digital ground.

- Jumper (connect with wire) Pin 35 to P18 on Counter/Timer Board

- (Optional) If your laser supports First Pulse Suppression connect pin 32 to the laser input for this function.

A signal configured to provide pulse width modulation (PWM) power control of RF excited CO2 lasers, like those produced by Synrad Inc. of Bothell, Washington, or Universal Laser Inc. of Scottsdale AZ, is also provided.  In this case, the DAC power control is not used.  The DAC power control and GATE signal can be used without this option by providing an external PWM controller such as the UC-1000 series manufactured by Synrad Inc.  To use this option:

- Address the CTM-05 board to hex 310.

- Use Pin 34 as PWM Control.  This connects directly to the Synrad Laser control input.

- Use Pin 11 as digital ground.

- Jumper Pin 35 to Pin 18 on Counter/Timer Board.

All of these signals include the ‘StartSeg TC’ logic that minimizes hot spots created by galvo acceleration.  Pin 35 can be used as a preferred GATE signal that also provides this logic.  The GATE signals on the DAC boards do not include this feature.

A Metra-Byte PC bus counter/timer board (Model CTM-05) or Computer Boards Inc. (Model CIO-CTR or PCI-CTR05) is required for any of these options.  The Computer Boards Inc. product, CIO-CTR05 is about $150 USD.

Laser control signals:

A fully implemented system using a DDA (or similar) driver and a CTR05 card supports 5 laser outputs: 1) Gate, 2) Delayed Gate, 3) First Pulse Suppression (FPS), 4) Pulse-width modulation (PWM), 5) Q-switch modulation.  All 5 signals are generated for all marks, however only those require should be used.  A typical Q-switched YAG system will require the FPS and Q-switch signals but not the PWM, which is used to control RF excited CO2 lasers.  The PWM signal is used for CO2 lasers.  The FPS and Q-switch modulation are used on YAG lasers.  The Gate (or NOT Gate) signals originate from the general purpose I/O.  The remaining signals are from the CTR05 card.  The following figure illustrates the relative timing of each of the signals:

Figure 3: Typical PWM CO2 laser signal timing diagram.

Figure 4: Typical Q-switchedYAG (normal FPS mode) laser signal timing diagram.

Figure 5: Typical Q-switched YAG (in Alt. FPS Mode) laser signal timing diagram.

In the preceeding figures, the period for the PWM and the Q-switch mod. signals vary with the frequency laser parameter setting of the object being marked.  The PWM pulse with varies with the object’s power level.  The StartSeg delay, the Q-Switch width, FPS delay and FPS mode toggle are parameters set in the Configuration pages (see Config. below).

For laser system power supplies that require indication of continuous wave (CW) or pulsed operation, a CW Signal is available.  This output signal is only available for systems that use the DDA card / 8255 chip for digital I/O, including some configurations of SCANLAB PC1000, RTC2 and RTC3 cards. The CW Signal is provided by the 8255 chip in Port C, bit 4.  On the DDA06 card, this maps to pin 25.  The CW Signal is active-high to indicate CW mode.  A low state indicated pulsing mode.

## SCANLAB RTC2 and RTC3:

The RTC cards from SCANLAB are independent, digital motion/laser control system that resides on a PC bus.  The RTC card provides X/Y galvo positioning control and laser control signals.  As options, the RTC can provide Z-axis focus control, digital I/O, and On-The-Fly marking features.  When this card is used, ProLase acts as a “front-end” system only and is not directly involved in the actual real- time control.  From the End-User’s point of view use of ProLase is very much the same as when using one of our direct control interfaces discussed above.  When control or implementation of a feature varies for RTC users, it is documented with the feature to which it applies.  Generally these changes are minor.  There is one notable exception: arcs are not supported by the RTC card motion control.  ProLase converts all arcs into a series of connected line segments (poly-lines) before passing them on to the RTC.  Although not as precise as actual arcs this approximation works for the vast majority of applications.  The user can control the ‘fineness’ of the line segments using the Arc Res parameter in the RTC configuration dialog. Setting the Arc Res to 1 yields the greatest number of line segments to approximate the arc and is the most precise.  However, this also means that is may take longer to mark.  As Arc Res is increased fewer and fewer line segments are used.  Eventually, only a single line segment will connect the end points.  The default value of 100 works well for the majority of applications and should only be modified when more precise or faster marks are required.

On-The-Fly marking capabilities are not part of standard ProLase or ProLase PLUS.  These features are only available using RTC driver with the ProLase RTC version of the software.  (Of course, the RTC card must have the On-The-Fly features hardware enabled from SCANLAB as well.)  In versions of ProLase lacking this feature, the controls are removed or disabled.

The RTC card handles X/Y galvo control, laser power control and laser gating/pulsing control. A Counter/Timer board is not required.  Digital I/O can be provided by either an optional SCANLAB External I/O add-on card or via a CIO-DDA06/12 card.  This selection is made on the RTC configuration dialog. ProLase DAC focus control requires the DDA06/12 card be used.  The DDA06/12 MUST be configured as described above for the normal DDA06/12 driver.  When the Digital I/O is provided by the SCANLAB RTC hardware, ProLase is mapped to the higher number inputs and outputs.  Simply add 8 to the ProLase input/output number to identify the corresponding pin on the SCANLAB card.  For example, ProLase input 0 = SCANLAB Input 8 = Ext.IO pin # 13.    This is done to allow ProLase user to access the built in optically isolated inputs (SCANLAB inputs 12-15).  The output channels are mapped the same way for continuity.

All RTC cards also come with additional External Start/Stop inputs.  These maybe enabled or disabled in the RTC configuration dialog.  When enabled, these signals work in conjunction (logical OR operation) with the normal ProLase digital I/O.  For example, ProLase will being marking when either the Start Mark signal from normal digital I/O is seen OR when the SCANLAB External Start signal is seen.

RTC boards support up to 6 analog output channels.  RTC2 boards support only Analog 1.   All RTC3 cards can support Analog 1 and Analog 2.  Analog 3 – Analog 6 (also called Output A-D) are only available with the RTC3 I/O Extension card installed and selected for use as the Digital I/O source for the RTC3.  The function of each of these channels is as follows:

Analog 1	Power control of YAG-type laser (similar to DDA06/12 DAC0)

Analog 2	Power control for grayscale marking

Analog 3	RESERVED

Analog 4	RESERVED

Analog 5	Focus control (similar to DDA06/12 DAC 5)

Analog 6	RESERVED

If a DDA card is chosen as the digital I/O source for the RTC3 card, then its DAC5 can be used for focus control.  Since Analog 1 is always available with the RTC cards, it should be used for power control and not DDA DAC0.

The first step in configuring the RTC driver in ProLase is to follow the complete installation instructions for the RTC hardware and accompanying software as specified by SCANLAB.  This must be done PRIOR to using ProLase.  The ProLase installation does NOT install any of the SCANLAB RTC software.  The RTC software includes a test application (HPGL.exe) that should be used to verify that the RTC card and driver software as installed properly and will identify the proper support files (*.DLL, *.HEX, and *.CTB) for your hardware and options.The RTC board comes with a diskette that contains a series of files.  The *.HEX files contains programming information for the hardware on the RTC card.  The *.CTB files are correction tables use to correct for optical distortions and are specific to the lens used.  Consult the SCANLAB documentation for a more complete description.  At least one of each of these files is required by ProLase.  The locations of the files are user selectable in the ProLase RTC configuration.  The user should specify the same files that were used by the SCANLAB software to verify the card’s operation.

The Version Data button (RTC2) or group (RTC3) displays information about the version of SCANLAB hardware and files being used.  This information is provided purely for user benefit to assist SCANLAB in solving any problems the may occur in your system.

NOTE:	NT systems using ProLase RTC and ProLase RTC DLL require the following:

Installation of GIVEIO driver using IOINST.EXE.   Example: IOINST.EXE GIVEIO GIVEIO.SYS

Installation of Sentinel driver.

Installation of SCANLAB RTC driver using RTCSetup.exe from SCANLAB.

The NT versions of the SCANLAB *.DLL and *.HEX files must be used (For example, select RTC24NT4.DLL rather than RTC24W95.DLL). The location of these files are parameters in the ProLase configuration when the RTC2 or RTC3 driver is selected.

The differences and features between the RTC2 and RTC3 cards as applicable to ProLase are itemized below:

## RTC2:

The SCANLAB RTC2 card is an ISA bus card and is NOT Plug&Play compatible.  Therefore, its address setting must be set using hardware jumpers and this setting must be specified in the ProLase RTC2 configuration dialog using the Base Address Jumpers controls.  The default for the card and the ProLase configuration is 240 hex.  In most cases this will work fine

NOTE:		ProLase RTC for Windows NT users must also install the RTC2 Driver using the SCANLAB provided RTC2SETUP.EXE program.  The address of the card is a parameter when installing this driver and must match the address entered in the Base Address Jumpers parameter of the ProLase RTC2 configuration dialog.

The card must be the right type for the kind of laser used.  The RTC2 comes in 3 basic types: CO2, Nd:YAG, and LEE.  This will be factory set by SCANLAB based on you needs.  You must tell ProLase the type of laser you are using, in the Laser Type parameter on the RTC2 configuration dialog.  The laser output signals for the RTC2 card is fully defined in the SCANLAB documentation.

The RTC2 card also imposes a minimum speed limit for galvo motion.  This is based on the field size used and is roughly given by FieldSize / 65 seconds.  For a typical 4 inch square field, the minimum speed then is 4 inch / 65 sec = 0.0615 inch/sec.

## RTC3:

The RTC3 card is designed for the PCI bus and will be assigned an address by the computer automatically. There is no user programmable addressing required.

The RTC3 driver is required by ALL operating systems, not just NT.  It is installed using the RTCSetup.exe program supplied with the RTC3 card and is fully described by SCANLAB.

The RTC3 does not have a minimum speed limit.

Laser output mode is completely software configurable.  This should be set the laser type being used, but there is no longer a hardware setting that it must match.  If you change your laser type (and associated optics), simply select the new type.  The differences between the settings are described in the SCANLAB documentation.

The RTC3 Laser Extension connector can be used to provide digital power control signals.  See the SCANLAB documentation for hardware settings of these signals.  The RTC3 configuration dialog allows the user to ‘negate’ the logic level of these signals as needed for their particular laser hardware.

## Synrad FLCC:

ProLase supports the Synrad Fiber Link Controller Card (FLCC) for use with the Synrad digital marking heads (DH, FH or Fenix).  Synrad customer’s with SH or analog mode DH heads should use the DADIO or CIO-DDA06/12 card configuration (see above).  The FLCC must be addressed to 330 hex.  Digital I/O connections through the head should done in accordance with Synrad’s instructions.  ProLase only supports the 4 bits of digital output and 6 bits of digital input typically available on the FH / Fenix series heads, regardless of which head is actually used.

Special configuration features are available for many Synrad lens models to aid in field size calibration and enable the head’s internal distortion correction.  The lens selection and effects are discussed in more detail under Synrad FLCC in the configuration section of the manual.

# Basic software controls

## Mouse Control:

ProLase uses the mouse like most other Windows based graphic programs.  The mouse can be used to select from lists, menus, toolbar buttons, etc.  The mouse can be used to ‘drag and drop’ objects in the drawing area.  Simply place the mouse cursor over the current object (the cursor will become a four headed arrow), hold down the left mouse button and move.  The mouse can also be used to manipulate the individual graphic objects via the “graphic handles”.  The graphic handles allow the user to directly change the position, size, aspect, rotation angle and tilt angle of any object.  Simply place the mouse cursor over a graphic handle (the cursor will change according to the handle selected), hold down the left mouse button and move the handle.  The current object is drawn in red.  A white dotted box, called a bounding box, surrounds it.  The current object also has a series of colored dots around it.  The black graphic dot is called the anchor point.  This is the point that corresponds to the (X, Y) position of the object as displayed in the Sizing/Orientation property sheet.  The object is drawn about the (X,Y) position depending on the justification.  The blue dots are the graphic handles for stretching.  Only the blue dots that allow the object to stretch without moving the anchor point are displayed.  If the object is justified “Center/Center” then all graphic handles are shown since the anchor point is the center of the object.  The red line with a red dot is a rotation graphic handle, used to rotate the object about the anchor point.  The green line with green dot is the tilt angle graphic handle.

In ring mode, the bounding box is a ring and there is an additional rotation handle.  A ring mode object can be rotated about the anchor point or about the center of the ring.  The extra blue dot in ring mode is for adjusting the radius of the ring.  Ring mode is used for marking round ring type parts like bearing races.

ProLase extensively uses the ‘right click’ mouse logic to give the user quick access to properties and power menus.  For example, ‘right click’ on a blank area of the drawing window and a power menu allows you to create an object, or create a layer.  ‘Right click’ on an object and directly access the objects properties.  A tree structure (Layer/Object Layout) displays the names of all data used to control the marking process.  Simply right click on any item within the tree structure for direct access to that items properties.

## Layer / Object Control:

Marked objects are grouped into a layer/object structure.  Objects within a layer are marked together.  Machine control and automation can occur between layers.  An object can be a paragraph of fixed text, a line of variable text, one of our stock objects (circle, square, line, etc.) or an imported graphic.  Each object has it's own set of parameters that control position, size, character spacing, font, orientation, etc.  Each layer can contain many objects so a marking program could consist of many paragraphs, and many different graphic files.  ProLase only marks ‘active’ layers, leaving other layers to be used for operator instructions and information.

In this example:

- “Wafer Marks” is the first layer, the laser spot indicates that the objects within this layer get marked.

“Column Label”, “Row Label”, “Serial/Lot Number”, and “Link to Serial Number” are marked objects in the first layer.

“Background” is the second layer, the blue box indicates that the objects in this layer are drawn on the screen, but do not get marked by the laser (background layer).

- “Circle”, “Fixed Text 1”, etc. are the background objects on the second layer.

The current object is highlighted within the tree structure and drawn in red within the drawing area.  The selected object is available for editing of mark properties such as position, size, and orientation.  There are several ways to access the object properties including the INSERT menu, right clicking the object in the drawing area, or right clicking the name of the object within the Layer/Object Layout tree structure.

## Layer Creation:

The Layer creation process can be initiated by right clicking in the Layer/Object Layout tree structure, or a blank section of the drawing area.  The Insert Menu also allows for insertion of a layer.  After selecting NEW LAYER, ProLase will prompt the operator for the name of the new layer.  ProLase will insert a default name that can be changed.  At anytime the operator can change the name of a layer by accessing and editing the Layer properties.

After creating the Layer, the operator should access the Layer properties and change as needed.

### Layer General Settings:

The General Setting Property sheet is used to define the name of the Layer and to select ACTIVE or background.  Additionally, it displays the total number of Objects in the layer and lists them all by name.

### Layer I/O Settings:

The Layer I/O Settings property sheet is used to control external I/O for automation.  Each Layer within a LAZ document can be configured to set TTL outputs and wait for TTL inputs.

This allows the user to configure interfaces between ProLase and parts handling equipment.  The programmable interface is two-way, allowing for setting of outputs and waiting for input combinations before marking the next layer.

#### Sequential processing example:

A box is to be marked on four sides.  A rotary indexer is used to present the four sides to the marking field.  An output signal from ProLase is defined to be the 90-degree ‘move’ command to the indexer.  An input is selected to be the ‘in position’ signal from the indexer, another input is selected to be the operator start sequence’ signal.  In ProLase, four layers are programmed, containing the objects to be marked on each of the sides.  All four layers are defined by the user as active (to be marked) by use of the Layer properties General Settings Page.  All four layers are programmed to mark on all conditions (Conditional Mark = XXXXXXXX).  Layer 1 is programmed to start marking when the operator start button is pushed, Layers 2,3,4 are programmed to start marking when an ‘in position’ signal is received.  This is accomplished by setting the Start Mark Input for each layer.  All layers are programmed to send a ‘move’ signal after marking by setting the Post Mark Output.  This marks all sides and returns the indexer to the 0-degree position before the next cycle.  As an alternative to the ‘in position’ input, a delay (Post Mark Delay) can be programmed so that the system will wait a user defined time period between the ‘move’ signal and the marking.  Many layers using up to six bits of output and eight bits of input can be programmed for advanced multi-step interfaces.

##### Layer I/O:

Layer IO provides a user programmable method of controlling digital I/O between program file layers.  The output selections allow the user to set a bit pattern on the digital port.  The input selection allows the user to program a bit pattern that the system will wait for before marking the layer.  The sequencing of the I/O is the same as the order of the menu, and the sequence is repeated for each active layer.  The Layer Active selection is used to program a layer as Active or Background.  Each layer has it's own set of programmed outputs and input.

Each item in the sequence is labeled with a number to indicate its order in the sequence.  The simple flow listing below shows the time sequence of events.

.

#### I/O Sequence:

Start Layer

Set Init Output on port

Display Pre Mark Message for the operator

Read Port until Port == Wait Input

Set Premark Output on port

Move External Axis (Focus or ProLase Plus axis)

Call User Control (user EXE file)

Mark Layer

Read Port

If Port == Object Conditional I/O, then mark object

Next Object

Step & Repeat Mark Layer

Set Postmark Output on port

Wait Postmark Delay time

Next Layer

Selecting any of the digital I/O functions will present the user with a control used to define the bit settings.  This control shows the number of each bit on the port and the programmed setting.  The mouse is used to select the bit to program.  The mouse button is used to toggle the possible settings.  There are three possible settings, a black check (1 or bit on), an empty box (0 or bit off), or a grayed out check (X or don’t care).  The grayed out check means leave that bit unchanged when setting the outputs, and it means ‘don't care’ or ignore when checking inputs.  For example: an output like XXXX1001 will set bits 0,3 to one, 1,2 to zero, and leave 4,5,6,7 unchanged.  Programmed as an input XXXX1001 would mean that the system would keep reading the port until bits 0,3 equal 1 and bits 2,3 equal zero, then mark the part.  The input routine would ignore the port values of bits 4,5,6,7.

#### User Control:

The User Control feature allows the user to extend the capabilities of ProLase by providing a user created BAT or EXE function to provide custom communication or parts handler control.  Every time the layer is processed the user defined BAT or EXE file is called by ProLase in a synchronous manner.  This means that the user program is executed to completion before ProLase continues to the next step (actual marking).  The user provided program can be used to communicate with various interfaces (network connections, RS232, etc.), get data to be marked from the interface and create files to provide ProLase variable text objects with the data. (see Variable Text type, Dynamic File).  User Control programs can create any number of text files used by Variable Text.  Name, address, and phone number, for example could be marked as three objects, with one User Control call to create all three Dynamic Files.  The User Control can also be used to move external motion systems for part placement.  If used in this way it is important that all motion is completed before the User Control program quits, otherwise ProLase will begin marking while the part is still moving.

The User Control has three properties: the name of the .BAT or .EXE file to be executed, any desired command line parameters, and whether or not the user wishes the program to have its own viewing window.  The first edit box is for the name of the program and includes a browse button for easy navigation.  The second edit box is for any desired command line parameters.  The Show Window check box is to indicate that the program should have a viewing window.  The viewing window is a simple DOS box for a .BAT file.  Visual Basic, C++, etc. type programs can have full blown user interfaces.  For simple .BAT file control the Show Window feature is a good way to debug the interface.  Simply enable Show Window while testing the interface, and disable during production.  Note: All batch files used by ProLase must have their CLOSE ON EXIT property set.  To set this property in Windows use explorer or My Computer to find the icon for the .BAT file.  Right click on the icon, and select Properties from the menu.  Go to the Program tab.  Make sure “Close on exit” is checked.

#### Digital I/O Default Settings:

The Init Output is set to XXX1XXXX (bit 4 on  = READY).  The Wait Input is programmed as XXXXXXX1 (bit 0 on  =  START).  The Object Conditional Mark is programmed as XXXXXXXX for every object meaning all objects will always be marked.  The Premark Output is set to XXX0XXXX (bit 4 off = BUSY or NOT READY signal).  The Postmark Output is set to XXX0XXXX  (bit 4 off).  When layer 1 is finished, the whole thing starts again.

The Abort Output is the Output setting that will be applied when a run is aborted.  This becomes the "standby" state of the output port when ProLase is not in run mode.

### Positioning Settings:

The positioning settings page is used to set the Z Table (External focus) for each layer.  This allows

for the marking of “Stepped” parts.  Z Table Timer is a delay time used to insure that the Z axis has completed its motion before the system begins to mark.

## Object Creation:

The Object creation process can be initiated by right clicking in the Layer/Object Layout tree structure, or a blank section of the drawing area.  The Insert Menu also allows for insertion of an object.  After selecting NEW Object, ProLase will prompt the operator for the type of object to create: fixed text, variable text, graphic, link, or empty.  Next, the system will prompt for the name of the new object.  ProLase will insert a default name that can be changed.  At any time, the operator can change the name of an Object by accessing and editing the Object properties.

After creating the Object, the operator should access the Object properties and edit as desired.  At the very least, the operator should enter the desired text or graphic file name.  After that the operator can continue editing the property sheets or can enter OK to the property pages and start using the mouse drag and drop, or toolbar buttons to manipulate the object.

### Default Object Properties:

ProLase allows the user or OEM to set the default values for each object type.  This allows the user to decide what set of values should be used to create a new object.  To use this feature simply create a new object and set the properties to whatever values you want that particular object type to start with.  For example if you want new graphic objects to be created with the Aspect Mode property set to NATURAL, you simply create a graphic object and set the Aspect mode to NATURAL.  Then go to the General Property sheet and press the USE AS DEFAULTS button.  From that point on all newly created graphic objects will be set to NATURAL Aspect Mode.  This feature can be used to set default sizes, fonts, positions, laser settings and all other object properties. Each object type (FIXTED TEXT, GRAPHIC, etc.) has its own set of default properties.  One of the best things about this feature is the ability to define default folders for each object type.  For example Fixture files, Material Files, Fonts, and Graphics can be organized in different folders, and the defaults can be used to set the locations of these folders. The default information is stored in the Preferences file (DEFAULT.PRE). If DEFAULT.PRE is deleted, ProLase will set all defaults to the original factory settings.

### General Object Properties:

The ACTIVE check box allows a user to define an object as a data object to be marked on the part (ACTIVE checked) or as an alignment object (ACTIVE unchecked).  An alignment object can be used in the MARK DIALOG control to show where particular alignment points will occur on the part.  LIGHTSHOW mode and SETUP are often used with alignment objects to align a part correctly before actual marking.  Alignment objects can be bounding boxes around the objects, a box of representing the lens field or a point on the part such as a corner or a outline graphic of the part itself.  Any type of ProLase object can be used as an Alignment object.  Alignment objects are only marked if the ALIGN check box is checked in the MARK DIALOG control, otherwise actual data objects (ACTIVE objects) are marked.

The General Property sheet also allows the user to define the name and type of Object.  There are five possible Object types:

Fixed Text is a paragraph of fixed text.  Fixed text is text that is known at the time the object is created and will not change between marks.  Fixed Text objects can have up to 10 lines (each up to 255 characters long) in a paragraph.  Multiple Fixed text Objects can be used to create paragraphs longer than 10 lines.

Text Plus

A paragraph of text can optionally include imbedded text from other Fixed Text or Variable Text objects using the Text Plus feature (see Text Plus below).

Variable Text is a line of variable Text.  Variable text is text that is not specifically known at the time the object is created and could change between marks.  There are five types of Variable Text:

Keyboard variable text is used when the operator can type in the variable text while marking.

Date Code variable text is a code based on the PC date and/or time.  Date Codes will be automatically computed on every mark.

Code Reader variable text is text received from an external source such as a bar code reader or the RS232 port.

Serial Number variable text is a sequence of alpha and/or numeric text that is  serialized on each mark.  The Serial increment can be any positive or negative number.

List File variable text comes from a disk file such as a mailing list.  The file must consist of a list of ASCII text with each item in the list separated by a carriage return/line feed pair.  The next item in the list will be used on each mark.  Multiple Objects can use the same list file.

Graphic is a graphic or logo imported from one of the supported graphic formats.  There are two basic types of Graphic objects: vector and raster.

Data Link is an object that marks the text or graphic defined by another object.  The link object can specify a size, position, angle, font, etc. for marking, but the data being marked comes from the object linked to.  An example use of this is printing human readable text below a serialized bar code.  The first object would be the serial number, variable text object using a bar code font.  The second, Data Link object would be linked to the first, but would use a human readable font.  As the first object serializes the second object will display the same number.

- Empty  is object is used as a simple placeholder in the Layer’s list of items.

### Image  Object Properties:

The Image property sheet allows the user to select characteristics about the to be marked.

- Different aspects of this page are available for selection based on the type of Object.

#### Text Objects:

For Text Objects, this page is used to select the font type, and font properties.  There are three types of fonts including Laser Engraving Fonts, True Type Fonts, and Bar Code Fonts.  Laser Engraving fonts are usually single line without fills.  These fonts can be marked very fast and at small sizes.  True Type Fonts are vector outlines that can be filled by ProLase.  Bar Code Fonts can also be filled by ProLase.

##### True Type Fonts and Hatch Fills:

When True Type Fonts are selected several additional properties become available for selection.  Kerning fractions allow the user to adjust the spacing between characters.  The Height parameter sets the character spacing to a number proportional to the height of the character.  The number is a percentage, so 5 means that the character spacing will be 5% of the text height.  If only height is specified then the character spacing will be the same for all characters.  Length adds a number to the character spacing that is proportional to the width of each character.  For example if 5% is selected for length then the spacing around wide characters like “W” will be more than the space for narrow characters like the letter “I”.  In general, most True Type Fonts look good with a height adjustment around 5 and a length adjustment of zero.

True Type Fonts can be hatch filled by ProLase according to the operator specified hatch properties.  Hatch  properties include hatch spacing (in the selected distance units), hatch angle (in degrees), and kerf compensation (beam size in distance units).  Kerf compensation means ProLase will only fill to within one beam width of the original outline.  This keeps the fill from overlapping the outline when both fill and outline are being marked.  When the user is manipulating a filled object with the graphic handles, the fill vectors are not displayed.  The operator can command the system to display the fill by using the VIEW/RENDER FILLS menu selection.  This forces ProLase to calculate and display the fill using the current object data.  ProLase automatically calculates the fills based on the current size, aspect etc., insuring the fill density remains constant.  From the Image property sheet the user can select OUTLINE ONLY, FILL ONLY, or BOTH when marking True Type Fonts.

##### Bar Code Fonts:

When Bar Code Fonts is selected the user can access the type of bar code and the Bar Code Parameters by pressing the FONT button.  From Bar Code Parameters, the user can select the desired Bar Code format from the drop list.  ProLase currently supports 25 of the most common standardized linear bar code formats, the Data Matrix 2-D format, plus a customizable user-defined format.  The DETAILS button can be used to see what kind of data is valid for the selected format.  Some formats only accept numbers, others only numbers and uppercase letters, some only pairs of numbers, and still others accept all ASCII characters.  If the current text is invalid for the current bar code font, ProLase will display an error and the drawing area will display a box with an X in it.  The REVERSE check box is used to create reverse contrast image bar codes (white marks on a dark background).  The CHECKSUM check box is used to include a checksum digit in those formats that can use this feature.

The Data Matrix bar code type specifies the public domain 2-D matrix symbology from RVSI Acuity CiMatrix.  The BORDER and MATRIX SIZE controls are only available with the Data Matrix bar code type.  A Data Matrix symbol can encode up to 3116 numeric or 2335 alphanumeric characters.  Any ECC200 standard size can be selected, from 10x10 to 144x144, including 6 rectangular formats. The Smallest size selection will have ProLase create the symbol with the fewest cells that can encode the given data.  This will always yield a square matrix.

User Defined is the final bar code type.  It can be used to have ProLase render any 2-D public or proprietary format.   Pressing the DETAILS button with the User Defined type selected will display the User Defined Bar Code Properties window.  These parameters are specified on an object basis and can be saved using the object default settings.  They are used as follows:

Name: This is a text description of the format for the benefit of the user.  This label will be appended to the User Defined name in the format list.  For example, if the name is “My new format”, then the drop list of available bar code formats will end with “User Defined – My new format”.

Encoding Program:  This is the fully qualified path name of the executable program capable of encoding the data and generating the proper output file.  This file shall accept a single command line argument of an input file name and must output a single data file.  Each of these files is an ASCII text file with data separated by CRLF as described below.  Acquisition, use and maintenance of the Encoding Program is the responsibility of the user.  The call to the encoding program will be made as

EncodingProgramPathName “Input file name”

ProLase will add the quotes to the command line parameter to ensure that the entire path name of the input file will be treated as a single argument despite any spaces.

Input File:  This is the name (including full path) of the ASCII text file that will be generated by ProLase for use as input to the Encoding Program.  The data is CRLF delimited and is in the following format:

- Version number, currently 0

- Output file name

- Number of characters to be encoded

- Data to be encoded.  This will be either all text of fixed text objects or the first line (variable data) of variable text objects.

- Active additional parameters (up to 5) in increasing order.

Output File:  This is the name (including full path) of the ASCII text file that will be generated by the Encoding Program for use by ProLase.  The data is CRLF delimited and is in the following format:

- Version number, currently 0

- Number of rows of data, R

- Number of columns of data, C

- Data.  Each module (cell) of the encoded data is represented by a 1 or 0 digit indicating either black or white mark in normal (non-reversed) printing.  There MUST be R CRLF delimited lines of C data per line.

Additional Parameters:  These are additional parameters that can be used to specify additional settings that the encoding program may support.  Note that normal ProLase bar code parameters still apply and ProLase will handle the black-on-white or white-on-black setting, kerfing, fills, and sizing/orientation geometry.  The Encoding Program using the Additional Parameters must handle other settings.  Examples of such parameters include specifying the dimensions of the resulting code or a particular encoding scheme or data redundancy level.  Up to 5 parameters may be specified with each parameter activated or deactivated as desired.  When fewer than 5 parameters are activated, the input file will list only the active ones in numerical order.

Included in the installation folder of ProLase is a sample of an encoding program named ‘GenCode.exe’.  This program works as described above to generate a DataMatrix representation of the specified text.  Using GenCode.exe will produce the same marks as the DataMatrix bar code type with the ‘Smallest’ size selected.  GenCode.exe does not accept any additional parameters.

NOTE: Please have patience when using larger 2-D codes.  These codes can contain many thousands of vectors (144x144 has 20736 elements).  Even on fast computers, this will take considerable time to draw or mark.

Tuning Bar Codes:

The Module Width Fine Tuning is used to control the ratio between different elements in a bar code.  Most bar codes are made up of bars and spaces of various widths.  Common bar codes have two widths, narrow and wide, where the ratio between narrow and wide is about 2.  Bar Code 128 uses four widths, all based on a ratio to the narrowest width.  In ProLase the narrowest width bar is the basic unit, all other bars and spaces are defined in terms of a ratio to the narrow bar.  In other words, the narrow bar always has a ratio of 1:1.  Normally the narrow space also has a ratio of 1:1 since it should be the same width as the narrow bar.  However, since the laser mark has a finite width to it, the bars tend to ‘bleed over’ into the adjacent space area.  When the marked part is carefully inspected, the spaces are narrower than the narrow bar.  One way to compensate for this ‘bleed over’ effect is to vary the narrow space ratio (for example 1.095:1) until the narrow space width marks equal to the narrow bar width.  Typically, this value is doubled to get the wide space ratio.  Optimally, the wide bar will have a ratio of 2.000:1.  Each of the ratios can be adjusted until the desired effect is achieved.  Another way to compensate for the ‘bleed over’ effect is to tell ProLase the width of the laser beam mark across the material.  This parameter (Beam Width) is used by ProLase to do kerf compensation, similar to the hatch fill feature.  The bars are created by a series of parallel, vertical lines spaced according to the user supplied Beam Spacing value.  ProLase will adjust the beam spacing slightly to force a consistent number of laser passes per bar, all exactly evenly spaced.  This prevents inconsistent pass spacing due to rounding errors, present on many laser-marking systems.  The H/W Ratio is the default Height to Width ratio for the bar code.  ProLase uses this default ratio when the object is in “Natural Aspect” mode.

Rasterized Bar Codes

If either Data Matrix or User Defined bar code format is selected, the fill method can be specified.  Lines indicates each cell is filled by a series of vertical lines.  Circles fills using a series of concentric circles.  Raster will result marking the bar code as a raster image using the settings found in the Raster Parameters window.  When Raster is selected, the Single Dot Per Cell checkbox becomes available.  The Single Dot Per Cell setting will result in the raster bar code being marked as small as possible, with only a single laser pulse for each cell in the code.  The raster dot density is based on the beam width parameter.

In the experience of ALI, it has been found that the fill method generally has little effect on the readability of the resulting code.  However, it can have a significant on marking times.  In some cases, vector filled bar codes mark faster.  In others, rasterized codes are better. Because the time is highly dependent on the size of the code and the material on which it is marked, the user should experiment with all the fill methods and laser settings to determine the combination that yields the best performance.

Text Plus

All Fixed Text type text objects automatically have the Text Plus feature enabled.  This feature allow for the embedding of text from other text objects into the fixed text of the current object.  Using this feature, a single complex text paragraph can be constructed from a number of separate parts including serial numbers or date codes.  This can be especially useful if the text will be marked using a high density, 2-D bar codes, such as Data Matrix, where the desired result is a single large code containing 3 lines of text rather than 3 smaller codes with only a single line each.

The Text Plus embedding is accomplished simply by including an insertion code anywhere into the fixed text of the current object.  The current text of a Variable Text object or the first line of a Fixed Text object will be replace the insertion code in the current object’s text.  The insertion code has the following form:

<<<Layer Name : Object Name>>>

Where Layer Name and Object Name are the names for the layer and object, respectively, of the Fixed Text or Variable Text object that is the source of the embedded text.  The insertion code must EXACTLY MATCH the desired source object information and format, including the ‘space, colon, space’ separator.  If no EXACT MATCH is found, the characters are marked as normal text.

The Text Plus window provides a helpful tool for creating correct insertion codes.  The text of the object can be manipulated in the Original Text region.  Below this, in the Output Text region, the current text is displayed as it would currently be marked.  Near the bottom, the Current Source Object list itemizes all the currently valid objects that may be used as a source of data.  When an insertion code is selected in the text box, that object is highlighted.  Changing the list selection will modify the insertion code.  The Insert and Append buttons replace the current selected text with an insertion code to the first source object in the list.

NOTE: The Text Plus feature is very powerful, but also very demanding.  The insertion code must EXACTLY MATCH a valid source object otherwise those characters will be marked exactly as they read.  Additionally, only the current value of variable text is used.  Variations in the Variable Text object’s parameters can change the frequency at which the text is updated.  Care must be taken to ensure that the update is accomplished prior to the Text Plus object being marked.  Finally, the insertion codes in the Text Plus object are normal fixed text that may, or may not, be substituted with text from another object.  Insertion, deletion, or re-ordering of any objects and layers may invalidate any and all insertion codes leading to undesired text being marked.  Therefore it is highly recommended that Text Plus objects only be defined at the end of document design (perhaps even on a separate layer to ensure that no variable text will be modified) and that Simulated marking is performed to verify that the resulting text gets marked as desired.

#### Graphic Objects:

For Graphic Objects, this page is used to select the graphic file.  ProLase support two types of Graphic Objects: vector and raster.  ProLase loads the required information from graphic files as needed.  If the graphic has already been loaded once during a session and the user creates a new LAZ file that uses the graphic, the graphic is not re-loaded since it already exists in the ProLase graphic memory structure.  This structure keeps track of all graphics and fonts during a session by keeping a list of the names.  This means the user should keep graphic file names unique.  ProLase performs all graphic loading automatically.  Whenever the user opens a LAZ file, ProLase loads all required graphics and fonts not already in graphic memory.  In many cases, ProLase produces a temporary version of a graphics file in a ProLase friendly format.  This allows ProLase to load such a file faster than re-importing from the original source.  If a file is not already loaded in graphic memory, ProLase checks the date on the original file to see if it has changed since ProLase made the temporary version.  If the date of the original file has changed ProLase re-imports the file and recreates the temporary file.  This insures that any changes made to the graphic file will be reflected in the laser marks.

The GRAPHIC TYPE drop list control allows the user to change between vector and raster graphics.  The GRAPHIC button that allows selection of the graphic file to be use will only list files with type extensions that correspond to the chosen graphic type.  Selecting Raster as the graphic type enables the RASTER SETTINGS button that shows a window displaying all the raster specific parameters.  New graphic objects are set to the Vector graphic type by default.

Vector graphics consists of a series of graphic primitives: lines, positions (lines with the laser off), and circular arcs.  Sources for vector graphics are many including CAD, artwork and other drawing software.  ProLase supports 23 of the most common vector formats.  Some formats generate Arc commands while others simply form approximate Arcs using PolyLines.  ProLase performs true arc generation, so it is better to use formats that support circular arcs whenever possible.  One of the most popular formats, DXF/DWG, does not explode its text into lines and arcs, therefor these formats can not be used if the drawing includes text.  HPGL files explode everything into lines and positions, but does not produce arcs.  Other formats, including Adobe Illustrator or Encapsulated PostScript, explode text into the primitives and support Arcs.  The user should try several of the formats and select whatever works best for the particular application.

ProLase uses dynamic scaling which means graphics are drawn at whatever size the user enters in ProLase.  Scaling and size information from the drawing program have no meaning in ProLase.  For example, the user may define a square in AutoCAD that is 1 mile on each side.  Once imported into ProLase this square will be drawn at whatever size the user specifies in the Object property pages.

Raster graphics, or bitmaps, are made up of many rows of tiny dots or pixels (picture elements).  All pixels have the same size and have 3 unique properties: raster (or row), column, and color.  Raster graphic sources include artwork, drawing and scanner/digital camera software.  ProLase supports 12 of the most common raster formats.

Pressing the RASTER SETTINGS button allows the user to control the way ProLase modifies the source raster image for marking.  Laser markers cannot mark in color, they make monochrome images.  ProLase can mark bitmaps in one of three possible Render Modes:B/W, Connect the Dots, or GrayScale.

B/W:

ProLase must convert the image from the source color depth (the number of colors available) to B/W.  ProLase must also scale the image to the size the user has specified.  These two processes are accomplished by dithering.  ProLase supports 3 common dithering algorithms: Bayer, Burkes, and Floyd-Stienberg.  The Dither Scheme property allows the user to specify which dither algorithm to use.  The quality of the resulting image for each algorithm depends strongly on the subject matter (portraits, landscapes, logos, etc.).  The user should experiment to decide which method works best for a particular image.  Dithering means that grays are simulated by dot patterns.

Connect the dots:

Connect the dots means that ProLase will draw vector lines for a series of  “on” dots.  This mode is for systems where ProLase can not control the individual laser pulses.  Examples of these kinds of systems would be any kind of laser that only uses GATE control instead of pulse control, such as some older Nd:YAG lasers or the Synrad FLCC card where the interface controls the PWM and ProLase only sends a GATE.  This mode should be used for any laser run in CW mode.

GrayScale:

This is the most advanced bit map control where grays are accomplished by varying the laser power for every dot in the bitmap.  This mode only works on lasers with high speed power control such as PWM controlled CO2 lasers and diode pumped Nd:YAG lasers.  (Currently American Laserware does not have a diode pumped YAG so this technique has not been tested on these lasers.  In the near future we expect to acquire such a laser.  The technique has been tested on a PWM controlled CO2 laser with excellent results.)  This mode should never be used on flashlamp pumped Nd:YAG lasers since the power supplies and flashlamps take a long time to actually  effect the laser power. In general flashlamp pumped, Q-Switched YAG lasers should use the B/W mode with dithering to simulate grays.  The GrayScale mode produces high quality images at lower resolutions compared to the B/W mode, which means the speed of producing an image is much higher.

The overall size of an image is determined by the ProLase editor.  The Resolution property allows the user to set the resolution for the resulting image following the dither modifications.  This value is given

in ‘dots per unit’, where the unit is the base units the user specified in the Configuration (see Config Units).  If inches are selected, then the resolution is in ‘dots per inch’ or DPI.  The total number of dots is the product of the graphic size and resolution.  A 2” wide x 1.5” tall graphic with 200 DPI resolution will be marked with 300 raster lines, each 400 across.  The best results are achieved when the resolution corresponds to the laser beam width.  Thus, a system with a 5 mil spot size should use a resolution close to 200 DPI.  The resolution only applies to the marked image.  Because the screen resolution is much less than the DAC/galvo resolution (typically 1024 x 768 versus 4095 x 4095), a different method is used to display the image to the screen.  For this reason, Mark Simulate shows a much darker image than will be marked.  It should be remembered that the screen images are only to give the user feedback for general object properties such as position and size.

The Negative checkbox inverts the colors, producing a photographic negative.  This is useful when making light marks in darker materials.  Brightness Adjustment alters the overall brightness level of the image to improve the image quality.  For example, ‘washed out’ marks can be darkened.

A Note about Editing Raster Images

ProLase is NOT an image editor.  Because marking raster images is a very complicated issue, ProLase provides a limited ability to modify the source image as a convenience only.  ProLase quickly produces high quality marks from high-contrast, simple, block images.  Portraits and other high fidelity images often require considerable time to achieve perfect results.  Scanned images often have artifacts that should be removed.  This MUST be done outside of ProLase.  In general, complicated raster images should first be modified by professional image editing software.

A Technical Note about Marking Raster Images

ProLase always marks a raster image from bottom to top in the reference frame of the mark field.  ProLase positions the laser at each pixel for a short time.  When ProLase has direct control over the laser pulsing, ProLase allows exactly one laser pulse for each pixel.  Because of this, the marking speed is governed by the laser frequency and the speed parameter is ignored.  Thus, a Q-switched Nd:YAG at 5kHz will mark a 100 pixel raster in 20ms.  Additionally, to achieve high quality marks, ProLase forces all pixels to have exactly the same size.  Consequently, ProLase uses the specified graphic size and resolution to determine an approximate number of dots and a galvo step size is determined for a dot.  Then the total number of dots is determined by how many of these dots fill the specified graphic size.  The result is a ‘perfect’ image whose size and resolution vary only slightly from the specified values.  Resolution, laser power, pulse frequency, and brightness all affect the quality of the image.  Because these parameters interact, tuning an image is a complicated and somewhat subjective issue.  The user should experiment with these parameters on different images to get a feel for the control.

### Orientation and Sizing:

The Orientation and Sizing property sheet allows the user to control the geometric aspects of an object.  Properties like X/Y Position, Height, Aspect ratio, character spacing, etc. can be controlled form this page.

#### Position X/Y:

The Position Fields are used to specify the X and Y starting position for the object.  The object is actually drawn relative to this position as a function of Spacing and Justification.  The anchor point, a small, black circle, indicates the position of the position.  X/Y Position can be specified to 0.0001 resolution of whatever distance units are being used.  If inches are being used, the Position can be specified to .0001” resolution (0.1 mil).  The maximum position is +/- 32 of the distance units (+/- 32 inches, if using inches).

The user interface allows the position to be dragged using the mouse, or the numeric X/Y values can be manually entered via the property sheet.  A four-headed cursor is displayed to indicate Object dragging of Position.  If Cancel is pressed while the property sheet is visible, then the object position will return to its pre-edited condition.

#### Rotation:

The Rotation property is used to control the writing angle of an object.  The object is rotated about the Position values.  For example when left justified, the left side of the object remains stationary, while the rest of the object is rotated about the position point.  Orientation of 180 degrees produces upside down writing.  The rotation angle can be specified in .1 degree increments.

#### Italic:

The Italic property is use to control the slant angle of an object.  Slanting a text object to the right gives the characters an italic look.

#### Ring Mode:

The Ring Mode property is used to control the writing of text on the radius of a ring.  Positive values will produce clockwise ring writing, meaning the bottom of the characters point towards the ring center.  Negative values produce counter-clockwise writing.  A value of zero produces normal linear marking.  The Position property is used to define the X/Y position of the marking.  The Rotation property is used to define the angle on the ring to be marked.  In other words Position, and Rotation are use to back calculate the center of the ring.  Justify, Spacing, and Italic work on ring writing just as they do on linear writing.  The Size/Length property refers to arc length when writing on a ring.  A paragraph of ring writing will produce a series of curved text lines, with the ring center moving down by the value of Spacing/Line.

#### Justification:

The Justification properties control how the object is drawn about the Absolute position.  The most common Justification is horizontal and includes Left, Right, and Center.  Left Justify is the way that most people and machines write text, with each line of text beginning at a left hand margin specified by the X Position property.  Right Justify text is written so that the end of text lines up on a right hand margin, specified by the  X Position.  Center Justify text is written so that the center of each line falls at the X position value.

Examples:

- Left Justify text

is written so that

the beginnings line

up.

- Right Justify text is

written so that

the ends line up.

- Center Justify text is

written so that

the middles line up.

#### Spacing:

The Spacing properties are used to control character and line spacing of text objects.  When Character Spacing is set to zero, the character will be drawn using the natural (fixed or proportional) spacing, built into the font.  Otherwise, the characters will be drawn with a fixed spacing defined by the value entered.  ProLase character Spacing refers to the spacing from character center to character center, not the intra-character space (space between characters).  For fixed spaced fonts like the Quantrad fonts, the ProLase Spacing value is set equal to the desired intra-character space plus the programmed character width (Length property).  Normally the built-in font spacing should be used, (Spacing set to zero); exceptions would be rulers and dials where characters need to be marked at specific locations.  When in fixed spacing mode (any Spacing value other than zero), the Size/Length parameter controls the width of each character.  When in natural spacing mode (Spacing set to zero) the Size/Length parameter controls the length of the entire line of text.  Spacing is easiest to understand by setting Length to zero, adjusting Character Spacing, and finally adjusting character width (Length).  Character Spacing must be greater than Length, otherwise the characters will overlap (except when Character Spacing is set to zero in which case proportional natural is used and Length refers to sentence length).  In fixed spacing mode, characters are automatically centered at the defined locations.  X Position and Y Position define  the first character position and the Spacing value is used to calculate subsequent positions.  This greatly simplifies graduation applications like rulers,  calipers,  meters, and dials.

If a limited "fixed space only" system (like the Quantrad) has been used in the past, this interaction between Spacing and Length may seem awkward at first, but once understood, it provides much more flexibility.

#### Size:

The size properties are used to control object height and length.  When Length is set to zero the graphic length is made proportional to the graphic height.  When Length is greater than zero then the defined length is used to adjust the object.  The object is adjusted to fit in a rectangle with dimensions of Length X Height.  This feature can be used to compress, or change the aspect ratio, of an object.  It can also be used to ensure that variable text will fit in a defined area no matter how many characters it contains.  To mark "blocked" text, the text in this manual for example, a Length is selected that is somewhere between shortest and longest sentences in a paragraph.  Each sentence in the paragraph is placed in a separate object.  Minimizing the difference in length between the shortest and longest sentences will optimize the appearance.

Remember that when a Character Spacing is defined (Character Spacing is set to anything but zero), Length refers to the length of each individual character not the length of the sentence.  Conversely, if Character Spacing is set to zero (natural character spacing) then Length refers to the sentence length.  This makes sense if you consider that fixed spaced characters must have a constant sentence length.  For example, 4 characters with a 0.5" spacing will allows have a sentence length (center of first character to center of last character) of 2" regardless of the character size.

In character sets, the height refers to the height of a rectangle completely enclosing a normal capital letter, like "A".  The bottom of the box is called the baseline.  Some letters, like "acemnorsuvwxz," are smaller than the box, and letters like "gjqy" descend below the bottom of the box.

### Variable Text:

The Variable Text properties effect the way variable text objects operate.

Variable text is text information that changes during processing.  Examples of changing text are serialized numbers, date codes, and operator inputs.  When a Variable Text type is selected, the system will accept a prompt property for the required data; this prompt is issued to the operator when the Document is selected for run.  At runtime, the prompt will display a default entry.  In the case of Date Code, the default is the current date.  In the case of a Serial Number, the default is the next number to mark.  The prompt, along with a Sample Response can be entered as Properties.  Variable Text Objects are limited to 1 line of marked data.  At runtime, the operator input can be entered or the operator can press <Enter> to use the default input.  The Prompt Timing property can be used to disable operator prompting for Date Code or Serial Number.  If the Document is saved after a run, then the last used input data becomes the new default data.  If a Sample Response is entered then it will be displayed when manipulating the object, otherwise the prompt will be displayed.

#### Keyboard:

The Keyboard input function is used for manual entry of data to be used while running a mark program.  For example, if parts are marked with a company logo and a person's name, the names can be manually input by the operator.  If Prompt Timing/Batch is selected then any number of parts can be marked with the same data, and then another batch can be marked with different data.

When Keyboard is selected, the system will ask for an operator prompt; this prompt is used to inform the operator that data is required and should indicate what type data is to be provided.

#### Date Code:

This is used to mark the current (or operator input) date.  This date is automatically updated based on the PC date variable.

When selected Date Code will request an operator prompt, just like Keyboard.  The actual prompt at runtime can be suppressed using the Prompt Timing/Never selection..

#### Serial Number:

Serial Number is used to select automatic serialization of a string of alphanumeric data.  When selected Serial Number will request an operator prompt and an increment amount.  The actual prompt at runtime can be suppressed using the Prompt Timing/Never selection.

To serialize with an operator prompt for a starting serial number, set Prompt Timing to Batch.  If the current default data is null, then the operator will be prompted for a starting serial number no matter how the system is configured.  The marked serial number always contains the same number of characters as the starting or default data.  For example, if the starting data is "0000" then the number will be incremented until it reaches "9999", at this point it will wrap around to "0000" again.  "0" wraps at "9", "A" wraps at "Z" and "a" wraps at "z".  Mixed data like "Ab5cD" is valid and each character will wrap as described above.

- An object in a non-marked layer can be used to keep a running total of parts marked.

#### List File:

The List File selection is used to set up sequential input of data from a disk file.  At runtime, List File allows a text string to come from a disk file such as a mailing list.  Each time the program is run for marking, the next record in the file is used to fill in the text.  Thus, a series of parts with different data can be run without operator input of data.  For example customized gift pens can be marked using a list of names from a simple ASCII mailing list file.  If multiple Variable Text objects in a program are defined as List File and are linked to the same text file, then the next record is read for each object.  For example if the file is set up with a name, address, and telephone number for each person, then 3 records must be read from the file for each name required.  To read a record without marking it, simply define a Variable Text List File object in an unmarked layer and use the same file name.  Multiple files can be used as input for one program.  For example, object #1 might get a name from NAME.LST and object #2 could get an address from another file called ADDRESS.LST.  In this case, the files must match, meaning the 101st name in NAME.LST goes with the 101st address in ADDRESS.LST.  This combination of mixed data in a file, and multiple files used by a mark program, provides a powerful link for variable data.

List Files can also contain ProLase modifiers.  Modifiers allow the user to change the properties of a list file object before marking.  This means the size, location and other object properties can be adjusted by commands within the list file record.  The commands are in the form “/c=pppp/“ where c is the one character command and pppp is the parameter.  In the case of size, position, and angle the parameter is a number that can contain decimal points and sign.  For example to move the X position to 1.5” left of center the command would be “/x=-1.5 “. Some parameters such as file names are simple ascii strings, for example to mark a graphic use the command “/g=C:\proplus\graphic.dxf/“.  The graphic command must include the complete path to the graphic.  Notice that every ProLase modifier command ends with a “/“ character.  Everything after the “/” is either another command or part of the list file data to be marked.  If the user wants to use the “/” character in the marked data, simply put two of them in a row (i.e. “//” = “/”, “///” = “//”, etc. when marking).  Make sure that there is a space between command sets.  When marking the list file data all modifier commands are removed.  All object properties remain at their last set value until changed by another modifier command.  The original property settings are as programmed in the LAZ file for the list file object.  The following is a list of the modifiers:

/a=pppp/		Object angle in degrees			/a=90.0/

/x=pppp/		Object X position 				/x=.053/

/y=pppp/		Object Y position				/y=-1.2359/

/h=pppp/		Object Height				/h=.5/

/l=pppp/			Object Length				/l=2.0

/g=pppp/		Mark graphic				/g=C:\proplus\columbia.dxf/

/m=pppp/		Use Material File 				/m=FALSE/ , /m=TRUE/

/f=pppp/		Object Material File Freq adj.		/f=100.0/ (in percent, /m=TRUE/)

/F=pppp/		Object Frequency				/F=5.0/ (in KHz, /m=FALSE/)

/p=pppp/		Object Material File Power adj.		/p=100.0/ (in percent, /m=TRUE/)

/P=pppp/		Object Power				/P=25.0/ (power units, /m=FALSE/)

/s=pppp/		Object Material File Speed adj.		/s=100.0/ (in percent, /m=TRUE/)

/S=pppp/		Object Speed				/S=5.0/ (in /sec, /m=FALSE/)

/r=pppp/		Set Size to Real				/r=TRUE/

/R=pppp/		Set Position and Size to Real		/R=TRUE/

EXAMPLE LIST FILE:

/h=.2/ /x=.5/ /y=-.5/ American Laserware

/y=-1/ Any Data

/y=-1.5/ More Data

.

.

.

In this example the first record will mark the word “American Laserware” .5” to the right and .5” below center, with a character height of .2”.  The second record will mark the word “Any Data”  1” below center, the x position remains at .5” to the right and the height remains .2”.  Although modifier commands can be place anywhere within the data string, it is best to put all commands at the front of the string, and the data at the back.

The prompt option is available with the List File selection.  This prompt is used to allow the operator to change the file name at runtime, allowing the same program to be run with different mailing lists.  After the prompt, the List File selection will request a file name.  This file name will be used as the default file name on the next run.

#### Dynamic File:

The Dynamic file selection is used to read data from a changing disk file.  Each time the object is marked, the first line of a text file is used to fill in the text for the object.  Unlike the List File, the text file is not kept open from mark to mark.  The file specified with the Dynamic File selection is closed after the data is acquired from it.  This allows the file to be changed dynamically from mark to mark.  The file must be changed by an external application.  This can be done with the Call User Control step of the Layer IO(see Layer IO for more information).  The executable file specified must open, modify, and close the same text file to which the object is linked.  Multiple objects can be linked to the same text file.  These objects will mark the same data unless the file is modified between them, such as if they are on different layers.

### Laser Control:

The Laser Control Property sheet is used to control the major process parameters in laser marking.  The main process parameters are writing speed, laser power, and pulsing frequency.  If Use Material File is selected then the values for these parameters come from whatever material file the Document is linked to.  These values can be adjusted for each Object using the provided slider controls.  The controls can be set between 1% and 200% of the material file values.  This gives the operator individual object control, while maintaining a connection to the material database.  If the user prefers to enter actual values (speed, power, and frequency) for each object and not use the database, then simply uncheck Use Material File and enter the desired values in the provided entry boxes.

#### Conditional Mark:

The Conditional Mark property provides conditional marking based on the digital I/O port (DIO).  Each object can be programmed to mark only if the DIO is in a specific state.  Programming this value to XXXXXXXX (grayed checks) means the object will always be marked (the default).  This might be used is a marker connected to a test station.  The station tests the parts and sets the DIO port to a value corresponding to an object to mark.  For example, if we have 4 possible results from the tester we can program four different objects at the same position and size in the mark field.  We can give each of these objects a unique Conditional Mark value:

Object			Conditional Mark		Marked Data

1			XXXXX00X			A

2			XXXXX01X			B

3			XXXXX10X			C

4			XXXXX11X			D

5			XXXXXXXX			Logo, text, etc.

.

.

.

When the marking is initiated, only the object whose Condition Mark value matches the state of the DIO port will be marked.  In this example, text and logos common to all parts can be programmed with a Conditional Mark value of XXXXXXXX (don't care).  This program will mark parts with A, B, C or D depending on the state of the I/O when the START signal is received.

#### Wobble:

Wobble provides a way of altering the beam path/shape by superimposing an additional ‘wobble’ signal to the normal path taken by the laser spot.  Most frequently this is an older method of ‘fattening’ lines, but it can be used to achieve other aesthetic effects.  The wobble settings vary greatly with the desired effect, response of the galvos, and marking speed used.  Users should experiment to become familiar with wobble settings for their machine.  Generally, the marking speed will decrease and the mark time will increase when wobble is used Wobble Width determines the amplitude of the wobble signal.  A wobble width of zero indicates no wobble.  The units for the wobble width are in 1/1000 of the selected units.  The actual increase in line width might vary from the setting, as it is dependant upon hardware response.

The wobble Type setting determines the shape of the superimposed wobble signal.  For example, the circle type will add a circular signal to the normal linear motion, resulting in a spiral shape. The wobble Denstiy setting corresponds to the frequency of the superimposed signal.  From the circle type example above, with a low density, you may be able to distinguish the individual spiral path of the galvos (provided the wobble width is large enough and the marking speed is slow enough).  At a higher density, the resulting mark might look like a completely solid line.

NOTE:  The SCANLAB RTC cards have built in hardware wobble generators that operate slightly different that the regular ProLase wobble.  If a SCANLAB RTC driver is selected in ProLase, a different set of wobble controls is displayed: Wobble Width and Wobble Freq.  The Wobble Width is the same.  The Wobble Freq sets an oscillation frequency and acts similarly to the Density setting above but has units of Hertz.  The SCANLAB RTC cards only support the circle wobble type.

# Touring the Menus

ProLase is an interactive graphics editing system used to define marking Documents.  It includes provisions for defining a series of objects, which can include imported graphic images, fixed text, and variable text.  This series of objects can be stored on disk, as a LAZ document file and later recalled for further editing or execution.  The system is menu driven, using the keyboard  or mouse for control.

- Main Menu:

FILE  Edit   View    Insert   Tools   Marking   Preferences    Window   Help

## File Menu:

New					Ctrl+N

Open					Ctrl+O

- Close

Save					Ctrl+S

- Save As

- Import Graphic File

- Import ALI File

Mark					Ctrl+M

Print					Ctrl+P

- Print Preview

- Print Setup

- The File menu is used for common file functions such as opening, saving, importing, etc.

### New:

New is used to load the "LAZ.NEW" document from the disk.  This is essentially a template document containing user defined default information.  A LAZ.NEW can be created and changed like any other mark Document, but it must be saved using another name, and then renamed LAZ.NEW outside of ProLase.  This prevents accidental modifications to the template.

### Open:

Open is used to load a Document file for editing, execution, or adding to the queue system.  Each mark Document (.LAZ) file contains the graphic information that defines each object in every layer.  This file also contains links to the material process database, the fixture database system, fonts, and imported graphic files.  Many sub-directories of .LAZ files can be on a hard drive.  Therefore, many thousands of Documents with corresponding links can be managed by ProLase.

When selected the Open function will prompt the user to save the current file (if changed) and then display a directory of existing mark documents.  The directory starts with the current file displayed in the file input box.  Once the desired file is highlighted or entered, the OPEN button is used to load the file.

The system will check for the existence of all external links, such as PLT or DXF graphic files.  If a required file is not present, then the system will inform the user and request a replacement file.  The User can select a new file or CANCEL to replace the file with a rectangle at the position and size of the missing object.

After resolving external file references, the system will draw the loaded Document.  ProLase is a multi-document interface, and therefor the user can have many LAZ documents loaded at one time.

### Save:

Save file is used to save the current state of a mark Document to disk.  The file is always saved under the name that was used to open the file, or the name last used in the Save As function.

### Save As:

Save As is used to write the current mark Document to disk.  When selected the system will prompt the user for the file name to use when writing the file.  This name becomes the current Document file name until changed by another Save As.  Writing a file to its current name is the same as the Save function.  A copy can be accomplished by using Open to read a file and Save As to copy it under another name.  Sometimes it is easiest to create a new Document by copying and modifying an existing one.

### Import Graphic File:

Import Graphic File is one method of inserting a graphic object into the currently selected Document.  Right clicking in the graphic area and selecting the graphics tool bar button are other methods of accomplishing the same thing.

### Import ALI File:

Import ALI File is used to import a mark document (.ALI) file from the DOS version of ProLase.  The Importation process includes converting the ALI file to the LAZ format.  The importer automatically converts linked DOS fixture and material files to their Windows counterparts.  The converted files are save at the same sub-directory as the original  ALI, FIX, and APS files.  The extensions are different for the Windows versions, so there is no interference between the two systems.  It is possible to have and run both the Windows and DOS versions of ProLase on the same PC, using the same sub-directories.

### Mark:

Mark is used to mark the current Document on the laser marking system.  This menu selection is the same function as the MARK tool bar button.  This is designed for testing or low volume marking.  Only one part is marked and the external I/O control start signal is ignored for the first layer, marking begins as soon as the selection is made.  If marking a multilayer document the process will stop between layers, display a dialog and wait until the operator wishes to continue, by either software selection or hardware I/O.

Complete automation control and batch marking is accomplished using the MARKING/MARK CONTROL dialog.

- Main Menu:

File   Edit   View    Insert   Tools   Marking   Preferences    Window   Help

## Edit Menu:

- Undo

- Cut

Copy

Replace

Insert

Delete

Duplicate

A note about ProLase Edit Menu features:

ProLase differs from most Windows programs in that its edit features do not include a paste feature.  In most programs, paste serves two functions.  Either it can replace selected items with the clipboard data, or it can insert the clipboard data into the document without removing any existing items.  In applications such as word processors, the cursor indicates where the clipboard data will be inserted, or highlights the text that will be replaced.  However, the ProLase environment is not conducive to this format as there is no text cursor.  ProLase breaks the paste feature into its two sub-components, Insert and Replace.  These two items effectively replace the functionality of the paste command.

Undo:

Undo will undo the last action that the user made in ProLase.  For example, if the user deleted an object accidentally, selecting Undo will bring the program back one step to where that item still existed.

### Cut:

Cut will copy the currently selected object to the clipboard and delete the object from ProLase.  It can be inserted back into the LAZ file later.  Note that it will be lost the next time that an object is copied to the clipboard.

### Copy:

Copy will copy the currently selected object to the clipboard.  Only objects can be copied to the clipboard.

### Replace:

Replace will replace the currently selected object with the object in the clipboard.  Note that the currently selected object will be lost when this feature is used.

### Insert:

Insert will place the clipboard object into the Layer / Object list ahead of the currently selected object.  If the layer itself is selected in the Layer Object Layout Window, the clipboard object will be inserted as the last item in the layer.

### Delete:

Delete will delete the currently selected object.  It will not copy the object to the clipboard.  Any object currently stored in the clipboard will remain valid.

### Duplicate:

Duplicate will create a new object identical to the currently selected object.  It will insert the new object at the end of the current layer.

- Main Menu:

File   Edit   View    Insert   Tools   Marking   Preferences    Window   Help

## View Menu:

- Render Fills

- Zoom  >

- Center

- Zoom In

- Zoom Out

- Center At

- Zoom To

- Zoom From

- Zoom In/Out

- Zoom Field

- Zoom Default

- Zoom Save

- Standard Toolbar

- Stock Objects Toolbar

- Object Controls Toolbar

- Zoom Toolbar

- Status Bar

- Layer/Object Info

- Layer Tabs

- Layer Tabs Setup

### Render Fills:

The Render Fills control is used to force ProLase to calculate the hatch fills for the current Document.  ProLase will automatically calculate the hatch whenever they are required for actual marking.  Whenever the user modifies an object property that changes the hatch (size, hatch density, hatch angle, etc.) the hatch is removed from the drawing area, and only the object outline is displayed.  This allows the user much smoother object manipulation as the hatch calculations take some time.  At anytime the user can select View/Render Fills to calculate and display the hatch.

### Zoom:

The Zoom functions allow the user to adjust the drawing display.  Zooming in allows for viewing of drawing details.  Zooming out allows for viewing of the entire drawing area and beyond.

#### Center:

The Center function is used to redraw the graphics screen about the real world coordinates defined by the user.  The cursor function is used to provide the position input.  The cursor will only redraw about a point currently on the screen.

#### Zoom In:

Zoom In used to magnify the graphics, centering on the current cursor location.  When selected, the magnifying glass cursor will be shown, and can be controlled by use of the mouse.  When the desired center is selected, the left mouse button is used to initiate the Zoom.

#### Zoom Out

Same as Zoom In except the area shown gets larger.

#### Zoom Default:

The Zoom Default level is the last zoom level selected prior to the execution of a Zoom Save command.  This zoom level is stored with the file when the file is saved.

#### Zoom Field:

The Zoom Field command is used to display the physical area of the marking device as defined by the Field Size configuration parameter.  In DAC devices, this is the lens field area, and in HPGL systems, this is the plotter area.

#### Zoom Save:

Zoom Save is used to save the current zoom, center and grid information as the Zoom Default settings.  This information will be save on disk with the mark file information if a subsequent File/Save or File/Save As is executed.  When first drawn after a File/Open, a file will be displayed using the stored Default Zoom information.

### Standard Toolbar:

This menu control allows the user to toggle on/off the display of the standard toolbar.  This common function toolbar includes buttons for several often used functions:

- File Tools:

- New

Open

Save

- Edit Tools:

- Cut

Copy

Insert

Undo

Output Tools:

Mark

Print

Display Tools:

Layer/Object Layout

Grid Settings

Modeless Dialog Displays

Mark Dialog

I/O Monitor

Material File Editor

Fixture File Editor

Help

Help System Contents

Context Help

Each of these toolbar buttons has a corresponding main menu selection that accomplishes the same function

### Stock Objects Toolbar:

This menu control allows the user to toggle on/off the display of the stock Objects toolbar.  This cursor control toolbar includes buttons for drag cursor, zoom cursor along with text, graphic, square, rectangle, circle, ellipse, and line creation cursors.

### Object Control Toolbar:

This menu control allows the user to toggle on/off the display of the Object Control toolbar.  This toolbar includes common functions used in control of object properties.

- Mirror:

- Mirror about X axis

Mirror about Y axis

- X Justification:

- Left

Center

Right

- Y Justification:

- Bottom

Center

Top

- Aspect Mode:

- Free

Natural

Fixed

Squeeze

- Angles:

- Ring Mode

Rotation

Tilt

- Fill Mode:

- Outline Only

Fill Only

Both

Unidirectional Fill

Bidirectional Fill

Live Fill

Render Fills

- Graphic:

- Show Blanking

- Tool Path Optimizer

Reload Graphic

Each of these buttons corresponds to an object property.  The current value of the properties can be viewed or adjusted using the object property sheets.

### Zoom Toolbar:

This menu control allows the user to toggle on/off the display of the Zoom Control toolbar.  This toolbar includes common Zoom functions:

- Zooms:

- Center

Zoom In

Zoom Out

Zoom Field

Zoom Default

Zoom Save

- Main Menu:

File   Edit   View    Insert   Tools   Marking   Preferences    Window   Help

## Insert Menu:

- New Layer

- Delete Layer

- Layer Properties

- New Object  >

- Fixed Text

- Variable Text

- Graphic

- Data Link

- Empty

- Delete Object

- Object Properties

### New Layer:

The New Layer menu item will insert a new layer into the current document.  A dialog asking the user to name the layer will appear.  A unique layer name must be used (or leave as the default name).  The Layer is then inserted into the document.  See the section above on creating layers and objects for further details.

### Delete Layer:

The Delete Layer menu item will delete the currently selected layer from the document.  This will remove the layer and all objects within that layer from the document.

### Layer Properties:

The Layer Properties menu item will open the Layer Properties window.  This gives the user access to all of the layer features, such as I/O settings, activation flags, layer name, etc.  See the section above on creating new layers and objects for further details

### New Object:

The New Object menu item allows the user to create a new object in the document.  The object will be one of the following types:

#### Fixed Text:

This item displays text in any desired font.  The text is static and will not change unless the user explicitly alters it in the object properties window.

#### Variable Text:

This item displays text in any desired font.  The single line of text can change between marks.  The text is generated by a number of sources including serial numbers, text files, or sources external to ProLase.

#### Graphic:

This item displays a graphic image.  Numerous graphic formats are supported by ProLase.

#### Data Link:

This item will use “data” from another item (either text or graphics).  The sizing, orientation, font, etc. are independent of the data source, but it will display the same text or graphic as the other item.  Any modifications to the source item (either by the user or by external sources) will be reflected in the data link item.

#### Empty:

This item has no display.  It is a placeholder and can later be modified to be a display item

### Delete Object:

The Delete Object menu item will delete the currently selected object from the document.

### Object Properties:

The Object Properties menu item will open the object properties window.  This gives the user access to all of the object features, such as position, orientation, graphics, fonts, text, etc.  See the section above on creating new layers and objects for further details.

File   Edit   View    Insert   Tools   Marking   Preferences    Window   Help

## Tools Menu:

- Snap Settings  >

- Grid

- Rotation

- Italic

- General

- Mirror  >

- X

- Y

- Justification  >

- Left

- Center X

- Right

- Top

- Center Y

- Bottom

- Aspect Mode  >

- Free

- Natural

- Fixed

- Squeeze

- Ring Mode

- Rotate

- Italicize

- Fill Type  >

- Outline Only

- Hatch Only

- Outline & Hatch

- Convert Files

### Snap Settings:

The Snap Settings are used to overlay grid points, or snap-to points, on the marking field.  These points allow for more accurate placement and orientation of the objects in a ProLase file.  Each submenu item (Grid, Rotation, Italic, and General) brings up a tabbed dialog box that allows the user to manipulate the snap settings.

#### Grid:

When Show Grid is checked, the grid will be displayed in the edit window, covering the mark field, when it is unchecked, the grid is not displayed.  When Snap to Grid is checked, objects being moved will jump between grid points.  The position handle of the object will snap to the grid points, allowing for precision placement.  When it is unchecked, free movement of the objects in the drawing area is allowed.

The Grid Increment drop list allows for the selection of the grid spacing.  These values are in the same units as the object placement and sizing values.

The Grid Type drop list toggles the grid between lines and points.  When Lines is selected, the grid is displayed as a cross hatch across the field, when Points is selected, only the points to which objects snap are displayed.

#### Rotation:

Show Rotation and Snap to Rotation behave similarly to Show Grid and Snap to Grid.  However, when the Rotation snap settings are shown, they appear as a ring of small red dots centered about the position handle of the object.  Either one or both ends of the object will fall on this circle.  As the object is rotated, it will snap to the angular settings indicated by the rotation snaps.

The Rotation Increment drop list allows for the selection of the angular increment.  These values are in degrees.

#### Italic:

Show Italic and Snap to Italic behave similarly to Show Grid and Snap to Grid.  However, when the Italic snap settings are shown, they appear as a ring of small green dots centered about the position handle of the object.  When the italic handle is manipulated, the italic angle will snap to these settings.

The Italic Increment drop list allows for the selection of the angular increment.  These values are in degrees.

#### General:

The General submenu item brings up the Snap Settings dialog with the most recently used tab selected.

### Mirror:

The Mirror submenu allows for the toggling of X and Y mirror mode on the currently selected object.  When Mirror X is selected, the object is mirrored about the vertical axis going through its position handle.  When Mirror Y is selected, the object is mirrored about the horizontal axis going through its position handle.

Mirroring about both axes is equivalent to rotating the object 180 degrees.  Mirroring about one axis will cause the object to be a “mirror image” of the original.  This is useful for images on the back of transparent materials.

### Justification:

- Left

Center X

Right

These three menu items toggle the selected object’s horizontal (X) justification.  When a particular menu item is selected, the justification handle of the object will be on the left vertical line of the object, the center vertical, or the right vertical.

- Top

Center Y

Bottom

These three menu items toggle the selected object’s vertical (Y) justification.  When a particular menu item is selected, the justification handle of the object will be on the top horizontal line of the object, the center horizontal, or the bottom horizontal.

Using these two settings, any one of nine possible justification points can be selected.  This point will serve as the position point of the object.  All rotations are centered about this point.

### Aspect Mode:

The Aspect submenu toggles the aspect mode of the currently selected text object.  This menu does not apply to graphics.  The aspect settings control how an object behaves to stretching with the graphic handles.

#### Free:

In free aspect mode, the object’s height and width can be freely manipulated with the graphic handles.  Wherever the user drags the handle to will set the height, length, or both values of the object.  In this mode, height and length are independent values.

#### Natural:

In natural aspect mode, the object’s height and width are set at a fixed ratio.  The characters will appear in their standard, or natural, aspect.  They will not appear squeezed or stretched at any size.  As either height or length is manipulated, the other value will be altered to maintain the natural aspect of the text.

#### Fixed:

When fixed aspect mode is selected, the current aspect (height to length ratio of the characters) is maintained.  From that point on, the characters will maintain the same shape, growing or shrinking as they are manipulated.  This is similar to the behavior of natural aspect, with the exception that the aspect is now defined by the user.

#### Squeeze:

When squeeze mode is selected, the object’s length and height control the size and shape of the characters.  When the text string is small enough to fit within the object’s length, it will appear in natural aspect.  As the string becomes too large to fit in this area, the characters will be shrunk so that they will fit into the box defined by the height and length of the object.  This mode is useful for marking text of unknown length (i.e., variable text) into a fixed size window.

### Ring Mode:

This item toggles a text object between standard and ring mode text.  This does not apply to graphic items.  Ring mode text is drawn such that the baselines of the characters form a circle whose radius is defined by the user.  The text can be drawn clockwise (with a positive radius) or counter-clockwise (with a negative radius).  When the ring mode button is used to toggle the text back and forth between standard and ring mode, the default radius will generate clockwise text over an approximately 90 degree arc.

When text is in ring mode, two additional graphic handles will appear.  A blue handle appears at the center of the circle formed by the text.  This handle is used to set the radius of the text.  It can be dragged from one side of the text to the other to change from clockwise to counter-clockwise.  The second handle is a red rotate handle.  This will rotate the text about the center of the ring.  Note that this will change the position value and angle of the text object, but it will keep it centered about the same point in the editing field.

### Rotate:

The Rotate menu item will rotate the selected object by 30 degrees clockwise.

### Italicize:

The Italicize menu item will step the text object through varying degrees of italic angles, both positive and negative.  This does not apply to graphic objects.

- Main Menu:

File   Edit   View    Insert   Tools   Marking   Preferences    Window   Help

## Marking Menu:

- Mark Control

- Queue

- Fixture

- Material

- Config

- Monitor

- Center Galvos

The Marking Menu contains functions that control how the system works with the operator and equipment.  This menu includes the Mark Control for production marking, access to the material and fixture databases, system configuration, hardware monitor, and a center galvo function.

### Mark Control:

The Mark Control selection is used to present the operator with the Mark Control Dialog.  This Dialog is the main operator interface for production marking of parts.

The Mark Control allows the operator to select a mode for marking, the number of parts in a run, and provisions for simulating a mark process to the CRT screen.  This Dialog also displays run status during the mark process, displays the cycle time for the mark, and displays the total number of parts marked so far in the run.  The currently used fixture and material file names are also displayed.

#### Part Count:

The Part Count display tells the operator how many parts have been marked thus far during the current run.  The Part Count is reset to zero whenever the run is stopped because the run was completed or because the mark mode was aborted.

#### Number of parts in run:

The Operator can instruct the system to run a specified number of parts in a run.  Simply enter the number of desired parts to mark.  If the number of parts to be marked is unknown, the operator can select Continuous, which instructs the system to mark until the run is aborted by the operator.  In other words, this instructs the system to run an infinite number of parts.  Most automatic assembly line markers run in this mode.

#### Mode:

The system can mark in one of three modes.  Manual mode a marks one part at a time and ignores any hardware start signal requirements for the first layer.  Marking starts as soon as one of the Enable Run buttons is pushed.  The Mark toolbar button performs the same function.  This mode is used for testing and “one off” type processing.

Auto Mode is used for automated or batch processing.  All start signal requirements must be met before marking starts.  To run in Auto Mode, select AUTO, enter the number of parts in the run, Enable Run, and provide the required start signals.

Light Show mode is for testing.  In this mode, marking starts as soon as the run is Enabled.  Start I/O signal requirements are ignored.  The marking will be automatically repeated as quickly as possible giving the effect of a laser light show.

Alignment is checked when the user wishes to align a part correctly before actual marking.  When checked on Alignment objects are marked.  An object can be defined as an alignment object by unchecking the ACTIVE check box in the objects GENERAL PROPERTIES.  Usually Alignment is used in combination with Light Show mode, using the Setup marking.

In the Mark Dialog you will notice a button called SETUP.  This button runs the LAZ file just like the MARK button except it uses a different MATERIAL file.  Usually named SETUP.MAT, this material file allows you to set parameters that do not mark but allow you to see where the mark will go.  On many systems (those with visable pointing lasers) this simple means that you set the power to zero in the SETUP material file.  We do it this way for flexibility, not all customers markers have pointing lasers, so they set the power very low and the frequency very high so that they can see the beam on the part without actually marking the part.

The second feature which can be used with setup is the ability to set objects as ACTIVE or ALIGNMENT.  You set this using the check box on the GENERAL property page (Object properties).  ACTIVE objects are the normal kind that you mark on your part.  ALIGHNMENT objects are used to align your part.  Most often customers use an outline of the part itself as an ALIGNMENT object.  Some customers use rectangles around the ACTIVE objects, and other customers use registration points.  You can do whatever you want since you create ALIGNMENT objects the same way you create regular objects (graphics, boxes, lines, etc.).  Many software packages simply mark boxes around objects as the ALIGNMENT feature, but many customers consider this too restrictive, that is why we have expanded our alignment feature to mark anything.  In the MARK DIALOG window you can check the box that says "Aligment" and press the SETUP button.  You can select LIGHTSHOW to make your alignment objects continously mark, giving you time to physically align your parts.  When you are ready to actually mark, simply uncheck the Alignment box, select AUTO or MANUAL and press the MARK button.  Of course you can also not check the alignment box and just press the SETUP button.  This will use the SETUP material file and display the actual objects.  You can do this in LIGHTSHOW mode if you wish.

Dont forget that you can also define a LAYER as a background layer, this is normally used to display a outline of the part that is calibrated to match the position of a part that is fixtured.  This way operators can place marks based on the position of the picture of the part in the background layer, if calibrated correctly the marks will be in the same place on the real part as it is on the picture of the part.  Background layers can also contain special instructions, and annotations.

#### Enable Run:

This selection is used to tell the system how to process the run, either as a live marking or as a simulation.  The simulation marks to the CRT just like it would to the live marker, including processing of machine control I/O and variable text processing.  This mode is used to test the function of the Document and to train operators without using an actual laser marker.  Setup run mode is the same as Mark except a special Setup material file is used.  The Setup material file is configured to a low or no power and high speed so that the galvos move but no actual marking of the part occurs.  This is so that a part can be aligned with the markings before actual marking.  If a HeNe alignment laser is available the Setup material file should use 0 for the power setting.  If no HeNe is available a very low power should be used so that the path of the laser can be seen, but no actual marking occurs.  Setup can show either the actual markings or alignment objects depending on the ALIGNMENT check box.

### Fixture:

Each document (LAZ) file is "linked" to a fixture (.FX2) file.  The fixture file describes what type of offset and step & repeat process is to occur during the execution of the program file.  Most laser marking software stores these parameters in the program file, requiring modification of each program file if the fixture changes.  The advantage of ProLase, with the fixture control parameters stored in a separate file, is that editing one fixture file can change the fixture setup for many program files.  For example, let’s assume the user has three different products that use the same 5x5 nested fixture.  There is a program file for each of the products, and each is linked to a fixture file called "5x5.FX2".  If the fixture needs to be offset to the right 0.5" and the fixture data needs to be changed to compensate, only the "5x5.FX2" file is changed, automatically adjusting all document files linked to it.  If a user has many document files, and only a few different fixtures, this feature becomes a very powerful way to manage the processes.  Whenever a File/Save or File/Save As is used to save a program file, the current fixture file is automatically linked.  To see which fixture file is currently loaded use either the Config/Information command or the Fixture/Load command (the directory will have the currently loaded fixture file highlighted).  DEFAULT.FX2 is the default fixture file.  The default data contained in this file can be edited by the user, but usually defines a fixture of 1 part with a zero value for all offsets.

The name of the current fixture file is saved in the Document file, when the program is stored with a File/Save or File/Save As command.  This becomes the linked fixture file for the program.  The linked fixture file is loaded automatically when the Document file is loaded (by a document File/Open command or automatically by the job queue feature).  At any time after loading a mark program, another fixture file can be loaded and used.  If a mark program is saved after loading a new fixture file, then the new fixture file is linked to the program.  This feature allows the same program to be easily marked on different fixture setups.

Mark Control/Simulate can be used to preview a program file with all fixture data incorporated, including step & repeat.

#### Offsets:

The Offset X and Offset Y values are used to offset the X/Y position of all objects in all documents linked to the fixture file.  The position is offset from the absolute positions defined for each object.  This feature is used to compensate for fixture offsets.  For example, consider a single part fixture is designed to be centered at the mark field center.  For some reason, the fixture is manufactured so that the fixture center is 0.1" to the right and 0.2" below the mark field center.  Instead of re-manufacturing the fixture the Offset values can set to X = +0.1" and Y = -0.2", compensating for the offset fixture.  When step/repeat is used, the first part in the array is positioned at the programmed object absolute X/Y positions, subsequent array positions are ‘stepped’ from these points.  Use Register to offset the entire array.

#### Angle:

The Angle function is used to offset the rotation a mark program.  The entire mark field is rotated, allowing this feature to compensate for any fixture rotations including nested part fixtures.

#### Focus Adjust:

The Focus Adjust parameter is used to offset the height of the optical system to the part.  This usually means the user must provide a mechanism that either moves the entire laser/optics system or moves the part.  The Focus Adjust value is added to the Layer Z Table value to determine the total Z position.  Focus Adjust is like the Offset X/Y parameters in that it is designed to compensate for fixture error.

#### Array X, Array Y:

The Array X and Array Y properties define the number of parts in a nested array.  Array X describes the number of parts in a row and Array Y describes the number of parts in a column.  This information along with the Step parameters can be used to define any rectangular array.  The total number of parts in the array is Array X Size times Array Y Size.  Nested arrays are processed in ‘Row major order’ meaning that a row of parts is marked (stepping Delta X between parts) then Y is stepped and another row of parts is marked.

#### Delta X, Delta Y:

The Delta X and Delta Y properties define the amount of distance between parts in a nested array (step and repeat).  Delta X defines the spacing between any two parts in a row, and Delta Y defines the spacing between any two parts in a column.

#### Open:

The File/Open selection provides a directory control box for selection of a fixture (.FX2) file.  The directory will indicate the current fixture file.  The files are referenced by any relevant name the user selects.

#### Save, Save As:

File/Save is used to save the current fixture parameters to a fixture file.  When selected the user will be provided with a Save dialog window with the current file indicated.  To keep the same name the user simply presses the Save button.  To change the name type the new name over the old one.  Remember the current program file will not be linked to the new fixture file unless a File/Save or File/Save As is executed after saving the fixture file under the new name.

### Material:

The Material dialog is the built-in applications data base system.  This system allows the user to develop material dependent processing parameters and store them.  These processes can be recalled later and used in any mark program.  The process files are referenced by material and can be loaded using the directory control provided.  The data values apply to all objects in the program, however the Laser Control property sheet from the Object property pages permits variations between objects.

#### File/Open:

The File/Open selection provides a directory control box for selection of a material (.MAT) file.  The files are generally referenced by a material name.  Each Document (.LAZ) file is linked to a material (.MAT) file.  This file describes what type of laser process is to occur during the execution of the document file.  Control of laser power, Q-switch frequency, and writing speed is defined by the material file.  In most laser marking system software, laser control parameters are stored with the program file, requiring modification of each program file if the process changes.  The advantage of ProLase, with the laser control parameters stored in a separate file, is that the laser process for many program files can be changed by editing one material file.  For example, let’s assume the user has three different products made of the same plastic material.  There is a program file for each of the products (PRODUCT1.LAZ, PRODUCT2.LAZ, and PRODUCT3.LAZ) and all three are linked to a material  file called "PLASTIC.MAT".  If the composition of the plastic material changes and the user needs to adjust the laser process to compensate he simply changes the "PLASTIC.MAT" file, thus automatically adjusting all of the document (.LAZ) files linked to it.  If a user has many document files, and only a few materials, this feature becomes a very powerful way to manage the laser processes.  Whenever a File/Save or File/Save As is used to save a program file, the current material file is automatically linked.  To see which applications file is currently loaded view the LAZ file properties.

- Pass (1...7)

The applications system allows the user to specify up to a seven pass process, with separate laser parameters for each pass.

- Power

- Frequency

- Speed

#### Power:

Power is the desired laser power level, usually calibrated in either watts or amps.  In Nd:YAG systems, equipped with external power control this parameter specifies the lamp current.  A voltage DAC is used to send the power command to the laser electronics.  In CO2 systems, this parameter controls the pulse width modulation to the laser tube.  A PC counter/timer board can be used to generate the pulse width modulation (PWM) signal.  A DAC can also be used if the laser includes pulse width modulation electronics that accept an analog power command.

This power level applies to the programmed pass for the entire mark program.  Individual objects can be marked at a power level that is a specified percentage of this value.

#### Frequency:

On Nd:YAG lasers, this controls the Q-switching frequency (in kHz).  The interface can be a voltage DAC for lasers accepting voltages for external frequency control.  A PC counter/timer can also be used as a MOD IN signal for lasers accepting TTL pulsing commands.  Pulse width is normally a constant, around 10 microseconds.  A frequency of zero indicates modulated CW marking, a TTL gating signal is provided by the PRO LASE system .

On CO2 lasers, this function sets the fundamental control frequency recommended by the laser manufacturer.  This frequency is fixed somewhere between 3 and 20 kHz and usually varies with laser size.  The pulse width modulation of this frequency controls power.  The interface is either a PC counter/timer board, or a DAC (see Power control).

#### Speed:

Speed controls the rate at which the laser beam is moved across the mark plane.  On DAC output systems, an internal timer is used to control the DAC update rate; this, along with the Field parameters, allows the system to control mark speed (distance units/sec).

#### File Save:

File Save is used to save the programmed laser parameters as a material (.MAT) file.  The user will be prompted to either overwrite the current file or enter a new file name.  It is recommended that the name refer to the material being marked.

The name of the current material file is saved in the document file when the program is stored with a File/Save or File/Save As command.  This becomes the default applications file for the Document.  The default applications file is loaded automatically when the Document is loaded.  At any time after loading a mark Document, another material file can be loaded and used.  If a Document is saved after loading a new material file, then that material file becomes the Documents new link.  This feature allows the same Document to be easily marked on different materials.

### Config:

The Config parameters configure the software for the type system being controlled, along with size, resolution, and coordinate system.  Normally these parameters will be set at the factory or during system installation.  The user should not try to adjust any of these parameters without factory advice.  When selected, the following property pages will be displayed:

#### Field Parameters:

This configuration page presents controls that affect the field size.  These parameters help tell ProLase the actual size and shape of the marking field available with the chosen hardware.

#### Field Size:

The Field Size parameter is used to describe to ProLase, the maximum extents of the laser marking systems galvos and lens.  Since program (.LAZ) files use real world units (inches, cm, etc.) changing the field (by changing lenses for example) does not require any Document changes.  In other words, all LAZ files are automatically scaled correctly no matter what lens is used.  One inch is always one inch.  Changing lenses requires calibration of the field size parameter.  To calibrate the systems field, create a square object in ProLase of some size (2” for example).  Enter a guess for the field size parameter.  Then mark the square and measure it with a ruler.  If the drawn square does not match the size programmed, then change the Field Size and repeat the mark.  Keep doing this until the marked square is the same size as programmed for the square object.  The maximum ProLase Field Size is 32 by 32 of whatever units are being used.

##### Field Guide:

The Field Guide controls toggle the display of an outline of the marking field in the working area.  ProLase allows both the typical square field and the larger circular field that circumscribes it.  The circular field yeilds about 40% more marking area.  The Field Guides are controlled seperately.  Thus, only one, both, or no guide may be displayed.

NOTE: The circle option is disabled when the Synrad FLCC driver is selected.

##### Units:

This property tells ProLase what units the user wants to work with.  No matter what units are selected, the maximum Field Size is 32.  If inches are selected, the maximum field size is 32 inches; if meters are selected, the maximum field size is 32 meters.  The positioning resolution of the system is 0.0001 of whatever units are selected.  If inches, the resolution is 0.0001 inch (0.1 mils); if meters are selected, the resolution is 0.0001 meters (0.01 centimeters).  When larger units are used ProLase can support a bigger field, but there is a trade-off in lower resolution.  Most ProLase marking systems use either inches or centimeters.

##### Orientation:

This property can be used to adjust the orientation of the field.  This compensates for the various

orientations of mounting the galvos.

NOTE:  Swap XY will also swap the Aspect Correction when the Synrad FLCC driver is selected

##### Distortion Correction:

This property is used to correct the field for barrel and pincushion distortions caused by X/Y galvo systems.  To adjust the correction, Insert a square object.  Mark the square and take a straight edge to see if the sides are flat.  If they are not flat, either they bow in towards the center of the square (pincushion distortion) or they bow out (barrel distortion).  Minus numbers correct for barrel and plus numbers correct for pincushion.  Change the numbers and repeat marking until the sides of the square become flat.

##### Aspect Correction:

This property can be used to square out a rectangular field.  For various reasons including servo tuning and hardware distortion correction, a marking field may not be exactly square.  Proper distortion correction will ensure that all the lines are straight, but a marked square may actually be rectangular.  ProLase can compress a long axis to make it equal to the shorter axis.  ProLase cannot lengthen the short axis because the DACs are already swinging full scale.  In other words, ProLase can’t make a +/- 10VDC DAC do more than that range.  ProLase can however reduce the range on the long axis.  It should be remembered that using this feature to correct for galvo amplitude sacrifice resolution.

NOTE:  The proper way to square out the field is to adjust the amplitudes in the servo hardware.

To square the field with the software, the shorter axis should get a value of 100%.  The longer one is simply the factor by which it must be multiplied to have the same length as the shorter axis, given as a percentage.  The valid range of the factors is 10%-100%, although typical values should rarely go below 75%.

NOTE:  Axis Correction may be set automatically when the Synrad FLCC driver is selected.

#### Lens Type:

This parameter is only available when the Synrad FLCC driver is selected.  When using the Synrad FLCC driver in conjunction with a Synrad DH, FH, or Fenix head, the appropriate lens type should be selected from the list.  The list associates ProLase configuration parameters with the physical performance of the lens.

Selecting an FH/Fenix lens that supports internal distortion correction has the following effects on the other field page configruation paramters:

- ProLase Distortion Correction is deactivated.  The axis values remain the same, but they are not used.

- Synrad’s distortion correction asymetrically affects the field size.  ProLase ‘squares’ the resulting mark field by adjusting the Aspect Correction values.  One axis will be compressed to about 80%.  Which axis is compressed depends on the Swap XY setting.

Additionally, selecting any lens with a known focal length will set the ProLase Field Size parameter accordingly.  The ‘Uncorrected’ lens type indicated that no special action is take by ProLase.

NOTE:  The settings made to the configuration parameters by selecting a lens are only suggested values.  Any of these values may be change freely following the lens selections.  This allows for fine-tuning by the user if these default values aren’t perfect.  Switching lens types will only modify those parameters that are known for the selected lens.

Switching to a different driver will leave the current configuration values the same.  Choosing the Synrad FLCC driver will impose the values associated with the last lens selection.

#### Delays:

Tuning delays are used by the system to compensate for servo system lag times.  These delays are set at installation by the integrator and user adjustment is not recommended.  The software is capable of marking at speeds far beyond the ability of ANY galvo to respond; marking speeds are limited by mirror inertia and servo electronics, not the software.  To see the software speeds that can be achieved, set all delay timers to 0, and use an X/Y configured oscilloscope connected to the DAC outputs to view the marking.  (The oscilloscope simulates a perfect galvo/servo system with zero lag time.)

To tune performance start with all delays at zero, and Pos Res at 25.  Adjust Pos TC (position time constant) and Distance TC (position distance factor) until the marking ‘tails’ (marks leading to the beginning of characters) are eliminated.  Very high values of these parameters will eliminate the tails but will unnecessarily slow the marking.  If tails exist on long moves then increase Distance TC.  If tails are equal for long and short moves increase Pos TC.  Adjust Poly TC to increase the ‘sharpness’ of points where polyline segments connect.  Too high a value for Poly TC will create unsightly dwell marks.  Too low a value will make the connection points rounded.  Adjust Endseg TC to increase the wait at the end of a series of polylines.  This insures that the bottom right leg of the letter "A", for example,  is complete before starting the next letter.  Too high a value will create dwells at the end of letters, too low a value will create ‘short legs’.   Pos Res controls the slew rate limit for galvo movements when the laser is off.  It is measured in galvo steps (GS).  Fast settling galvos can use high Pos Res in the range of 20 GS or more.  5 GS is a good average limit.  Older, slower galvos may require lower limits in the range of 1 to 2 GS.  Old galvos that accept step inputs to the destinations can achieve this by setting Pos Res to 0.  Step inputs means the position command is instantly set to the new location without ramping.  A Pos Res setting of 1 GS is the slowest movement possible whereas a Pos Res of 0 is the fastest possible.  Too fast of a Pos Res setting will make the galvos “ring” for long periods at the destination point.

WARNING:  Some Galvos can not accept step inputs (Pos Res = 0) and may even be damaged.  Before using step inputs contact the galvo manufacturer to see if this would cause damage.

##### Polyline TC:

The Polyline TC parameter is used to control how long the software will wait at vector connection points.  The wait is required due to the lag time between the software/DAC position and the actual hardware/mirror position.  This timer applies to all vectors whose endpoint is also the start point of the next vector (polyline connection points).  In other words, this timer applies to end of all vectors in a series of connected vectors, except for the last one (the end of last one is controlled by the Endseg TC parameter).  The three connected points in a square or the intermediate connection points in a polyline circle are examples of points the Polyline TC parameter can effect.  The starting point of the square is controlled by StartSeg TC parameter.  The last corner of the square is controlled with the Endseg TC timer.

##### Pos TC & Distance TC:

The delays required for laser off position moves are calculated based on a formula that includes an offset and a distance proportional component.  Pos TC is the offset portion and is added to all position moves.  The Distance TC parameter is multiplied by the distance moved and normalized.  These two components are added together to determine the total delay required.

- Total delay = (length of move X Distance TC) + Pos TC

Pos TC is set to eliminate tails on small moves and Distance TC is used to eliminate the remaining long move tails.  If these timers are set too high the marks will look good but the marking throughput will be adversely affected. It is very important to find the smallest values that eliminate the tails.

##### StartSeg TC:

The StartSeg TC delay is used to minimize the acceleration effects at the beginning of vectors.  Increase this timer to reduce hot spots at the beginning of marks.  If too large a number is used the vector will be short or shapes will not close.  This timer effects only the hot spots that occur at the beginning of vectors or the beginning of a series of polyline vectors.  Endseg TC is used to control hot spots that occur at the end of vectors.  StartSeg TC will only work if the optional counter timer board is used to control the CO2 PWM directly or the Nd:YAG Q-switch directly.

##### Endseg TC:

The Endseg TC parameter is used to control how long the software will wait at the end of a series of vectors.  The wait is required because the software is always "ahead" of the hardware and must wait for the hardware to catch up.  This delay applies to the end of all vectors in which the laser is to be turned off after execution.  For example in the letter "A" , this timer only applies to the right hand leg and the middle bar.  The Polyline TC parameter applies to the top of the "A".

Another example of and Endseg vector is the end of the last line that completes a HPGL polyline circle.

Software StartSeg:

This toggle enables a software-based implementation of the laser signals at the start of a mark.  Normally, this is accomplished by programming the CTR05 card’s 9513 chip with the appropriate delays and activating the counters with the free running 1 MHz clock.  In the software mode, the same laser signals are set/cleared during the motion control.  This is simply an alternate method to produce the same signals and is recommended for users that are experiencing problems with wiring or CTR05 gating.

#### Driver:

This parameter configures the software for one of several output devices.  When selected, a list of possible devices is displayed; the currently selected device will be highlighted.  The hardware wiring for a particular driver is documented in Chapter 2 of this manual.

##### I/O Driver Type:

This selection configures the system for the DAC position command outputs, normally used for driving a galvanometric type system.  Some DACs used are twelve bit, giving 4096 individual positions across the field for each axis.  Other DACs are 16 bit (DDA06/16 for example) giving 65536 steps across the field.  The voltages used are jumper configured on the PC card, but normally +/- 10 VDC is used.  The 0,0 position is in the center of the field, positive X is right, and positive Y is up.  General Scanning 660, and Cambridge servo systems are examples of DAC control devices.

##### SCANLAB RTC2:

The RTC2 card from SCANLAB is an independent, digital motion/laser control system that resides on a PC bus.  When this card is used, ProLase acts as a “front-end” system only and is not directly involved in the actual real time control.  From the End-User’s point of view use of ProLase is very much the same as when using one of our direct control interfaces discussed above.  There are a few exceptions: wobble, ALI bitmap control, and arcs are the main features not supported by the RTC2 card.  ProLase converts all arcs to line segments before passing them on to the RTC2, although not as good as actual arcs this approximation works for the vast majority of applications.  The user can control the fineness of the line segments using the ARC RES parameter in the RTC2 configuration dialog.  At this time bitmaps are not supported.  The RTC2 card handles XY galvo control, laser power control and laser gating/pulsing control, therefore a Counter/Timer board is not required.  However the RTC2 does not support Digital I/O for machine control, so ProLase assumes the user has a DDA06/12 card for the Digital I/O, and focus control.  The DDA06/12 should be jumpered as described above.

The RTC2 board comes with a diskette that contains a series of files, 3 of these files are required by ProLase.  The locations of the 3 files is user selectable in the ProLase RTC2 configuration.  RTC24WE95.DLL is the DLL that allows ProLase to communicate with the RTC2.  RTC24W95.HEX is the program for the RTC2 and is downloaded from ProLase.  COR_1TO1.CTB is the RTC2 distortion correction file.  The names of these files may change as SCANLAB introduces new versions.  To update to SCANLABs latest version, simply select the new files using the ProLase RTC2 configuration dialog browse buttons.   The ProLase installation installs a default set of these files, however these should be replaced with the actual files that came with the RTC2. The Version Data button will provide

information about the version of SCANLAB files being used.  ProLaseRTC2 for Windows NT users must also install the RTC2 Driver using the SCANLAB provided RTC2SETUP.EXE program.  The address of the card is a parameter when installing this driver and must match the address entered in the BASE ADDRESS JUMPERS parameter of the ProLase RTC2 configuration dialog.  Of course the card must be physically jumpered to the same address.  The default for the card and the ProLase configuration is 240 hex.  In most cases this will work fine.  The card must be the right type for the kind of laser used.  The RTC2 comes in 3 basic types: CO2, Nd:YAG, and LEE.  You must tell ProLase the type of laser you are using, in the LASER TYPE parameter on the RTC2 configuration dialog.  The pin outs for the RTC2 card is fully defined in the SCANLAB documentation.

NOTE: NT systems like ProLaseRTC2 and ProLaseRTC2 DLL require the following:

Installation of GIVEIO driver using IOINST.EXE.   Example: IOINST.EXE GIVEIO GIVEIO.SYS

Installation of Sentinal driver.

Installation of SCANLAB RTC2 driver using RTCSetup.exe from SCANLAB.

RTC24NT4.DLL, RTC24NT4.HEX, and Cor_1to1.CTB or their equivalence are requried and are supplied by SCANLAB.  The location of these files are parameters in the ProLase configuration when the RTC2 driver is selected.  The default address for the RTC2 card is 240 hex.  This is a parameter in the ProLase configuration and a parameter of RTCSetup.exe, the SCANLAB driver installer.

#### PCI CIO-CTR:

This control is used to indicate that the CIO-CTR card being used is located on the PCI bus (NOT the ISA bus) and it is Plug & Play compatible.

Port Addresses:

This section provides feedback as to the address assignments expected by ProLase for the driver selected.

#### Calibration:

The Calibrate function is used to calibrate the DAC control outputs that are normally used for power/current, focus and frequency control.  A menu is displayed that shows a list of voltages (0-5 VDC).  At the far right of the menu is a list of power or frequency values.  When one of these values is selected, the DAC will be activated to the corresponding voltage.  The user measures the value of the power/current, focus or frequency (a power/current meter or frequency counter is required to measure the parameter) and inputs the results.  This procedure is repeated for each value in the table.  This table is used as calibration data so that when a desired power/current or frequency is programmed in the material database, the DAC output will produce the correct value.  Applications developed on calibrated systems can be moved to other calibrated systems without adjusting parameters.  Re-calibrating a system can be used to compensate for time dependent degradations (like old flash lamps, or old CO2 gas), thus avoiding periodic changes to the process database.

#### Alt. FPS Mode:

Activating this feature will make ProLase set use the Alternate First Pulse Suppression mode for use with Q-switch YAG lasers.  Normally, the Q-switch modulation signal will begin pulsing only after the FPS delay has expired.  When the Alt. FPS Mode is selected, the Q-switch pulsing begins at the end of the StartSeg delay and coincidentally with the beginning of the FPS signal.  See Figures 4 and 5 in Chapter 2 for more detailed timing diagrams.  This selection should be made in accordance with the laser manufacturer’s specifications and is available only for those drivers options using the CTR05 card.

#### Load Config:

This selection will load the current configuration data from the disk Config file.  This can be used to restore a configuration to its original state after Config values have been edited.

#### Save Config:

This selection saves the current configuration values to the configuration disk file.

- Main Menu:

File   Edit   View    Insert   Tools   Marking   Preferences   Window   Help

## Window Menu:

- New Window

- Cascade

- Tile

- Arrange Icons

#### New Window:

The New Window menu item will create a new window of the current document.  This is not the same as a new document, it is a new view of the same document.  Changes made in one window will be reflected in the other.  However, window position, sizing and zoom level, grid settings, etc. are independent.  For example, in one window the zoom level could display the entire marking field while another window was zoomed in on a particular item.

#### Cascade:

Selecting this menu item will organize all of the current windows so that they appear on top of each other, each consecutive window offset from the ones above it slightly.

#### Tile:

Selecting this menu item will arrange all the windows so that they do not overlap at all.  The sizes and shapes of each window will be roughly equal and will take up the entire program window.

#### Arrange Icons:

Selecting this menu item will align any minimized window icons that are currently in the program window.

- Execute Main Menu:

File      Config      QUEUE     Run       Display

Process I/O    Fixture                        Edit

## QUE:

- Add

- Insert

- Delete

### Add:

The Add function allows the user to add a program file to the Job Queue.  First a file is selected (File/Open), and variable information is provided.  When Que/Add or Que/Insert is selected the user will be prompted for the quantity of parts to marked.  The file, its variable information and the quantity are saved at the bottom of the queue.

### Insert:

The Insert command works like the Add command except the user can determine where in the queue to add the file.  The user will be prompted for the Insert position, the file is placed at this position and all files that were at or below this position will be moved down the queue.

### Delete:

Delete is used to remove an entry from the Queue.  The system will prompt for a Delete position, the entry at that position will be deleted and all entries below it will be moved up the queue.

- Execute Main Menu:

File      Config      Queue    Run      Display

Process I/O    Fixture                        Edit

## RUN:

- Program

- Que

- Simulate

- Simulate Que

In any of the Run modes a status area at the bottom left of the screen will display the following information:

Status - empty, Awaiting Start, or Marking Layer number.

.ALI file name.

Number of parts marked.

Time of last mark sequence.

Variable information name and data.

A marking sequence is initiated by either the F1 key or a valid digital I/O start signal.  After mark completion the system will return to the "Awaiting...Start" mode and marking can again be initiated.  A "Mark Complete" signal is provided to facilitate parts handler interfacing.  This procedure is repeated until all parts have been marked.  If programmed for multiple part fixtures, all parts on a "tray" are marked with one start initiation.  If the quantity entered is less than a full tray then a partial tray is marked.   The tray must filled according to the step and repeat programming.  Usually X steps are positive and Y steps are negative requiring trays to be filled left to right and top to bottom.  If no quantity is entered then all trays are assumed to be full, if the entered quantity is larger than a full tray, then full trays are assumed until the number of parts remaining to be marked becomes less than a full tray.  The job queuing feature can be used to pre-enter part counts (and/or other variable information) for a sequence of trays.  Using <Escape> at any time while in the Run mode will cause the system to abort the marking and exit the Run mode.  The system actually completes the current object and then aborts executing the rest of the part.  Using <Escape> when the system is "Awaiting...Start" insures that there are no partially marked parts.  When returned to the Run main menu, the operator can either re-run the same file with a new quantity, File/Open the same file to change variable information, or File/Open a completely different file.

### Program:

Run/Program will execute the current mark program (ALI) file.  Motion and laser process control are in full effect during this mode.  The variable information obtained by the system when the mark program was loaded is used during the marking process.  Var_Text objects defined as Code_Reader will prompt for information after marking each part. Var_Text objects defined as File will read information from the disk file after each part.  The selection will prompt for the quantity of parts to be marked.  If a number is entered, the system will mark that number of parts, and return to the main Execution menu when finished.  If <Enter> is used without entering a number then the system will mark parts until <Escape> is used to exit the mode.  In other words entering no quantity is used to mark an undetermined quantity of parts.  The Config/Menu Options/Auto Quant can be used to disable the quantity prompting.  If this option is selected a quantity of 0 (infinite) is assumed.  The <Escape> is used to abort the run after the desired quantity is marked.

### Que:

The Run/Que selection is used to execute the sequence of programs currently stored in the job Queue.  When this mode is selected the program at the top of the queue is loaded and prepared for execution.  The system will wait for a start signal as in the Run/Program mode.  When the quantity has been marked, the program is removed from the queue and the next program is loaded.  This sequence is repeated until the queue is empty.  If <Escape> is used to abort processing, the number of parts remaining for the current queue entry is placed in the queue as the new quantity.  This allows queue processing to be aborted and continued at any time.  Queue processing can be suspended, a non-queued file loaded and run, and then the queue continued. In this way routine work can be interrupted for special runs.  The current queue entries are stored to disk if the operator returns to the DOS system.  This allows the queue to be memorized between invocations of ProLase.  This feature is especially useful in a process where many different types of marking go on the same part.

#### Job Queue Example:

lets assume we have a product that requires an ID plate containing specific information such as color.  We have orders for 15 red, 10 blue and 7 black versions of the product and we need marked ID plates for each.  All ID plates are the same when blank.  A parts handler system is used to remove marked plates and present the blank plates to the mark field.  A file called Example.ALI is programmed with a Programmed_Text/Keyboard object called color.   The operator executes a File/Open, selecting Example.ALI and entering "red" at the "Color" prompt.  Que/Add is selected and 15 is entered as the quantity.  This procedure is repeated for blue (10) and black (7).  Now Run/Que can be selected, all 32 plates will be marked resulting with the correct number of red, blue and black plates.  At any time the processing can be suspended, more plate orders added to the queue, and processing continued.

### Simulate and Simulate_Que:

Simulation modes are used for testing of programs and interfaces without actually marking parts.  The graphics area of the CRT screen is used to simulate the laser marking field.  Serialization, and reading records from disk are active just as in real marking.  Simulation is an good way to test program files, experiment with system configurations, and train operators without using an actual laser system.

# Supplemental User Instructions ProLase PLUS

- This section of manual applies only to the ProLaseXP Plus for Windows  software package.

ProLaseXP Plus is ProLaseXP with additional capability to control up to four external stepper motor axes (X, Y, Z, A).  The X/Y axis control allows for marking of large parts in sections.  Each Layer within a document (*.LAZ file) can specify a table X/Y offset position.  The step and repeat (Fixture page) has been expanded to allow array processing using the X/Y positioning capability.  Alternately, ProLase can automatically re-center the marking field over each logo or character.  The focus axis (Z) can be set for each layer allowing for the processing of ‘stepped’ parts (parts with marks at more than one elevation).  A rotary axis (A) is used to do cylindrical marking.  ProLase indexes this axis automatically during the marking process, providing the capability to mark cylinder shaped objects.

ProLase controls the external axes through a user stepper motor controller program.  ProLase sends step and directions signals to the controller and may receive feedback from it.  ProLase can communicate directly only with Indexer LPT.  Many other stepper motor controller programs can be used through the use of simple batch files written by the user.  Of course the user must also provide tables, amplifiers and stepper motors.

## Indexer LPT

Indexer LPT is a multi-axis stepper motor controller program provided by Ability Systems Corporation (215-657-4338, www.abilitysystems.com).  Indexer LPT sends step, direction, and limit switch control signals conveniently via the printer port.  Up to two (2) axes may be controlled by each printer port.  ProLase systems that use printer port dongles (software locks) cannot use the same port for the ProLase dongle and to control an external axis.  One or more additional printer ports must be installed.  Any Ability Systems dongle is designed to be ‘pass through’ for stepper motor control.

Indexer LPT should be installed, tested, and used as instructed by Ability Systems documentation.  Computers running in the Windows XP operating system require additional set up steps as described in the note below.

Indexer LPT on Windows XP

Indexer LPT running on a computer with the Windows XP operating system behaves differently that previous versions of Indexer LPT for Windows 95/98/ME.  To provide backward compatibility with existing ProLase systems, ProLaseXP PLUS comes with several small ‘helper’ programs.  TableServer.EXE, Table.EXE and TableBATConverter.EXE are installed in and must be present in the ProLaseXP PLUS installation folder.  Upgrading to the Window XP environment may also require the modifications of the InitXYZA.BAT file, see Initialization Proceedure below.  Please see Appendix C for more information about using ProLaseXP PLUS with Indexer LPT on Windows XP.

Indexer LPT on Windows 95/98/ME operating systems works using a driver accessed through the motor file.  Axes can be controlled by a set of commands from a batch file such as

echo set_home:a > motor

Here, the DOS command echo is used to display the command to the screen for the benefit of the user.  The text set_home:a is then redirected (the DOS ‘>’ command) into the motor file, where it is interpreted by the Indexer LPT driver.

When using the Windows XP version of Indexer LPT, the commands should have the form:

table set_home:a > motor

which invokes the Table.EXE program to appropriately pass the set_home:a command to the Indexer LPT driver and place the response in the motor file.

## ‘Other’ Stepper Motor Controllers

Customer using non-Indexer LPT stepper motor controller programs must provide an interface to relay commands from ProLase to the controller program.  Due to their ease of use, this is most often done using batch files (*.BAT), although any executable file format is acceptable.  Regardless of file type, it must accept 8 command line arguments of the form:

move.bat a, A, b, B, c, C, d, D

where a,b,c,d are the axis IDs for the X,Y,Z,A ProLase axes and A,B,C,D are the positions to set the each axis, respectively.  The positions are given as absolute or relative values as specified by the ProLase configuration, External Axis page.  The batch file must convey this data to the stepper motor controller as necessary.  An example of one such move.bat batch file is included with the ProLasePLUS installation.

In addition to a ‘move’ batch file, the user must supply a ‘status’ and an ‘initialize’ file.  The status file should retrieve the current positions/status from the controller and write this data to the Status Read File text file.  The initialization file should configure the axes as needed for proper use as described below.  The names and locations of each of these files must be specified in the External Axis Control configuration page window.

## Initialization Procedure

Before ProLase can send meaningful position commands the motion profile parameters must be set and each enabled axis must be homed.  ProLase initializes the axes upon program start up and whenever the configuration is modified.  To perform this initialization, ProLase calls a simple batch file created by the user.  A batch file called INITXYZA.BAT is provided in the Table sub-directory.  The user should modify this simple batch file.  The correct axis IDs and motion parameters must be selected.  This batch file should contain the following commands as a minimum:

echo set_accel:a,15000  >  motor

echo set_lowspeed:a,2000 > motor

echo set_highspeed:a,20000 > motor

echo set_home:a > motor

NOTE:  All batch files used by ProLase must have their CLOSE ON EXIT property set.  To set this property in Windows use Explorer or My Computer to find the icon for the *.BAT file.  Right click on the icon, and select Properties from the menu.  Go to the Program tab.  Make sure “Close on exit” is checked.

The batch file shown above is for use with Indexer LPT as the stepper motor controller and running on a Windows 95/98/ME operating system and the numeric values shown are just examples.  The correct values for speed and acceleration depend on motor power, table mass, gearing, etc. and should be determined by experimentation.  The numbers should be selected so that the table positions in minimum time without missing steps.  The Indexer LPT axis (‘a’ in the above example) is determined by the address of the printer port being used (See the Indexer LPT manual for complete details).  Of course each axis (four maximum) to be used by ProLase must be setup in this manner.

The set_home command sets the internal position keeping logic of Indexer LPT to zero ( 0 ).  Failure to execute a set_home prior to running ProLase will generate a table positioning error.  If at any time an axis hits a limit switch, the initialization batch file (InitXYZA.BAT) should be re-executed.  Otherwise ProLase will generate a table positioning error. Use the Marking / External Axis Control menu selection to re-initialize the table system.  The initialization batch file should perform a procedure that moves the table to a consistent position prior to executing the set_home command.  One way to do this would be to command the table to move in one direction until it hits a limit switch (and then back to the zero position if not at that edge).  If the switch is hit at a low speed the positioning will be more consistent than if the switch is hit at a high speed.

Before using the external axes from ProLase, several configuration parameters within ProLase must be set.  These parameters are in the Config/External Axis menu.  Basically this involves giving ProLase enough information to build the command string required to position the tables.

ProLase PLUS Capable

ProLase PLUS provides general feedback as to the status of the initialization of the external axes with the Marking | ProLase PLUS Capable menu item.  A button on the Standard Toolbar mirrors this state.  If not selected, no axis motion will occur.  The item is only enabled when a PLUS version of ProLase is running.  For ProLase PLUS configured to use the ‘Other’ Indexer Type and those using Indexer LPT on Windows 95/98/ME systems, the ProLase PLUS Capable item is selected only when at least one axis is enabled.

When using the Windows XP version of Indexer LPT, this item is selected only when one or more axes are enabled AND TableServer is running and completely initialized.  ProLase PLUS continuously polls for the presence of TableServer.  If TableServer is terminated for some reason, ProLase PLUS Capable will be deselected.  Manually selecting it will reinitialize the ProLase axes, restarting TableServer in the process.

X Axis ID is the unique identifier used to describe which axis is the ‘X’ axis in ProLase.  Iwhen using Indexer LPT, this is a letter between ‘a’ and ‘f’ depending on the printer port address as described in the Indexer LPT manual.

- Y, Z, A Axis ID identify their respective axis in the same manner as X Axis ID.

XRes, YRes, ZRes are the parameters that describe the number of steps per unit (inch, feet, m or cm as selected in the Configuration Field page) of travel for each respective axis.  This number is based on the number of stepper motor counts per motor revolution times the gearing ratio of the system.  For example on a table system with a 5 pitch lead screw (5 revolutions per inch) and stepper motors set up for 200 steps per revolution will have a Res factor of 1000 (1000 total steps per one inch movement of the table).

These values are best determined experimentally.  Using the External Axis control window, move an axis a number of steps and measure the resulting travel.  Use this as the Res number for that axis.  Test this in ProLase and adjust until the required resolution is obtained.

A negative Res may be used to move the tables in the opposite direction.  However, the value must be an integer.

ARes is similar except that it describes the number of steps required to rotate a cylinder 360 degrees (one revolution).  Remember this may also involve gearing.

This completes the set-up, making the table control ready to accept positioning commands from ProLase.

## X/Y/Z/A Axis by Layer:

External axis motion is programmed on a layer by layer basis.  First, right-click on the desired layer and select the Properties menu option.  Next select the External Axis tab to modify the parameters which govern external axis motion.

The Post Move Delay is provided to allow the tables to completely settle following motion commands.  To maximize throughput, this value should always be set to the minimum value that provides acceptable mark quality.

The Table Mode selection determines how and when the Layer will move the tables.  Normal mode allows the user to specify the four co-ordinates (X,Y,Z,A) for the layer, it’s ‘starting point’.  At the start of any layer, the external axes move to re-center the marking field over the layer’s position.  Several layers within a program can be used to create marking programs that mark over an area much larger than the scanning lens field.  The only limitation is that each object within a layer must be marked within the lens field.

ProLase will not tile a graphic (break a large graphic up into a number of smaller graphic).  To perform this operation properly, very precise knowledge about the hardware of the marking system is required.  Since American Laserware, Inc. does not provide this hardware, no assurance can be made regarding the agreement of external axis stepper motor motion and the position response and optical distortion of the galvanometer marking head.  Even tiny errors can result in unacceptable alignment between the ends of a single vector that spans multiple tiles.  Hence, ProLase does not support this feature.

During part design, the blue Marking Field box (or circle) will be redrawn to represent the new location for the current layer in the ProLase world.  All objects for that layer must be found completely within this boundary or the laser will not be physically able to mark them.

AutoXY mode will have ProLase automatically move the tables as needed to mark all the objects on a layer.  As ProLase marks the layer, the extents of each object are examined.  If any part of the object is outside of the current lens field, ProLase repositions the tables to center the current object under to lens field.

AutoXY mode is useful and greatly simplifies the part design process.  However, it may not be the most efficient.  Since table motion can represent a significant portion of the cycle time, minimizing it by manual programming with Normal mode may be optimal.  Consider the example below.

The diagram shows the same part (of 2 graphics) marked in both modes.  The 2 left hand images show the mark process in AutoXY mode.  In this mode, the table oscillates between the center of the 2 objects.  The table moves twice for each part.  The right hand image shows a Normal mode configuration with a judiciously chosen layer position.  Both objects are completely within the lens field and no more part motion is required.

## Cylindrical marking (A axis):

Cylindrical marking can be done by setting the Table Mode to A as X or A as Y.  ProLase marks around a cylinder by positioning a rotational axis so that the tangent point is the center of a character or logo to be marked.  ProLase can be configured to mark cylinders that rotate along either the X or Y axis.  Graphically ProLase represents the cylinder as if it has been sliced along is rotational axis and laid out flat.  The 0 degree position is at the zero axis position and the ruler represents arc length.  The total arc length around the cylinder (circumference) is 2R, where R is the cylinder radius value specified.  If a string of text is marked that is longer than the circumference then that sentence will end up writing over itself.  When A as X is selected, text should be rotated to zero or 180 degrees.  When A as Y is selected, text should be rotated to 90 or 270 degrees.  For each object in the layer, ProLase calculates the appropriate angle based on position and mark accordingly.  For text marking this means the motor will be rotated for each character.  If a Z-axis is used it must be set to insure that the tangent point of the cylinder is at the focal point of the lens.

## External Axis Step & Repeat:

ProLase can also move the X/Y table in a Step & Repeat (S&R) manner allowing the user to mark large, nested arrays of parts.  The controls for this are found in the Fixture window on the External Axis Step & Repeat tab.  This function works the same as the normal S&R except that the table is moved to a new location for each part (or nest of parts if Galvo S&R is used at the same time).  If both Galvo and External Axis S&R are done in the same process the Galvo S&R is executed first.  For example a 5x5 Galvo S&R (25 parts) can be placed within an external axis S&R of 2x5 allowing for marking of 250 (25 x 2 x 5) parts per run.

# Supplemental Information ToolKit

## Files and Directories:

This information is for the ProLase ToolKit product only.  All information for ProLaseXP for Windows or ProLaseXP Plus is contained in other sections of this manual.

All the files needed for the dll are included in the ProlaseDLL directory <DevDir>.  This directroy includes the .dll, .lib and .h files that are needed to build and execute Visual Studio C++ projects using ProLase.dll.

In addition to the files needed for compilation, there are a set of ProLase support files that the DLL needs access to at run time.    These should be copied to the applications execution directory <InstallDir>.  See appendix A for a complete list of support files ProLase (or any program using ProLase.dll) requires.  Look at the end-user ProLase sub-directory structure after installation for a complete picture of the support files and sub-directories required for fully functional ProLase.  To set everything up for development, install the included ProLase setup.  This will install the end-user version of ProLase.  UnZip the ProLase DLL project.  Replace the end-user version of ProLase.exe with ProLase.exe from the DLL project.  Now the end-user ProLase can be used to create “template” documents (LAZ files) that can be modified by the DLL.

Note the the DLLs themselves (ProLase.dll and Laser.dll, found in the ProlaseDLL subdirectory) must be present either in the application's execution directroy <InstallDir> or the Windows System directory <WinSys>.  In reality there are two DLLs that are used with the toolkit.  First is ProLase.dll which is an MFC extension DLL, requiring the MFC runtime library DLLs.  Secondly is Laser.dll which contains the actual motion and laser control logic.  ProLase.DLL is designed to be implicitly loaded and is built using the ProLase.lib export library.  Laser.DLL is explicitly loaded by ProLase.DLL when certain ProLase.DLL member functions are executed.  Laser.DLL does not require an import library.  The software security system does not take affect until Laser.DLL is loaded.  That means it is possible to have an application that includes ProLase.DLL that will load without the security locking device.  However none of the member functions in ProLase.DLL can be called without the device.  Any attempt to call those functions without the device will result in a “Security device not found” error message, and the application will be shut down.

The TESTDLL directory contains the files required to build and execute the sample application that uses the DLL.  There is a 	4.0 Developer Studio WorkSpace Project file.  We currently use

version 4.0 (Visual C++), but the project can loaded and converted by 5.0 or 6.0.

The TestDLL directory also contains ProLase for Windows 95, along 	with it's support files, fonts, import filters, etc.  This can be used to create LAZ file templates that can be manipulated with

the DLL.

## Member variable naming convention:

American Laserware Inc. uses the following naming conventions for all member properties:

m_        		member property

m_l			long (32 bit integer)

m_i*		 	int  (16 bit integer)

m_b		 	BOOL (TRUE or FALSE)

m_f		 	float (normal floating point)

m_s		 	CString (C++ String)

m_n		 	UINT (unsigned integer)

m_u		 	unsigned char (unsigned 8 bit character)

*Note:  int is really a 32 bit number in Windows.  We use it to represent 16 bit numbers that may overflow.  The range of values for data we specify as “int” is +/- 32767.

## Units:

Several types of units are used in the software.  Distance Units can be Inches, feet, Centimeters or meters.  The selection is made by the operator on the Config/Field property page.  Internally ProLase does not care what units are being used, except to print the correct units on various dialog boxes and property sheets.  Distance units are stored internally as 16 bit numbers, scaled at .0001 distance unit increments.  So if the system is configured to be in inches, then distance unit properties like X position would be 10000 counts per inch.  Angular units are .1 degrees internally.  To set an angle to 30 degrees, you would set the property to 300.  Time units are labeled as used, either microseconds or millisecond depending on the property.

The coordinate system is a standard Cartesian coordinate system where 0,0 is the center of the field.  Positive X is to the right and Positive Y is up (from the CRT frame of  reference).  A standard laser marker is oriented with the mark plane parallel to the ground.  If oriented so that the X axis is east-west then west is negative X, east is positive X, north is positive Y and south is negative Y.  Up and down is the focus axis (Z axis).  Up is positive Z and down is negative Z.

## Exported ProLase.dll Classes:

These are the basic classes exported by the DLL.  These exports are true classes, which means the developer can derive classes from them.  All supported member functions work across the EXE/DLL boundary except Cstring.  Cstring objects seem to only work if the Cstring being changed is originally longer than the new value.  This seems to be a bug in the Windows Operating system.  There are two ways around this, the first is to make sure the Template LAZ file contains very long strings and always modify them with shorter strings.  The second method is to use the American Laserware Inc. supplied SetItemText function to change the object text.

### CWSData Class:

This class loads and manipulates a .laz (ProLase) document.  In addition to the data corresponding to the layers and 	items, it also contains the material and fixture information that is attached to the .laz file.

There should be 1 instance of this class in each document of the application.  (Assuming each document corresponds to a LAZ file as it does in ProLaseXP for Windows).

Supported Member functions:

#### BOOL Load(LPCTSTR sPathNamePtr);

Loads a .laz (ProlaseXP) file.  SPathNamePtr is a string that contains the complete path specifying the location of the LAZ document file to load.  Load returns TRUE if the load was successful, otherwise a return of FALSE indicates an error condition

Example:

CWSData WSData;  			// declare an instance of WSData;

WSData.Load(“Demo.laz”);		// Load WSData object with Demo.laz  document file

#### CItem* operator ()(int nLayer, int nItem);

Returns a CItem pointer to point at the Item specified by nLayer and nItem.  Each WSData object contains a zero based, two-dimensional array of objects (Items) and layers.  The first layer shown in the Document is Layer 0, the first Item in Layer 0 is Item 0.  In this way any Item in a document can be uniquely identified by two numbers, i.e. nLayer, nItem.  The returned Item pointer can be used to access any of the Items methods or properties.  A NULL pointer is returned if the specified object does not exist.

“Wafer Marks” is Layer 0, “Column Label” is Item 0 of Layer 0.  “Background” is Layer 1 and “Circle” is Item 0 of Layer 1.

Example:

CItem *pItem;				// declare Item pointer

pItem = WSData(0,0);			// set pointer for the first layer, first item in the document

pItem->m_lXPos = 0;			// Sets Items X position to zero

WSData(0,0)->m_lXPos = 0;		// Does the same exact thing

#### CLayer* GetAt(int nLayer);

Returns a CLayer pointer for access to the specified Layer.  nLayer is the zero based layer array ordinal.  A NULL pointer is returned if the Layer does not exist.

Example:

CLayer *pLayer = WSData.GetAt(0);        	// Get pointer to first layer in Document

#### SetItemText(int nLayer, int nItem , LPCTSTR sTextString);

Sets the Text in a Fixed or Variable Text object.

Example:

WSData.SetItemText(0,0, “This is some text to be marked”);   // Set Layer 0, Item 0 Text property

#### SetItemFont(int nLayer, int nItem, LPCTSTR sFontName);

Sets the font file name for Fixed or Variable Text objects.  This function works for *.ALF and *.FNT file types.

#### Example:

WSData.SetItemFont(0,0, “C:\ProLase\Complex.fnt”); 	// Set Layer 0, Item 0 font file name.

#### WSData Properties:

##### m_Fixture

The m_Fixture object can be used to access the fixture database file linked to the WSData Document.

Example:

WSData.m_Fixture.m_ldOffsetAngle = 300; 		// Set Fixture angular offset.  Angle Units are .1 degrees.

WSData.m_Fixture.m_lOffsetX = 20000;		// Set Fixture X Offset to +2 “.  Distance Units .0001 “.

WSData.m_Fixture.m_lOffsetY = -20000;		// Set Fixture Y Offset to –2”.

WSData.m_Fixture.m_lFixFocusAdjust = 10000;	// Adjust Focus Height by +1”;

WSData.m_Fixture.m_nArrayX = 3;			// Set up 3 x 4 array of parts, 3 parts wide

WSData.m_Fixture.m_nArrayY = 4;			// 4 parts high.

WSData.m_Fixture.m_lDeltaX = 5000;			// .5 “ spacing between columns

WSData.m_Fixture.m_lDeltaY = -7500;			// .75 “ spacing between rows

##### m_Material

The m_Material object can be used to access the material database file linked to the WSData Document.

Example

WSData.m_Material.m_adFreq[0] = 5.0;           	// Sets the Pass 1 frequency to 5 kHz.

WSData.m_Material.m_adFreq[1] = 6.0;		// Sets the Pass 2 frequency to 6 kHz.

.

.

.

WSData.m_Material.m_adFreq[6] = 25.0;	// Sets the Pass 7 frequency to 25 kHz.

WSData.m_Material.m_adSpeed[0] = 10.0;	// Sets the Pass 1 speed to 10 “/sec

.

.

.

WSData.m_Material.m_adSpeed[6] = 32.0;	// Sets the Pass 7 speed to 32”/sec

WSData.m_Material.m_adPower[0] = 5.0;	// Sets the Pass 1 Power to 5.0

.

.

.

WSData.m_Material.m_adPower[6] = 22.0;	// Sets the Pass 7 Power to 22.0

##### m_sApsFile

The m_sApsFile is the CString that contains the name and path of the Material.  The name is stored on disk as a relative Path, but is always converted on a <InstallDir> based absolute path during the Serialize in process.  M_sApsFile will always be an Absolute path.

Example:

CString MaterialFileName = WSData.m_sApsFile;	// Get The Material File Name

##### m_sFixFile

The m_sFixFile is the CString that contains the name and path of the Material.  The name is stored on disk as a relative Path, but is always converted on a <InstallDir> based absolute path during the Serialize in process.  M_sApsFile will always be an Absolute path.

Example:

CString FixtureFileName = WSData.m_sFixFile;

### CLayer Class:

This class provides access to the Layer properties.  A Layer is a member of the WSData class.  A pointer used to access the Layer properties can be obtained from the WSData class using the WSData::GetAt(int nLayer) function.

#### CItem* GetAt(int nItem);

Returns a CItem pointer for access to the specified Item.  nItem is the zero based Item array ordinal.  A NULL pointer is returned if the Item does not exist.

Example:

CLayer *pLayer = WSData.GetAt(0);        	// Get pointer to first layer in Document

CItem *pItem = pLayer->GetAt(0);		// Get pointer to first Item in first layer

#### int GetNumberItems(void);

Returns the total number of Items in the Layer.

Example:

Int TotalNumberOfObjects = *pLayer->GetNumberItems();

#### CLayer Properties:

##### m_bActive

Holds the Active/Background state of the Layer.  TRUE means the layer is Active and will mark.  FALSE means the Layer is background only, and will not be marked.

Example:

CLayer *pLayer0 = WSData.GetAt(0);			// Get pointer to first Layer

CLayer *pLayer1 = WSData.GetAt(1);

pLayer0->m_bActive = TRUE;				// Set first Layer to Active

pLayer1->m_bActive = FALSE;				// Set Second Layer to Background

##### m_sLayerName

Holds the CString Layer name.  This name corresponds to the name displayed in the ProLase Layer/Item Tree control.

Example

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to the first Layer

CString LayerName = pLayer->m_sLayerName;	// Get the Layer Name

##### m_uInitOut, m_uInitOutMask

Holds the tri-state programming of the Initial Output I/O control for the Layer.  The Initial Output is set on the digital I/O port at the start of the Layer mark processing.  This property is displayed as a Layer property on the Layer Control Sequence property sheet.  The eight bits of the Mask property correspond to the eight output bits available on the I/O card.  A bit value of 1 means that bit is to be set, a value of zero means that bit should be left in its current state.  The 8 bits of the m_uInitOut property are the actual values to be set.  For example if bit 7 of the m_uInitOutMask property is a one, and bit 7 of the m_uInitOut property is a zero, then the hardware bit 7 output will be set to zero.  If on the other hand bit 7 of the Mask is set to zero, then the hardware bit 7 will be left unchanged.  In the ProLase control, grayed check marks means the Mask bit is zero (leave Unchanged), black checks indicate the Mask bit is set to one, and the corresponding m_uInitOut bit is set to one (force the output bit to 1).  A black box without any check means the Mask bit is set to one, and the corresponding m_uInitOut bit is set to zero (force the output bit to 0).  Bits 0 and 2 are used internally by ProLase for the GATE and NOT GATE signals, and should not be changed (their Mask bits should always be set to zero).

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to the first Layer

pLayer->m_uInitOutMask = 0x80;			// Set bit 7 mask bit set

pLayer->m_uInitOut = 0xff;				// Set bit 7 to ON, the other bits are left // // //unchanged even though they are set in the //m_bInitOut instruction.  This is because the //Mask value for these other bits is zero, and //therefor they are left unchanged.

##### m_sPreMarkMessage

Holds the CString that contains the Pre Mark Message displayed by ProLase during mark processing.

Example:

CLayer *pLayer = WSData.GetAt(0);					// Get pointer to the first Layer

pLayer->m_sPreMarkMessage = “Press Mark Button to Begin”;	// Inform Operator how to begin

##### m_uPreIn, m_uPreInMask

Holds the tri-state programming of the Start Mark Input.  This describes the Input port condition required to start marking the Layer.  A Mask bit value of zero means that the input bit will be ignored.  In other words, if the Mask bit is zero, any state of the corresponding input bit will meet the start requirements.  In ProLase grayed checks mean “don’t care”, black checks mean the corresponding bit must be a one, and a blank means the corresponding bit must be a zero.

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to the first Layer

pLayer->m_uPreInMask = 0x07;				// Set bits 1,2,3 mask

pLayer->m_uInitOut = 0x03;				// Set bits 1,2 to ON, and bit 3 to off.  The other //bits are ignored.  This Layer will not mark until //the Input I/O port has bits 1 and 2 set to a logic //high and bit 3 to a logic low.  The state of the //other bits does not matter since the Mask is set //to zero for those bits.

##### m_uPreOut, m_uPreOutMask

Holds the tri-state programming of the Pre Mark Output I/O control for the Layer.  The Pre Mark Output is set on the digital I/O port after the start conditions are met and just before actual marking begins.  The eight bits of the Mask property correspond to the eight output bits available on the I/O card.  A bit value of 1 means that bit is to be set, a value of zero means that bit should be left in its current state.  The 8 bits of the m_uPreOut property are the actual values to be set.  For example if bit 7 of the m_uPreOutMask property is a one, and bit 7 of the m_uPreOut property is a zero, then the hardware bit 7 output will be set to zero.  If on the other hand bit 7 of the Mask is set to zero, then the hardware bit 7 will be left unchanged.  In the ProLase control, grayed check marks means the Mask bit is zero (leave Unchanged), black checks indicate the Mask bit is set to one, and the corresponding m_uPreOut bit is set to one (force the output bit to 1).  A black box without any check means the Mask bit is set to one, and the corresponding m_uPreOut bit is set to zero (force the output bit to 0).  Bits 0 and 2 are used internally by ProLase for the GATE and NOT GATE signals, and should not be changed (their Mask bits should always be set to zero).

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to the first Layer

pLayer->m_uPreOutMask = 0x80;			// Set bit 7 mask bit set

pLayer->m_uPreOut = 0xff;				// Set bit 7 to ON, the other bits are left // // //unchanged even though they are set in the //m_uPreOut instruction.  This is because the //Mask value for these other bits is zero, and //therefor they are left unchanged.

##### m_uPostOut, m_uPostOutMask

Holds the tri-state programming of the Post Mark Output I/O control for the Layer.  The Post Mark Output is set on the digital I/O port after the completion of the marking for that layer.  The eight bits of the Mask property correspond to the eight output bits available on the I/O card.  A bit value of 1 means that bit is to be set, a value of zero means that bit should be left in its current state.  The 8 bits of the m_uPostOut property are the actual values to be set.  For example if bit 7 of the m_uPostOutMask property is a one, and bit 7 of the m_uPostOut property is a zero, then the hardware bit 7 output will be set to zero.  If on the other hand bit 7 of the Mask is set to zero, then the hardware bit 7 will be left unchanged.  In the ProLase control, grayed check marks means the Mask bit is zero (leave Unchanged), black checks indicate the Mask bit is set to one, and the corresponding m_uPostOut bit is set to one (force the output bit to 1).  A black box without any check means the Mask bit is set to one, and the corresponding m_uPostOut bit is set to zero (force the output bit to 0).  Bits 0 and 2 are used internally by ProLase for the GATE and NOT GATE signals, and should not be changed (their Mask bits should always be set to zero).

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to the first Layer

pLayer->m_uPostOutMask = 0x80;			// Set bit 7 mask bit set

pLayer->m_uPostOut = 0xff;				// Set bit 7 to ON, the other bits are left // // //unchanged even though they are set in the //m_uPostOut instruction.  This is because the //Mask value for these other bits is zero, and //therefor they are left unchanged.

##### m_iPostMarkDelay

Holds the value for the post mark delay.  This can be used to wait a fixed time period before going to

Process the next Layer.  This delay is in milliseconds, i.e. 1000 = 1 second.

Example

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to the first Layer

pLayer->m_iPostMarkDelay = 2000;			// Set Post Mark Delay to 2 seconds

### CItem Class:

This class provides access to the Item properties.  An Item is a member of the CLayer class.  A pointer used to access the Item properties can be obtained from the CLayer class using the CLayer::GetAt(int nItem) function.  Items have no useful external methods, but have lots of properties.

#### CItem Properties:

##### m_sItemName

Contains the CString name of  the Item.

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to first Layer

CItem * pItem = pLayer->GetAt(0);			// Get pointer to first Item of first Layer

CString ItemName = pItem->m_sItemName;		// Get the Item Name.

##### m_sFileName

Contains the CString graphic file name for  the Item.  The graphic filet name is an absolute path and name.

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to first Layer

CItem * pItem = pLayer->GetAt(0);			// Get pointer to first Item of first Layer

CString GraphicFileName = pItem->m_sFileName;	// Get the Graphic file Name.

##### m_lXPos

Contains the X position for the Item.  0 is the center of the field on the X axis.  The units are .0001 of the user units.  If the user units is inches then 10000 = 1 inch.

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to first Layer

CItem * pItem = pLayer->GetAt(0);			// Get pointer to first Item of first Layer

pItem->m_lXPos = 25000;				// Sets the X Position to 2.5 user distance units

##### m_lYPos

Contains the Y position for the Item.  0 is the center of the field on the Y axis.  The units are .0001 of the user units.  If the user units is inches then 10000 = 1 inch.

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to first Layer

CItem * pItem = pLayer->GetAt(0);			// Get pointer to first Item of first Layer

pItem->m_lYPos = -20000;				// Sets the Y Position to -2.0 user distance units

##### m_iAngle

Contains the rotation angle for the Item.  0 is the angle where a perpendicular to the sentence axis is pointed north.  Positive angles rotate clockwise (like a compass).  The units are .1 degrees.

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to first Layer

CItem * pItem = pLayer->GetAt(0);			// Get pointer to first Item of first Layer

pItem->m_iAngle = 900;				// Sets the Angle to +90 degrees

##### m_iItalic

Contains the Italic or tilt angle for the Item.  0 is the angle where a perpendicular to the sentence axis is pointed north.  This results in no tilt.  Positive angles rotate clockwise (like a compass).  The units are .1 degrees.

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to first Layer

CItem * pItem = pLayer->GetAt(0);			// Get pointer to first Item of first Layer

pItem->m_iItalic = 50;					// Tilts the Item by 5 degrees

##### m_lRadius

Contains the Ring Radius for marking on the circumference of a ring.  0 indicates the object is to be drawn flat (default).  The units are .0001 user distance units, same as the position units.  A positive radius will mark clockwise on the ring (bottoms of letters towards the center of the ring).  A negative radius will mark counter clockwise on the ring.

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to first Layer

CItem * pItem = pLayer->GetAt(0);			// Get pointer to first Item of first Layer

pItem->m_lRadius = 20000;				// Mark on ring of radius 2.0 user distance units.

##### m_nXJust, m_nYJust

The justification properties control how an object is drawn about the m_lXPos and m_lYPos coordinates.  If both X just and Y just are set to center then the object will drawn such that the center of the object will be at m_lXPos, m_lYPos.

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to first Layer

CItem * pItem = pLayer->GetAt(0);			// Get pointer to first Item of first Layer

pItem->m_nXJust = 	JX_LEFT;

JX_CENTER;

JX_RIGHT;			    // Sets X Justification (1 if 3 values)

pItem->m_nYJust = 	JY_BOTTOM;

JY_CENTER;

JY_TOP;			    // Sets Y Justification (1 of 3 values)

##### m_lHeight

Contains the Height for the Item.  For text objects this is the Height of a standard capital letter, like the letter “A”.  For Graphics it is the total height of the extents of the graphic.  The units are .0001 of the user distance units.  If the user units is inches then 10000 = 1 inch.

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to first Layer

CItem * pItem = pLayer->GetAt(0);			// Get pointer to first Item of first Layer

pItem->m_lHeight = 20000;				// Sets the Height 2.0 user distance units

##### m_lLength

Contains the Length for the Item.  For Natural aspect mode objects with proportional character spacing, m_lLength is zero, indicating the length should correspond to the natural length of the object based on the Height.  For Free aspect mode with proportional character spacing, Length is the Length of a sentence for text object, for graphic objects Length is the total length of the graphic.  For fixed space Text objects, Length refers to the character length (or character width).  The units are .0001 of the user distance units.  If the user units is inches then 10000 = 1 inch.

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to first Layer

CItem * pItem = pLayer->GetAt(0);			// Get pointer to first Item of first Layer

pItem->m_lHeight = 20000;				// Sets the Height 2.0 user distance units

##### m_lCharSpace

Contains the character spacing property.  If 0, then the character will be marked with the natural spacing built into the font.  Fixed spacing (m_lCharSpace > 0) is used for dials and rulers.  Characters will be drawn such that their centers will be equally spaced by the m_lCharSpace value.

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to first Layer

CItem * pItem = pLayer->GetAt(0);			// Get pointer to first Item of first Layer

pItem->m_lCharSpace = 5000;				// Sets the Char Space to .5 user distance units

##### m_lLineSpace

Contains the sentence line spacing property.  If 0, then the lines of text will be marked with the natural spacing built into the font.  Fixed spacing (m_lLineSpace > 0) is used for dials and rulers.  Sentences will be drawn such that their centers will be equally spaced by the m_lLineSpace value.

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to first Layer

CItem * pItem = pLayer->GetAt(0);			// Get pointer to first Item of first Layer

pItem->m_lLineSpace = 2000;				// Sets the Char Space to .2 user distance units

##### m_bMirrorX, m_bMirrorY

Contains the mirror flags for the object.  MirrorX = TRUE means the object will  be drawn mirrored about the X axis.  MirrorY = TRUE will cause the object to be mirrored about the Y axis.  Setting one of the Mirrors to TRUE will cause the object to be drawn “inside out”.

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to first Layer

CItem * pItem = pLayer->GetAt(0);			// Get pointer to first Item of first Layer

pItem->m_bMirrorX = TRUE;				// Mirror about the X axis

pItem->m_bMirrorY = TRUE;				// Mirror about the Y axis

##### m_bUseMaterialFile

The laser process parameters (writing speed, laser power, pulsing frequency) can be calculated based on a material file or specified by object.  The m_bUseMaterialFile property determines which set of numbers will be used.  If m_bUseMaterialFile = FALSE, then the Object values will be used, else values will be calculated based on the Material file entries and the object percent modifiers.

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to first Layer

CItem * pItem = pLayer->GetAt(0);			// Get pointer to first Item of first Layer

pItem->m_bUseMaterialFile = TRUE;			// Specify use of Material File

##### m_fFreq2

The Object laser pulsing frequency in kHz.  This value will be used to mark the object if m_bUseMaterialFile is set to FALSE.

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to first Layer

CItem * pItem = pLayer->GetAt(0);			// Get pointer to first Item of first Layer

PItem->m_bUseMaterialFile = FALSE;

pItem->m_fFreq2 = 5.0;					// Specify use 5 kHz laser pulsing

##### m_fPower2

The Object laser power in user selected units (usually either AMPS or WATTS).  This value will be used to mark the object if m_bUseMaterialFile is set to FALSE.

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to first Layer

CItem * pItem = pLayer->GetAt(0);			// Get pointer to first Item of first Layer

PItem->m_bUseMaterialFile = FALSE;

pItem->m_fPower2 = 50.0;				// Specify use 50 watts

##### m_fSpeed2

The Object writing speed in user distance units per second.  This value will be used to mark the object if m_bUseMaterialFile is set to FALSE.

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to first Layer

CItem * pItem = pLayer->GetAt(0);			// Get pointer to first Item of first Layer

PItem->m_bUseMaterialFile = FALSE;

pItem->m_fSpeed2 = 50.0;				// Specify use 50 user distance units per second

##### m_iFreqFac

The Percentage of the Material File specified Frequency to use during marking.  The resultant value will be used to mark the object if m_bUseMaterialFile is set to TRUE.  Units are in .1 % increments.

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to first Layer

CItem * pItem = pLayer->GetAt(0);			// Get pointer to first Item of first Layer

PItem->m_bUseMaterialFile = TRUE;

pItem->m_iFreqFac = 500;				// Specify 50% of Material File frequency value

##### m_iPowerFac

The Percentage of the Material File specified Power to use during marking.  The resultant value will be used to mark the object if m_bUseMaterialFile is set to TRUE.  Units are in .1% increments.

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to first Layer

CItem * pItem = pLayer->GetAt(0);			// Get pointer to first Item of first Layer

PItem->m_bUseMaterialFile = TRUE;

pItem->m_fPower2 = 750;				// Specify use 75% of Material File Power value

##### m_fSpeedFac

The Percentage of the Material File specified Speed to use during marking.  The resultant value will be used to mark the object if m_bUseMaterialFile is set to TRUE.  Units are in .1% increments.

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to first Layer

CItem * pItem = pLayer->GetAt(0);			// Get pointer to first Item of first Layer

PItem->m_bUseMaterialFile = FALSE;

pItem->m_fSpeed2 = 1500;				// Specify 150.0% if Material File Speed Value

##### m_uCondMark, m_uCondMarkMask

Holds the tri-state programming of the Conditional  Mark Input.  This describes the Input port condition required to mark the object.  If the condition is not met the object is skipped..  A Mask bit value of zero means that the input bit will be ignored.  In other words, if the Mask bit is zero, any state of the corresponding input bit will meet the mark condition.  In ProLase grayed checks mean “don’t care”, black checks mean the corresponding bit must be a one, and a blank means the corresponding bit must be a zero.

Example:

CLayer *pLayer = WSData.GetAt(0);			// Get pointer to the first Layer

CItem * pItem = pLayer->GetAt(0);		       // Get pointer to first Item of first Layer

pItem->m_uCondMarkMask = 0x07;			// Set bits 1,2,3 mask

pItem->m_uCondMark = 0x03;				// Set bits 1,2 to ON, and bit 3 to off.  The other //bits are ignored.  This Object will be skipped //unless the Input I/O port has bits 1 and 2 set to //a logic high and bit 3 to a logic low.  The state //of the other bits does not matter since the //Mask is set to zero for those bits.

### CMark Class:

This class contains the vector information and the laser marking engine.  It also contains the configuration data.  There should be only 1 instance of this class in the application.  This should be placed either in the Mainframe or CApp class.  The Cmark object should be created by initializing a pointer as follows:

CMark m_pMarkEngine;

m_pMarkEngine = CMark::NewMark();

The pointer should be used to access all member functions of CMark.

The CMark object will automatically initialize the interface cards according to the driver selected in the Configuration.  The simplest thing to do is set up and test the configuration using the ProLaseXP program.  You can directly read and write to the DIO ports if you wish.  You MUST NOT change bit 0 and bit 2 of the output port, these are used internally as the GATE and NOT GATE signals.  The chip used for the DIO allows you to read the outputs, so read them and use ANDs and ORs to only change the bits you want to change and leave bits 0 and 2 unchanged.  For the DDA06 card the input port address is 0x33c,and the output port is 0x33d.  These are 8 bit ports so use the _inp and _outp commands.  If you decide to control the I/O yourself make sure your "template" has all Layer I/O settings of "don't care" (XXXXXXXX).  Also make the ABORT OUTPUT property of the parts program are all "don't care".

#### CConfig* GetConfig();

Returns a pointer to the ProLase Configuration object.

Example:

CMark *m_pMarkEngine = CMark::NewMark();				// Create MarkEngine Object

M_pMarkEngine->GetConfig()->Load((LPCTSTR) “Default.cfg”);	// Load the Config File

#### SimpleMarkRun(CWSData* pData, int iNumberParts);

Marks all layers and all objects in the passed WSData object.  All I/O will be in effect and behave as programmed.

Example:

CMark *m_pMarkEngine = Cmark::NewMark();				// Create MarkEngine Object

m_pMarkEngine->GetConfig()->Load((LPCTSTR) “Default.cfg”);	// Load the Config File

CWSData WSData;				  			// declare an instance of WSData;

WSData.Load(“Demo.laz”);						//Load WSData object with //Demo.laz  document file

m_pMarkEngine->SimpleMarkRun(&WSData,5);					// Mark a run of 5 parts.

#### void SetDLLLayerMode(int iMode);

#### int GetDLLLayerMode(void);

The layer mode setting affects the way the toolkit processes layers during a call to SimpleMarkRun().The layer mode is set to MARK_DLLLAYERMODE_NORMAL upon creation and remains constant unless changed by using SetDLLLayerMode() or re-initialization.  GetDLLLayerMode() is provided for access to the current setting.  The argument for SetDLLLayerMode() must be one of the following constants defined in the Mark.h file:

MARK_DLLLAYERMODE_NORMAL

Layer processing halts only for layer with specified start conditions (NOT all ‘don’t cares’).

No visual indication is provided.

Layer start signals are accepted via the general purpose digital I/O or the <ENTER> key.

MARK_DLLLAYERMODE_MODAL_ALL

Layer processing halts for ALL layers regardless of specified start conditions.

Message box displayed with Pre-Mark Message.

Message box OK/CANCEL is ONLY layer control input.  NO digital I/O start signal.	MARK_DLLLAYERMODE_MODAL_SOME

Layer processing halts only for layer with specified start conditions (NOT all ‘don’t cares’).

Message box displayed with Pre-Mark Message.

Message box OK/CANCEL is ONLY layer control input.  NO digital I/O start signal.

Example:

CMark *m_pMarkEngine = Cmark::NewMark();				// Create MarkEngine Object

m_pMarkEngine->GetConfig()->Load((LPCTSTR) “Default.cfg”);	// Load the Config File

CWSData WSData;				  			// declare an instance of WSData;

WSData.Load(“Demo.laz”);						//Load WSData object with //Demo.laz  document file

m_pMarkEngine->SetDLLLayerMode(MARK_DLLLAYERMODE_MODAL_SOME);

m_pMarkEngine->SimpleMarkRun(&WSData,5);					// Mark a run of 5 parts.

CDisplay

This class creates and displays the screen images.

There should be 1 instance of this class for each view of

the application.

Drill Objects:

The ProLase DLL contains additional features for drilling applications.  A set of functions is available for creating and marking various drill objects.  An example of the use of these drilling features can be found in the DrillDlg.h and .cpp files in the TestDLL project.   See below for more details.  For all of the functions, the position values are given in 1 / 10,000 units and the angle values are in 1 / 10 degrees.  For instance a position of (0.5 in, 1.25 in) would be given as (5000 , 125000) and an angle of 45 degrees is given as 450. nRepRate is in pulses per second (pps).

The following functions are used to assign drill object properties to existing CItem instances.

void CItem::CreateDrillSpot(long lX, long lY, UINT nRepRate, UINT nPasses);

Positions the laser spot at (lX, lY) and fires nPasses laser pulses at the frequency corresponding to nRepRate pps.

void CItem::CreateDrillLine(long lX1, long lY1, long lX2, long lY2, long lVel, UINT nRepRate, UINT nPasses);

Draw nPasses lines from (lX1, lY1) to (lX2, lY2).  When on, the laser spot moves at lVel velocity and is operating at the frequency corresponding to nRepRate pps.

void CItem::CreateDrillCircle(long lCenX, long lCenY, long lX, long lY, long lVel, UINT nRepRate, UINT nPasses);

Draws nPasses circles centered at (lCenX, lCenY).  (lX, lY) is a point on the circle and marks the start/end point of the mark.  When on, the laser spot moves at lVel velocity and is operating at the frequency corresponding to nRepRate pps.

void CItem::CreateDrillTrepan(long lX, long lY, long lAngle, long lDiameter, long lVel, UINT nRepRate, UINT nPasses, long lDeltaAngle = 0);

Draws nPasses trepans centered at (lX, lY) with a lDiameter OUTER diameter size.  When on, the laser spot moves at lVel velocity and is operating at the frequency corresponding to nRepRate pps.  lAngle specifies the angle offset of the tangent point of the inner and outer circles from the center  horizontal.  lDeltaAngle is an incremental change in the angle that is applied for each additional pass.

void CItem::CreateDrillSpiral(long lX, long lY, long lAngle, long lInnerDia, long lOuterDia, long lPitch, long lVel, UINT nRepRate, UINT nPasses, long lDeltaAngle = 0);

Draws nPasses spirals centered at (lX, lY) with a lInnerDia inner diameter, lOuterDia outer diameter and lPitch spacing between marks along the same radial.  When on, the laser spot moves at lVel velocity and is operating at the frequency corresponding to nRepRate pps.  lAngle specifies the angle offset of the tangent point of the inner and outer circles from the center  horizontal.  lDeltaAngle is an incremental change in the angle that is applied for each additional pass.  The spiral generated will always begin and end at the specified angle.  This may extend the spiral past the specified outer diameter at the specified pitch until the current revolution is complete and has a diameter greater than the outer diameter specified.

void CItem::CreateDrillSpiral2(long lX, long lY, long lAngle, long lInnerDia, long lOuterDia, long lPitch, long lVel, UINT nRepRate, UINT nPasses, long lDeltaAngle = 0);

Spiral2 is the same as Spiral except that where a normal spiral would terminate, a sprial2 will continue to mark at the outer diameter until it intersects itself.

void CItem::CreateDrillMove(long lX1, long lY1, long lX2, long lY2, long lVel, UINT nPasses);

Move is the same as a line except that the laser is NEVER turned on while marking it.  It can be used for controlled movement of the galvos at precise speeds rather than the high-speed slewing normally used during moves between marks.

The following CWSData member functions will create objects in the same manner as the CItem functions listed above.  Rather than modifying and existing object however, these functions will return a reference to newly created CItem.  The calling program is responsible for freeing the memory associated with the referenced CItem.

CItem *CWSData::CreateDrillSpot(long lX, long lY, UINT nRepRate, UINT nPasses);

CItem *CWSData::CreateDrillLine(long lX1, long lY1, long lX2, long lY2, long lVel, UINT nRepRate, UINT nPasses);

CItem *CWSData::CreateDrillCircle(long lCenX, long lCenY, long lX, long lY, long lVel, UINT nRepRate, UINT nPasses);

CItem *CWSData::CreateDrillTrepan(long lX, long lY, long lAngle, long lDiameter, long lVel, UINT nRepRate, UINT nPasses, long lDeltaAngle = 0);

CItem *CWSData::CreateDrillSpiral(long lX, long lY, long lAngle, long lInnerDia, long lOuterDia, long lPitch, long lVel, UINT nRepRate, UINT nPasses, long lDeltaAngle = 0);

CItem *CWSData::CreateDrillSpiral2(long lX, long lY, long lAngle, long lInnerDia, long lOuterDia, long lPitch, long lVel, UINT nRepRate, UINT nPasses, long lDeltaAngle = 0);

CItem *CWSData::CreateDrillMove(long lX1, long lY1, long lX2, long lY2, long lVel, UINT nPasses);

The CItems created or modified by the previous functions are expected to be marked using the CMark::MarkSingleItem member function.  The CMark class provides additional functions that encapsulate this functionality.  These functions create and mark the drill object in one simple step using the same arguments as the previous CItem and CWSData functions.  All these functions simply return the result of the internal MarkSingleItem call.

UINT CMark::MarkDrillSpot(long lX, long lY, UINT nRepRate, UINT nPasses);

UINT CMark::MarkDrillLine(long lX1, long lY1, long lX2, long lY2, long lVel, UINT nRepRate, UINT nPasses);

UINT CMark::MarkDrillCircle(long lCenX, long lCenY, long lX, long lY, long lVel, UINT nRepRate, UINT nPasses);

UINT CMark::MarkDrillTrepan(long lX, long lY, long lAngle, long lDiameter, long lVel, UINT nRepRate, UINT nPasses, long lDeltaAngle = 0);

UINT CMark::MarkDrillSpiral(long lX, long lY, long lAngle, long lInnerDia, long lOuterDia, long lPitch, long lVel, UINT nRepRate, UINT nPasses, long lDeltaAngle = 0);

UINT CMark::MarkDrillSpiral2(long lX, long lY, long lAngle, long lInnerDia, long lOuterDia, long lPitch, long lVel, UINT nRepRate, UINT nPasses, long lDeltaAngle = 0);

UINT CMark::MarkDrillMove(long lX1, long lY1, long lX2, long lY2, long lVel, UINT nPasses);

## TestDLL Sample Application:

In the TestDLL demonstration application, all the code needed to implement the Prolase.DLL is racketed by:

/*** PROLASE DLL ***/

/*******************/

to make it easy to find.  This application demonstrates the easiest way to use the DLL, which is to create a Template LAZ file using the standard ProLase front end program.  The Template should contain the desired number and types of objects.  As many properties as possible should be set in the Template so the DLL will only have to set those properties that will change during the processing.  The ProLase front end should also be used to create and test Fixture, Material, and the Configuration files.  Again this is to avoid setting a lot of properties in the DLL that will never change.  It is much faster, easier and simpler to test these properties using the ProLase front end.  The frame work for the sample application was created using the Visual Studio Apps Wizard.  Only a small number of additions were required to implement laser marking using ProLase.DLL.  The Sample Application can load any LAZ Document file and mark it.  The “Test” menu selection creates a WSData Object, a MarkEngine Object, loads CONFIG.SYS, loads DEMO.LAZ, changes several object properties and marks the resultant WSData Object.

6 files have been altered:

MainFrm.cpp			MainFrm.h

TestDLLDoc.cpp		TestDLLDoc.h

TestDLLView.cpp		TestDLLView.h

MainFrm.h:

Included ProlaseDLL\Prolase.h

Declared CMark as a member of the mainframe

MainFrm.cpp

In the constructor (CMainFrm::CMainFrm), first

load the config file, then create the mark engine.

TestDLLDoc.h

Included ProlaseDLL\Prolase.h

Declared CWSData as a member of the Doc

Declared a flag to tell us if the WSData class

is valid (has been loaded)

TestDLLDoc.cpp

Included mainfrm.h --- Document needs to access the mainfrm,

or where the CMark class is a member of

In the constructor, set the valid flag to false

In OnFileOpen, allow the user to select a file,

then load the file.

In OnFileMark, if the data is valid, get a

pointer to the mainframe and call the mark engine's

marking routine

TestDLLView.h

Included ProlaseDLL\Prolase.h

Declared an instance of the display class as a member.

TestDLLView.cpp

In OnInitialUpdate, create the display

In OnDraw:

Zoom to the field size

Draw the field box

Draw all the items

Update the screen

In OnSize:

First two times, don't do anything

After that, call the display's onsize function

with the same parameters

The simplest way to use this is to create a ‘template’ of items and layers, using the regular

ProLaseXP for Windows editor, and modifying the properties using the examples above.  You can find 	the complete list of member properties in the ITEM.H file.

## AXLase Active X Sample Application:

The AXLase sample is an Active X control project using the ProLaseXP DLL to perform its functions.  American LaserWare has included a basic set of interfaces that, when possible, parallels the COM automation interface documented in Chapter 7 of this manual.  Users are invited to add functionality to this Control, all source code is included.  A VB sample project called “TestActiveX” is included which uses the LaseAX Active X control.  A very similar VB project called “TestVB” uses the COM AUTOMATION capability described in Chapter 7.  One of the differences between the Active X control and the COM AUTOMATION is the function “sCreateMarkEngine(ProLasePath)”.  This function creates a LAZ object in the Active X control, and must be executed before any loading of LAZ files.  The parameter “ProLasePath” is a path to a ProLaseXP installation folder.  This is where the Active X control can find all the support files (fonts, .CFG, box, circle, etc.) needed to run.  The default installation creates “C:\Program Files\American LaserWare\ProLaseXP\” as the installation folder. So an example of the sCreateMarkEngine function would be “sCreateMarkEngine(“C:\Program Files\American LaserWare\ProLaseXP\”.

# Supplemental Information  ProLaseXP Server

## Command Line interface:

ProLaseXP command line interface is a method for calling ProlaseXP from another program or a DOS command line, passing parameters such as LAZ file name, mode, and quantity to mark.

PROLASEXP.EXE lazfilename, mode, quantity

Modes:

AUTO 			Goes to mark dialog, mark button pushed.  Remains in GUI when done.

AUTOEXIT		Same as AUTO except exits when done.

SIM			Same as AUTO except SIMULATE button pushed for testing.

SIMEXIT		Same as SIM except exits when done, also used for testing.

For example the command line: PROLASEXP.EXE SAMPLE.LAZ AUTOEXIT 12, will command ProLase to come up with the file SAMPLE.LAZ in the mark mode, waiting for start signals.  As soon as ProLase has marked 12 parts it will quit back to the command line.

## Automation Server interface:

ProLase Server is a COM based automation interface to the core marking technology within the ProLase marking software.  When this interface is used ProLase runs in the background providing services to a customer created user interface.  Since ProLase Server is based on COM software technology its services can be accessed from any language that supports COM.  This includes Visual C++, Visual Basic, and many others.  The interface is defined through a Type Library (.TLB) file, called “Prolase.TLB).  In Visual C++, an interface class can be automatically created through Class Wizard.  Simply go to the AUTOMATION tab and press the ADD CLASS button.  Select FROM A TYPE LIBRARY.  Use the file browser to select “ProLase.TLB”.  In Visual Basic use the PROJECT/REFERENCES menu selection and use the BROWSE button to select “ProLase.TLB”.  The OBJECT BROWSER tool can be used to view the interface.

There are several groups of functionality including Document functions, Layer functions, Item functions, and marking control functions.  American LaserWare uses a function naming convention as follows: Document functions begin with “d”, Layer functions begin with “l”, Item functions begin with “I”, and marking control functions begin with “m”.  Some controller development enviroments (like Visual Basic for example) uses the term Properties for some functions, even though these are functions from the servers point of view (GET, SET functions).  These Properties still follow the naming conventions.  Any GET functions without a SET function listed means the property is meant to be read only, for example the marking status functions.  The interface includes the SET functions but they will not do anything if called.

## Document Functions

### BOOL	 dLoadLazDocument(LPCTSTR lazfilename)

lazfilename is the complete name and path to a ProLase created .LAZ document file.  This function loads the document into the ProLase Server.  Returns TRUE if the document successfully loaded.

### BSTR	 GetdMaterialFileName()

This function returns the name of the Material File linked to the current document. The name includes the complete path.

### BOOL	 SetdMaterialFileName(LPCTSTR materialfilename)

materialfilename is the complete name and path.  This function will change the current document’s material file link.  Returns TRUE if a file loaded successfully.  If the specified file is not available from the disk, then a default material file is loaded.

### BOOL	 dIsDocumentLoaded()

Returns TRUE is the previous call to dLoadLazDocument(…) was successful.  Returns FALSE otherwise.

### BOOL	 dIsValidLayer(short LayerIndex)

Returns TRUE if LayerIndex specifies an existing layer in the currently loaded document; FALSE otherwise.

### BOOL   dIsValidItem(short LayerIndex, short ItemIndex)

Returns TRUE if Layer Index and ItemIndex specify an existing item on an existing layer in the currently loaded document; FALSE otherwise.

## Layer Functions

Most Layer functions access the Layer through use of a zero based index(short layerindex …).  Most Layer functions are actually Layer Properties in the ProLase UI, and consists of GET, SET functions.

## Item Functions

For the purposes of the server, ProLase marked objects are called “Items”.  Hopefully this will be less confusing since the word “object” refers to so many things when programming.  A ProLase Item consists of a paragraph of fixed text, a line of variable text, a graphic, or a item link.

Most Item functions access the Item through use of two indexes (short layerindex, short itemindex…):  The zero based index to the Layer that contains the Item, and the zero based index (within the Layer) to the Item itself.  In the example the Layer named “Wafer Marks” is layerindex  = 0, “Backgound” is layerindex = 1, etc.  The Item named “Column Label” is layerindex = 0, itemindex = 0.  “Link to Serial Number” is layerindex = 0, itemindex = 3.  “Circle” is layerindex = 1, itemindex = 0.  As with Layers, most Item functions are actually “Object Properties” in the ProLase UI and consists of GET, SET functions.

Dimensional information (XY position, height, length, etc.) is in whatever units ProLase is currently configured for.  For example if ProLase is in inch mode, SetiXPosition(0,0,1.003) will set the items X Position to 1.003 inches.  If ProLase were in centimeter mode then the item would be set to 1.003 cm.  0,0 is the center of the mark area, with + values being to the right and up.  – values are left and down from center.

### void 	SetiFixedText(short layerindex, short itemindex, short textline, LPCTSTR text)

This function sets a line of fixed text.  textline is the zero based line of text to be set (Fixed text objects can have up to 10 lines of text), and text is the string of text to set.

### BSTR	GetiFixedText(short layerindex, short itemindex, short textline)

This function returns a line of fixed text. textline is the zero based line of text to be returned.

### void 	SetiSerialNum(short layerindex, short itemindex, LPCTSTR text)

This function sets the current serial number for a serial number object.  This is used to set the starting serial number where ProLase will increment the number after every mark.

### BSTR	GetiSerialNum(short layerindex, short itemindex)

This function returns the current serial number for a serial number object.

### BOOL	SetiGraphicName(short layerindex, short itemindex)

This function sets the name of the desired graphic file.  The path is relative to where ProLase Server is located.  Of course if a complete absolute path is provided there is no question where the file is.

### BSTR	GetiGraphicName(short layerindex, short itemindex)

This function returns the name and path for the graphic file.  If this name was set by ProLase (original name in the LAZ document) then it will always be an absolute path.

void 		SettiXPosition(short layerindex, short itemindex, float Position)	         											  This function sets the X Position.  Units are the same as set in the ProLase configuration file.  For example SetiXPosition(0,0,1.003) sets the first items X position to 1.003 inches if ProLase is configured for inches, or 1.003 cm if Prolase is configured for centimeters.

float 		GetiXPosition(short layerindex, short itemindex)           	   														  Returns the current value of the items X Position.

void 		SettiYPosition(short layerindex, short itemindex, float Position)												  This function sets the Y Position.

float 		GetiYPosition(short layerindex, short itemindex)           	   														  Returns the current value of the items Y Position.

void 		SettiHeight(short layerindex, short itemindex, float Height)	             											  This function sets the Height.  For Graphic items it is the total height of the graphic.  For text items it is the character height.  If the item is in Natural Aspect mode then the Length will change with height to keep the item at natural aspect.  Height controls the item size, SetiLength(…) commands will have no effect in Natural Aspect mode.

In Free Aspect mode, height and length are controlled separately.

float 		GetiHeight(short layerindex, short itemindex)           	   														  Returns the current value of the items Height.

void 		SettiLength(short layerindex, short itemindex, float length)	             											  This function sets the Length.  Setting the Length to 0 will force the object to Natural Aspect, no matter which Aspect Mode is set.  Height controls the item size, SetiLength(…) commands will have no effect in Natural Aspect mode.  In Free Aspect mode, height and length are controlled separately.

float 		GetiLength(short layerindex, short itemindex)           	   														  Returns the current value of the items total length.

void 		SetiAspectMode(short layerindex, short itemindex, short mode)

Sets the value of the items Aspect Mode:

0 = Free							1 = Natural						2 = Fixed 						3 = Squeeze

If Squeeze is selected (3), then the SetiFixedFactor(…) function should be used to set the fixed aspect factor.

short 		GetiAspectMode(short layerindex, short itemindex)

Returns the current Aspect Mode setting.

void 		SetiFixedFactor(short layerindex, short itemindex, float percentage)  	   								     		         Sets the value of the items Fixed Factor.  This value sets the aspect of an item as a percentage of Natural aspect.  For example 50% would compress the item length to one half of its natural length at the current height.  200% would expand the length to twice the natural length at the current height.

float 		GetiFixedFactorAspectMode(short layerindex, short itemindex)                    	   								     Returns the current Fixed Factor value.

BOOL iUseRealSize(short LayerIndex, short ItemIndex)

This function applies only to vector graphic items that are linked to source files created by CAD packages.  This function scales the height and length of the item to the real-world dimensions specified in the source graphic file.  The values will be scaled to match the units chosen in the configuration.  This function will fail if the graphic size exceeds the world size.  This function returns TRUE if successful, otherwise FALSE.

BOOL iUseRealPosSize(short LayerIndex, short ItemIndex)

This function applies only to vector graphic items that are linked to source files created by CAD packages.  This function locates the item to the real-world position and size specified in the source graphic file.  The values will be scaled to match the units chosen in the configuration.  The item will be bottom-left justified and set to natural aspect mode. This function will fail if the graphic position or size exceeds the world size.  This function returns TRUE if successful, otherwise FALSE.

void 		SettiAngle(short layerindex, short itemindex, float Position)  	         				  This function sets the Item rotation angle.  Units are in degrees.  The convention is like a compass, 0 degrees is north and angles increase clockwise.  The range is 0-360.

float 		GetiAngle(short layerindex, short itemindex)           	   							  Returns the current value of the items rotation angle.

void 		iReloadGraphic(short layerindex, short itemindex)	                              											  This function reloads the items graphic from disk to graphic memory.  Use this function if using a generic graphic name in the document, but the user wants to copy different graphic files to that name.  For example if the document has an item linked to TEMP.DXF as the graphic name and the user wants to copy different DXF files to TEMP.DXF before marking.  iReloadGraphic(…) needs to be called after the copy and before the mark so that the new TEMP.DXF is copied into memory instead of the old one.  The old version of TEMP.DXF is purged from ProLase’s graphic memory, releasing the memory space.  This is one way to implement a variable graphic.

Another way would be to simply change the name of the items graphic using the SetiGraphicName(…) function.  This technique will keep all of the previous graphics in memory, so if the name is changed to something that has been loaded before, it will exist in graphic memory without loading from disk.

BOOL 		GetiXMirror(short Layerindex, short Itemindex)

This function returns TRUE is the object is mirrored in the X axis (along its length), FALSE otherwise.

void 		SettiXMirror(short Layerindex, short Itemindex, BOOL bNewValue)

This function will mirror the in the X axis (along its length) if bNewValue is TRUE or turn off mirroring if bNewValue is FALSE.

Note: Mirroring occurs about the object’s location and will change the extents of the object if it is not center justified.

BOOL 		GetiYMirror(short Layerindex, short Itemindex)								  This function returns TRUE is the object is mirrored in the Y axis (along its height), FALSE otherwise.

void 		SettiYMirror(short Layerindex, short Itemindex, BOOL bNewValue)

This function will mirror the in the Y axis (along its height) if bNewValue is TRUE or turn off mirroring if bNewValue is FALSE.

Note: Mirroring occurs about the object’s location and will change the extents of the object if it is not center justified.

short 		GetiXJustify(short Layerindex, short Itemindex)

This function returns the X justification (along its length) for the specified object.

0	=	Left

1	=	Center

2	=	Right

void 		SettiXJustify(short Layerindex, short Itemindex, short nNewValue)

This function will set the X justification (along its length) of the specified object to the value given by nNewValue.  See GetiXJustify() above for valid arguments.

short 		GetiYJustify(short Layerindex, short Itemindex)

This function returns the Y justification (along its height) for the specified object.

0	=	Bottom

1	=	Center

2	=	Top

void 		SettiJustify(short Layerindex, short Itemindex, short nNewValue)

This function will set the Y justification (along its height) of the specified object to the value given by nNewValue.  See GetiYJustify() above for valid arguments.

## Marking Functions

BOOL		mRunBatch(double batchquantity)			batchquantity is the number of parts to be marked in the run.  This function will return TRUE if the batch marked to completion.  If the mark was aborted, this function will return FALSE.  The GetMarkStatus function can be used to find out how the mark was aborted.  This function is equivalent to pressing the MARK button (with AUTO selected) in the ProLase UI, MarkControl Dialog.

BOOL		mForceStart()						Overrides the ProLase start conditions and forces a valid start.  The controller program should use the GetMarkStatus function to make sure that ProLase is waiting for a start before issuing this command.  This command is latched, meaning a new ForceStart command is required for each part or tray of parts.

BOOL		mAbortBatch()						Aborts the current run.  Returns TRUE if the mark is successfully aborted.

long		GetmMarkStatus()					Returns the status of the current batch run.  The status is either normal operation (codes 0-10) or an abort code.  Abort codes are generated whenever the batch was stopped for any reason before completion of the batchquantity.

0 = Not Running

1 = Initializing Mark

2 = Waiting for Start Condition

3 = Marking Layer

4 = Post Mark Delay

10 = Batch Complete

States 2 through 4 are repeated for each layer of each part in the run.  State 10 is reset to zero when it is read once.

Abort Codes

50 = Abort Source Unknown

90 = Automation Controller Abort

100 = User Abort (Escape key)

101 = Digital I/O Abort

102 = List File Error

103 = “Waiting for Start” Timeout

void		SetmMarkStatus(long markstatus)			Sets the MarkStatus.  This can be used to clear the status after reading an abort code for example.

short		GetmLayerStatus()					Returns the layerindex of the current Layer being marked.  This can be used to monitor the progress of each part.

short		GetmItemStatus()					Returns the itemindex of the current Item being marked.  This can be used to monitor the progress of each part.

short		GetmPartCount()						Returns the number of completed parts in the batch.  This can be used to monitor the progress of the batch.

BSTR		GetmUnits()						Returns the current unit size used by the configuration.  Values can be:

in.	= inches

ft.	= feet

m	= meters

cm = centimeters

short		GetmDigIO()						Returns the state of the 8 digital input ports.  The return value should be masked with 0x00ff.

void		SetmDigitalIO(short newValue)			Sets the state of the 8 digital output ports.  NewValue will be masked with 0x0ff and only the lower 8 bits will be set.  Note: bits 0 and 2 are reserved for Gate/Not Gate laser control signals and will not be modified by this command.

float		GetmOffsetX()						Returns the current value of the Field X offset in the current configuration units.  The X offset changes the X position of the entire field including all marked items in the document currently active in the marker (not necessarily the controllers document).

This property along with mOffsetY and mOffsetAngle can be used to externally control the field offsets based on information from a device that measures part offsets.  A vision system for example can determine the current X, Y, ANGLE of a part and give that information to ProLase through the controller interface so that ProLase can correctly transform the marks to hit the part in the right place and orientation.  These properties can be used to control the transformation of any document being currently marked including the ProLase UI, or another controller’s document.

void		SetmOffsetX(float position)              			Sets the Field X Offset in the configuration units.

float		GetmOffsetY()						Returns the current value of the Field Y offset in the current configuration units.  The Y offset changes the Y position of the entire field including all marked items in the document currently active in the marker (not necessarily the controllers document).

.

void		SetmOffsetY(float position)              		    Sets the Field Y Offset in configuration units.

float		GetmOffsetAngle()					Returns the current value of the Field Angle offset in degrees.

void		SetmOffsetAngle(float position)		    Sets the Field Angle Offset.

# Appendix A

ProLase support Files

Required?

File			Description		Directory			ProLase    +	DLL	Dev

ProLase.exe		ProLaseFront End		<InstallDir>		yes	   yes	no	no

ProLase.dll		ProLase DLL		<InstallDir> or <WinSys>	no	   no	yes	yes

Laser.dll			Laser Control DLL		<InstallDir> or <WinSys>	yes	   yes	yes	yes

Ssiact.386                    	AEGIS Driver                	<WinSys>		yes	   no	no	no

Imgman32.dll		Raster Graphic DLL		<InstallDir> or <WinSys>	yes	   yes	yes	yes

Im31bmp.dil		Raster Importer		<InstallDir> or <WinSys>	yes	   yes	yes	yes

Ezbar32.dll		Bar Code DLL		<InstallDir> or <WinSys>	yes	   yes	yes	yes

ImsType.dll		Vector Graphic DLL		<InstallDir> or <WinSys>	no*	   no*	no*	no*

Isgdi32.dll		Vector Graphic Importer	<InstallDir> or <WinSys>	no*	   no*	no*	no*

Mfc40.dll			MicroSoft MFC DLL		<WinSys>		yes	   yes	yes	yes

Mfc42.dll			MicroSoft MFC DLL		<WinSys>		yes	   yes	yes	yes

Msvcrt.dll			MicroSoft DLL		<WinSys>		yes	   yes	yes	yes

.flt			Vector Filters		<InstallDir>\Filters		no*	   no*	no*	no*

Pmpbt9k.exe		Vector Processor		<InstallDir>		no*	   no*	no*	no*

Default.cfg		ProLase Configuration	<InstallDir>		yes	   yes	yes	yes

Simplex.alf		ProLase Default Font	<InstallDir>		yes	   yes	yes	yes

Bcali.alf			ProLase BarCode Font	<InstallDir>		no*	   no*	no*	no*

Idmali.alf			ProLase ID Matrix Font	<InstallDir>		no*	   no*	no*	no*

Bmp.alg			ProLase Raster Graphic	<InstallDir>		no*	   no*	no*	no*

Box.alg			ProLase Box graphic	<InstallDir>		yes	   yes	yes	yes

Circle.alg			ProLase Circle Graphic	<InstallDir>		yes	   yes	yes	yes

Laz.New			“New” Laz file		<InstallDir>		no*	   no*	no*	no*

Default.Pre		Preferences File		<InstallDir>		no**	   no**	no**	no**

Default.Mat		Default Material File		<InstallDir>		no**	   no**	no**	no**

Default.Fx2		Default Fixture File		<InstallDir>		no**	   no**	no**	no**

Default_Power.Cal		Power Calibration File	<InstallDir>		no**	   no**	no**	no**

Default_Freq.Cal		Frequency Calibration 	<InstallDir>		no**	   no**	no**	no**

Default_Focus.Cal		Focus Calibration File	<InstallDir>		no**	   no**	no**	no**

ProLase.lib		ProLase DLL Interface	<DevDir>			no	   no	no	yes

*.h			DLL Class Interfaces	<DevDir>			no	   no	no	yes

Indexer LPT		Stepper Motor Indexer	Autoexec.bat, Config.sys	no	   yes	no	no

Initxyza.bat		Stepper Initialize		<InstallDir>\Table		no	   yes	no	no

Movexyza.bat		Axis Move (non LPT)	<InstallDir>\Table		no	   no*	no	no

GetStatus.bat		Axis Status (non LPT)	<InstallDir>\Table		no	   no*	no	no

Status.txt			Axis Feedback (non LPT)	<InstallDir>\Table		no	   no*	no	no

NOTE: NT systems like ProLaseRTC2 and ProLaseRTC2 DLL require the following in addition to those defined above:

Installation of GIVEIO driver using IOINST.EXE.   Example: IOINST.EXE GIVEIO GIVEIO.SYS

Installation of Sentinal driver.

Installation of SCANLAB RTC2 driver using RTCSetup.exe from SCANLAB.

RTC24NT4.DLL, RTC24NT4.HEX, and Cor_1to1.CTB or their equivalence are requried and are supplied by SCANLAB.  The location of these files are parameters in the ProLase configuration when the RTC2 driver is selected.  The default address for the RTC2 card is 240 hex.  This is a parameter in the ProLase configuration and a parameter of RTCSetup.exe, the SCANLAB driver installer.

<WinSys> = Windows System directory, usually C:\Windows\System on most PCs. .  As indicated in the above table, some items can be installed in <WinSys> as an alternative to the <InstallDir> sub-directory.

<InstallDir> = Installation Sub-directory.   This is the sub-directory that the EXE file is installed in.  The EXE file is either ProLase.exe or any program using ProLase.DLL.  For example lets assume LaserMark.exe is a customer created program using the ProLase DLL.  If LaserMark.exe is installed in C:\Program Files\CustProg, then <InstallDir> = C:\Program Files\CustProg.  To run this program all items marked <InstallDir> should be installed to C:\Program Files\CustProg.

<DevDir> = Development sub-directory.  This can be any directory since you specifically include the files into Visual Studio projects.  All files marked <InstallDir> need to be copied to the Projects main directory, in order to run from Visual Studio.

No* = The file is not required to execute ProLase, but is required for some major feature to operate.  For example ProLase will run without the vector graphic DLLs, however the graphic import capabilities will not operate correctly.  Do not try to use capabilities that require missing components.

No** = The file is not required to execute ProLase.  If  the file is not present, ProLase will create default versions of the file give it a name of DEFAULT and save it to disk.

A minimum installation requires all files marked YES.  This will allow the system to process SIMPLEX text objects only.

A complete and fully functional installation of ProLase requires all files marked YES, NO*, or NO**.

# Appendix B

This appendix lists the details of the publicly defined file formats for file types created and used by American Laserware, Inc. and ProLase software.

## *.fnt

The *.fnt format is a vector font file format used by ProLase.  These fonts become available to text objects in ProLase when the ‘Laser Engraving Fonts’ font type is selected.  Typically these are single or multiple stroke fonts similar to those used by mechanical engraving systems.  The *.fnt file specifications are as follows:

- ASCII text file (almost any text editor will do).

- File must have the *.fnt extension.

- Numeric data entered as text.

- ONE number per line.

- Whitespace preceeding the numeric data is ignored.

- Carrige retrun/Linefeed (CRLF) terminates each line.

- Data range is –32768 to +32767 (signed short integer).

- Characters must correspond to non-extended ASCII values, 0-127 (However, the ProLase editor will only allow you to enter viewable characters, ASCII 32-127).

- Characters may be omitted.   Missing characters are not output by ProLase.

- Character definitions may occur in any order and characters may be redefined.  ProLase will use the data corresponding to the last instance of the character in the file.

- Characters are listed consecutively in the file; no blank lines to separate them.

- Character definition:

  - ASCII value

  - Header

    - Reserved

    - Min. X*

    - Min. Y*

    - Max X*

    - Max Y*

    - Total X

    - Total Y

    - Character Height**

  - Vector List

    - Relative vector data

      - Delta X = new X value - current X value

      - Delta Y = new Y value - current Y value

      - Delta I = X value of center of circle - current X value

      - Delta J = Y value of center of circle - current Y value

    - 4 Types

      - Move

        - Code = 1

        - Delta X

        - Delta Y

      - Line

        - Code = 2

        - Delta X

        - Delta Y

      - Circular Arc***

        - Code = 3

        - Delta X

        - Delta Y

        - Delta I

        - Delta J

      - End Of Character

        - Code = 255

* The min/max extents of the character are given in absolute coordinate values.   The vector data is relative to the current position.  The initial position is always (0,0).

** Character Height is the height of a standard full height character drawn in the same scale as this character.  For example, a lowercase ‘a’ that is half as tall as a capital ‘A’ might have Total Y = 100 and Character Height = 200.

*** Arcs MUST NOT cross a cardinal axis.  Arcs that extend to both sides of a cardinal (horizontal or vertical line) must be broken into 2 separate arcs, first to the intersection with the cardinal and then one to the end point.  Thus, a full circle can be made from 4 arcs.

Vectors

The vector information for the characters is given in a relative coordinates.  The all characters are assumed to begin at (0,0) absolute position.   The relative coordinates are simple the delta X and delta Y values to get to the next position.  For arcs, the delta I and delta J values indicate the relative position of the center of the circle from the current, starting position.  The coordinates specified by the arc command must actually lie on a circle.  If not, the results are uncertain.  Examples of the vector types are shown below.  The list of numbers below the vector type represent the 3 or 5 values that would be found in the font file for that vector.

Vectors may extend in any direction from the origin.  Characters that have vectors below the baseline (any absolute X value less than zero) will have decenders, such as ‘g ’or ‘p’.  If any absolute Y value is greater than the Character Height value in the header, that character will have ascenders, such as ‘Å’.

Because ProLase uses dynamic scaling of every character, it is NOT required that all characters be drawn to the same scale relative to each other (although it is usually easier for the designer that way).  ProLase will scale every character relative to its own Character Height value.  This value represents the full size height (usually the height of most capital letters like ‘A’ or ‘T’ draw at the same scale.  If the vector data for ‘B’ and ‘C’ are take from 2 separate graphics and ‘B’ has Total Y = 100 and ‘C’ has Total Y = 2000, they will both have the same height provided that Character Height = 100 and 2000 respectively.

Example

Following is a sample font made from parts of the Simplex.fnt file that comes with ProLase.  It includes three characters, the space character (ASCII 32), ‘A’ (ASCII 65) and ‘a’ (ASCII 97).

32

1

0

0

300

0

300

0

250

1

300

0

255

65

255

0

0

254

250

254

250

250

1

32

0

2

95

250

2

95

-250

1

-35

83

2

-119

0

1

-36

-83

1

222

0

255

97

255

0

0

206

166

206

166

250

1

174

166

2

0

-166

1

-24

12

2

24

24

1

-24

-24

2

-23

-12

2

-36

0

2

-24

12

2

-23

24

2

-12

35

2

0

24

2

12

35

2

23

24

2

24

12

2

36

0

2

23

-12

2

24

-24

1

-142

-130

1

174

0

255

# Appendix C

This appendix applies only to ProLaseXP PLUS with Indexer LPT on the Windows XP operating system.  The names, locations and uses of all relevant ‘helper’ files are fully described.

ProLaseXP PLUS can be run on any Windows operating system and it can use either Indexer LPT or any ‘Other’ stepper motor controller to driver the external axes.  If Indexer LPT from Ability Systems Corporation (215-657-4338 or www.abilitysystems.com) is selected, the version must be matched to the operating system used.  The Indexer LPT driver compatible with Windows 95/98/ME uses a driver that is accessed directly by ProLase as described above in Chapter 5.  The Indexer LPT driver that runs in Windows XP is different and, as a result, helper files are required by ProLase to provide backwards compatibility with existing ProLase PLUS systems.

ProLaseXP PLUS requires that the TableServer and Table applications be present in the ProLase installation folder to manipulate the external axes.  Of course, the Indexer LPT driver must previously be installed and working properly.  Also included in the ProLase folder is the TableBATConverter application.  This is a useful tool for updating the batch files by ProLase PLUS.  Finally, The TableClient project folder is included as a tutorial for advanced users that wish to write a program that interacts with the stepper motors and runs concurrently with ProLase.

TableServer

TableServer is a small application that must be running for ProLase to interact with the Indexer LPT driver.  The TableServer.EXE file must be located in the ProLase installation folder.  Only a single instance of TableServer can exist on a computer at a single time; attempts to run multiple copies will not succeed.  When running, TableServer is the one and only application that can access the Indexer LPT library and driver.  The Ability Systems Hardware Assist Module must be present and the driver must load and run properly or TableServer will terminate.  Even the Ability Systems IXDiag.EXE utility cannot run concurrently with TableServer.  Because TableServer links to the Indexer LPT library, the Indexer LPT Hardware Assist Module must be present or TableServer will terminate.  ProLase will automatically launch TableServer during its initialization of the external axes if it does not detect that it is already running.

TableServer has a single window interface as seen in the figure below.

TableServer relays commands to the Indexer LPT library and displays the responses.  The Next Command To Be Sent window allows the user to enter a command for Indexer LPT to process directly.   The command is issued with the Send button.  The Recent Commands and Recent Responses windows list the 10 most recent interactions with the Indexer LPT library.  The Clear button will erase the data in all of the windows.  The About button displays a window with version, build date, and interface data.

In addition to direct user commands, TableServer can relay remote command to Indexer LPT.  These commands are received over a windows socket connection.  TableServer listens on the port specified in the About window for a command.  When it receives a command, TableServer shows it as the Last Socket Command Received, logs it in the Recent Commands list, and issues the command to Indexer LPT.  The response from Indexer LPT is first displayed in the Recent Responses list, then send back over the socket that sent the command.

The primary use for TableServer is in this background relay capacity.  The main visual interface is mainly used only during initial setup or to debug an error in the table motion system.  As such, TableServer begins, and should generally remain, in a minimized state on the ‘System Tray’.  This is the small area near the clock on the Windows Task Bar.  Double-clicking on this TableServer icon will open the main TableServer window.  Alternately,  right-clicking will display a pop-up menu that allows opening the main window or exiting the application.

Table

Table is a command line utility application designed to provide full interaction with Indexer LPT through the TableServer application (see above).  The Table.EXE file MUST be located in the ProLase installation folder.  The complete Table command line must have the following form:

table command [/a address]

where command is the required command to send to Indexer and address is an optional IP address for the computer hosting an active instance of TableServer.  Any additional parameters are ignored.  Table will output the response from TableServer / Indexer LPT.

TableBATConverter

TableBATConverter is a small, utility application designed to help convert existing ProLase PLUS batch files to a form that can be used by ProLaseXP PLUS running on WindowsXP operating system.  While Table is called directly from ProLaseXP PLUS to move the external axis between or during layers, initialization of the system is accomplished by the InitXYZA.BAT batch file which issues a list of Table command lines to locate the axis unambiguously.

Previous versions of ProLase PLUS would call use an InitXYZA.BAT file of the following format:

echo set_accel:c > motor

echo set_lowspeed:c > motor

echo move:c, -50000 > motor

echo set_home:c > motor

where echo is the DOS command to display messave and > is the DOS instruction to redirect the output stream.  motor is the file name used to access the Indexer LPT driver.

To implement this initialization with the WindowsXP system, the Table program must be used instead of the echo command.  The previous example must then become

table set_accel:c > motor

table set_lowspeed:c > motor

table move:c, -50000 > motor

table set_home:c > motor

TableBATConverter facilitates this conversion.  The figure below shows an example of this.

An InitXYZA.BAT file is selected using the Browse button.  Once a valid file is chosen, the contents of that file are loaded, converted, and displayed in the listing on the right.  The original file contents are displayed on the left side for reference.  The converted file may be edited to add additional comments and to correct automated conversion errors.  The Save button will store the converted file.  The original file contents are backed up in a file with the same title but the ‘BAK’ extension.

TableClient

TableClient is a small, GUI application that demonstrates communication with TableServer application (see above) via Windows Socket technology.  Commands for Indexer LPT are accepted, redisplayed, and sent to TableServer for processing.  The responses are then received and logged as well.

Due to the design of the Indexer LPT driver for Windows XP, only a single application is allowed access.  With ProLaseXP PLUS, this application is TableServer.  ProLase does NOT access the Indexer LPT driver directly specifically to allow other programs to operate.  This allows for complex installations where stepper motors may be used for more than just ProLase control.  Consider the fairly common case of a ‘job shop’ with a fixture for Step & Repeat processing of various parts.  All of the parts have the same footprint and are easily accommodated by the same fixture.  However, their heights are varied.  Since the mark applied to them is the same for all parts, say the company logo, only a single ProLase document (LAZ file) and a single fixture file are required.  However, every tray of parts must be adjusted to in the Z axis to focus the laser at the top.  This is quickly and easily preformed manually using a second Z axis control program.  The user jogs the fixture’s elevation to bring find the appropriate elevation.  Then ProLase marks normally. Ignorant of the change in the Z axis.

TableServer allows both ProLase and the Z Axis control program to manipulate the appropriate axes using the same Indexer LPT stepper motor controller.  Rather than directly accessing the Indexer LPT driver, the Z Axis control program must, just as ProLase, be written to use TableServer to interact with the axes.  TableClient is a simple demostration of just how to accomplish this.

The entire TableClient project is provided with the ProLaseXP Plus installation.  This project contains all the source code for the TableClient application.  ProLase customers who require such additional motor control as well advised to use the TableClient application source as a guide to writing their own application.